| Resume page |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No. | Revision Content |  | Version | Revisionist | Revision date |
| 1 | New Release |  | 1.0 | Zhang Zehui | 2024.07.11 |
| Approval: Date: |  | Auditor: Date: | Maker: Date: |  |  |

Content

1. Summary	6

1.1 Brief Introduction	6

1.2 Instructions	6

1.3 Important Concepts	6

2. CTL100H System API Reference	7

2.1 Help information	7

2.2 Status information output	7

2.3 Configure the GPIO direction on the rear panel of CTL100H	8

2.4 Configure the GPIO output level on the rear panel of CTL100H	9

2.5 Obtain the GPIO input level on the rear panel of CTL100H	10

2.6 Obtain GPIO status information on the rear panel of CTL100H	10

2.7 Set CTL100H serial port baud rate	11

2.8 Reset CTL100H system configuration	12

2.9 Reset CTL100H network configuration	12

2.10 Reset all configurations of CTL100H	13

3. RX Control Module API Reference	14

3.1 Set RX ID number	14

3.2 Set RX name	14

3.3 Set RX routing	15

3.4 Lock RX video routing	15

3.5 Lock RX IR routing	16

3.6 Lock RX RS-232 routing	17

3.7 Lock RX USB routing	17

3.8 Control the RX power light to flash	18

3.9 Control the digital tube on the front panel of RX	19

3.10 Set RX OSD on/off	19

3.11 Set the front panel button on/off	20

3.12 Set RX HDMI OUTPUT on/off	21

3.13 Set RX HDMI OUTPUT MUTE	21

3.14 Set RX HDMI OUTPUT pause	22

3.15 Set RX output resolution	22

3.16 Set RX screen rotation	23

3.17 Set IR on/off	24

3.18 Set RX IR level	25

3.19 Set RX transmission mode	25

3.20 Set RX standby time	26

3.21 Set RX image output mode	26

3.22 Set RX serial port parameters	27

3.23 Activate RX serial port Guest mode	28

3.24 Exit RX serial port Guest mode	28

3.25 Set RX IP mode	28

3.26 Set RX IP address	29

3.27 Set RX subnet mask	30

3.28 Set RX gateway address	30

3.29 Set RX network reboot	31

3.30 Remove RX from the system	32

3.31 Reboot RX	32

3.32 Reset RX	33

3.33 Obtain RX status information	33

3.34 Set RX preset IP mode	34

3.35 Set RX preset IP starting address	35

3.36 Set RX preset IP end address	35

3.37 Set RX preset subnet mask	36

3.38 Set RX preset gateway address	36

3.39 Save RX preset configuration	37

3.40 Set RX HDCP mode	37

4. TX Control Module API Reference	39

4.1 Set TX ID number	39

4.2 Set TX name	39

4.3 Control the TX power light to flash	40

4.4 Control TX front panel digital tube	40

4.5 Set TX audio source	41

4.6 Set TX EDID	42

4.7 Set TX to copy RX EDID	42

4.8 Set TX IR level	43

4.9 Set TX encoding bandwidth	44

4.10 Set TX mainstream encoding parameters	44

4.11 Set TX secondary stream encoding parameters	45

4.12 Set TX audio encoding format	46

4.13 Set TX serial port parameters	46

4.14 Activate TX serial port Guest mode	47

4.15 Exit TX serial port Guest mode	47

4.16 Set TX IP mode	48

4.17 Set TX IP address	48

4.18 Set TX subnet mask	49

4.19 Set TX gateway address	50

4.20 Set TX network reboot	50

4.21 Remove TX from the system	51

4.22 Reboot TX	51

4.23 Reset TX	52

4.24 Obtain TX status information	52

4.25 Set TX preset IP mode	53

4.26 Set TX preset IP starting address	54

4.27 Set TX preset IP end address	54

4.28 Set TX preset subnet mask	55

4.29 Set TX preset gateway address	55

4.30 Save TX preset configuration	56

5. Video Wall Module API Reference	57

5.1 Create a video wall	57

5.2 Delete video wall	57

5.3 Change the name of the video wall	57

5.4 Set the size of the video wall	58

5.5 Assign RX to Video Wall	58

5.6 Create a video wall preset	59

5.7 Delete video wall preset	59

5.8 Modify the preset name of the video wall	60

5.9 Activate video wall preset	61

5.10 Set video wall preset grouping	61

5.11 Set the signal source for the preset grouping of the video wall	62

5.12 Set the preset matrix group for the video wall	62

5.13 Set the signal source for the preset matrix group of the video wall	63

5.14 Set the width bezel of the video wall screen	63

5.15 Set the height bezel of the video wall screen	64

5.16 Get the status of the video wall	65

6. System Management Module API Reference	66

6.1 Device Search	66

6.2 View device search results	67

6.3 Clear device search results	67

6.4 Automatically add new devices to the system	68

6.5 Add a new TX device to the system	69

6.6 Add a new RX device to the system	69

6.7 Clear existing devices from the system	70

7. CTL100H Network Configuration API Reference	71

7.1 Set CTL100H IP mode	71

7.2 Set CTL100H IP address	71

7.3 Set CTL100H gateway address	72

7.4 Set CTL100H subnet mask	73

7.5 Reboot CTL100H network	73

7.6 Set CTL100H TELNET on/off	74

7.7 Set CTL100H TELNET port number	74

7.8 Set CTL100H SSH on/off	75

7.9 Set CTL100H SSH port number	75

7.10 Set CTL100H HTTPS on/off	76

7.11 Set CTL100H WEB GUI on/off	76

7.12 Modify CTL100H domain name	77

Summary

Brief Introduction

This document is used to introduce API commands based on CTL100H, and is suitable for testing engineers and software engineers.

Instructions

Before using API commands on CTL100H, it is necessary to remotely log in to the corresponding terminal using TELNET or use a serial port terminal to interact with API commands. Any of the following methods can be used to enter the control terminal for API interaction.

- TELNET login to CTL100H, default port number 23.

- Connect the serial port on the rear panel of CTL100H to the PC using a serial port cable. Open the serial port terminal tool on the PC, select the corresponding serial port number to connect, and then enter the controller terminal for API interaction. The default baud rate is 57600, with 8 data bits, 1 stop bit, and no parity bit.

Important Concepts

RX1：
The RX1 mentioned in the following text refers to the RX with ID number 1.

TX1：

The TX1 mentioned in the following text refers to the TX with ID number 1.

CTL100H System API Reference

Help information

| API interface |  |
| --- | --- |
| HELP or ? |  |
| Describe |  |
| Print API commands supported by the current system |  |
| Parameter | Describe |
| None |  |
| Return value | Describe |
| HELP Information | Print HELP information |
| Example |  |
| TELNET login CTL100H Enter the command: HELP Enter the command: ? |  |

Status information output

| API interface |  |
| --- | --- |
| GET STATUS |  |
| Describe |  |
| Output CTL100H status information and TX/RX status information added to the current project. |  |
| Parameter | Describe |
| None |  |
| Return value | Describe |
| Status information |  |
| Example |  |
| TELNET login CTL100H Enter the command: GET STATUS Return: ================================================================ ================================================================ IP Control Box CTL100H Status Info FW Version: 2.03.19 Power IR Baud On On 57600 In EDID IP NET/Sig AudioFormat 001 DF000 169.254.010.001 On /Off PCM 002 DF000 169.254.010.002 Off/Off PCM Out FromIn IP NET/HDMI Res Mode Osp 001 001 169.254.020.001 Off/Off 01 MX SNK 002 001 169.254.020.002 Off/Off 01 MX SNK 003 001 169.254.006.003 On /Off 01 MX SNK LAN DHCP IP Gateway SubnetMask 01_POE Off 169.254.008.100 169.254.008.001 255.255.000.000 02_CTRL On 192.168.006.100 192.168.006.001 255.255.255.000 (static:192.168.006.100 192.168.006.001 255.255.255.000) Telnet SSH2 HTTPS WEB LAN01 MAC LAN02 MAC 00023 Off Off On A2:9C:E4:17:56:F9 00:00:00:00:00:08 Domain Name Controller.local ================================================================ API Interface 002 001 169.254.020.002 Off/Off 01 MX SNK 003 001 169.254.006.003 On /Off 01 MX SNK LAN DHCP IP Gateway SubnetMask 01_POE Off 169.254.008.100 169.254.008.001 255.255.000.000 02_CTRL On 192.168.006.100 192.168.006.001 255.255.255.000 (static:192.168.006.100 192.168.006.001 255.255.255.000) Telnet SSH2 HTTPS WEB LAN01 MAC LAN02 MAC 00023 Off Off On A2:9C:E4:17:56:F9 00:00:00:00:00:08 Domain Name Controller.local ================================================================ |  |

Configure the GPIO direction on the rear panel of CTL100H

| API interface |  |
| --- | --- |
| SET GPIO [gpio] DIR IN/OUT |  |
| Describe |  |
| Configure the GPIO direction on the rear panel of CTL100H |  |
| Parameter | Describe |
| gpio | 1: GPIO1 2: GPIO2 3: GPIO3 4: GPIO4 |
| IN | GPIO as input function |
| OUT | GPIO as output function |
| Return value | Describe |
| [SUCCESS]Set GPIO 01 as input port. | GPO1 is configured as input mode |
| [SUCCESS]Set GPIO 01 as output port. | GPO1 is configured as output mode |
| Example |  |
| TELNET login CTL100H Configure GPIO1 as input mode, enter the command: SET GPIO 1 DIR IN return: [SUCCESS]Set GPIO 01 as input port. Configure GPO1 as output mode, input command: SET GPIO 1 DIR OUT return: [SUCCESS]Set GPIO 01 as output port. |  |

Configure the GPIO output level on the rear panel of CTL100H

| API interface |  |
| --- | --- |
| SET GPIO [gpio] LEVEL Low/High |  |
| Describe |  |
| Configure the GPIO output level on the rear panel of CTL100H. This API only works on GPIO with output direction. |  |
| Parameter | Describe |
| gpio | 1: GPIO1 2: GPIO2 3: GPIO3 4: GPIO4 |
| Low/High | Low: Output low level High: Output high level |
| Return value | Describe |
| [SUCCESS]Set GPIO 01 output level 0. | GPO1 output low level |
| [SUCCESS]Set GPIO 01 output level 1. | GPO1 output high level |
| Example |  |
| TELNET login CTL100H Configure GPO1 to output low level, enter the command: SET GPIO 1 LEVEL Low return: [SUCCESS]Set GPIO 01 output level 0. Configure GPO1 to output high level, input command: SET GPIO 1 LEVEL High return: [SUCCESS]Set GPIO 01 output level 1. |  |

Obtain the GPIO input level on the rear panel of CTL100H

| API interface |  |
| --- | --- |
| GET GPIO [gpio] LEVEL |  |
| Describe |  |
| Obtain the GPIO input level on the rear panel of CTL100H. This API only works on GPIO with input direction. |  |
| Parameter | Describe |
| gpio | 1: GPIO1 2: GPIO2 3: GPIO3 4: GPIO4 |
| Return value | Describe |
| [SUCCESS]Get GPIO 01 real input level 1. | GPO1 obtains input level as high level |
| Example |  |
| TELNET login CTL100H Get GPO1 input level, enter the command: GET GPIO 1 LEVEL return: [SUCCESS]Get GPIO 01 real input level 1. |  |

Obtain GPIO status information on the rear panel of CTL100H

| API interface |  |
| --- | --- |
| GET GPIO [gpio] STATUS |  |
| Describe |  |
| Obtain GPIO status information on the rear panel of CTL100H. |  |
| Parameter | Describe |
| gpio | Optional parameter, when not specified, represents obtaining all GPIO statuses 1: GPIO1 2: GPIO2 3: GPIO3 4: GPIO4 |
| Return value | Describe |
| Return GPIO status information |  |
| Example |  |
| TELNET login CTL100H Get GPO1 status information, enter the command: GET GPIO 1 STATUS Return: ================================================================ IP Control Box CTL100H GPIO Info FW Version: 2.03.19 GPIO DIR Set Get 01 Out 0 0 ================================================================ |  |

Set CTL100H serial port baud rate

| API interface |  |
| --- | --- |
| SET RS232BAUDRATE [a] |  |
| Describe |  |
| Set the baud rate of CTL100H serial port to a, with a factory default of 57600 |  |
| Parameter | Describe |
| a | [0:115200 1:57600, 2:38400, 3:19200, 4:9600] |
| Return value | Describe |
| [SUCCESS]Set RS232 Baud Rate to 115200bps. | Set baud rate to 57600 successfully |
| Example |  |
| TELNET login CTL100H Set the serial port baud rate to 115200, enter the command: SET RS232BAUDRATE 0 return: [SUCCESS]Set RS232 Baud Rate to 115200bps. Set the serial port baud rate to 57600, enter the command: SET RS232BAUDRATE 1 return: [SUCCESS]Set RS232 Baud Rate to 57600bps. |  |

Reset CTL100H system configuration

| API interface |  |
| --- | --- |
| SET RESET |  |
| Describe |  |
| Reset system configuration information and clear devices that have been added to the system. |  |
| Parameter | Describe |
| None |  |
| Return value | Describe |
| [SUCCESS]System will reset to default config, it will take about 40 seconds, and RS232 will disable during this time, please wait... | Reset successful |
| Example |  |
| TELNET login CTL100H Reset system configuration, enter the command: SET RESET return: Sure to RESET system to default settings? Type "Yes" after next prompt to confirm... enter yes return: [SUCCESS]System will reset to default config, it will take about 40 seconds, and RS232 will disable during this time, please wait... |  |

Reset CTL100H network configuration

| API interface |  |
| --- | --- |
| SET RESET NETWORK |  |
| Describe |  |
| Reset CTL100H network configuration. |  |
| Parameter | Describe |
| None |  |
| Return value | Describe |
| [SUCCESS]Network will reset to default config, it will take about 40 seconds, and RS232 will disable during this time, please wait... | Reset successfully |
| Example |  |
| TELNET login CTL100H Reset the network configuration, enter the command: SET RESET NETWORK Return: Sure to RESET network config to default settings? Type "Yes" after next prompt to confirm... Enter yes Return: [SUCCESS]Network will reset to default config, it will take about 40 seconds, and RS232 will disable during this time, please wait... |  |

Reset all configurations of CTL100H

| API interface |  |
| --- | --- |
| SET RESET ALL |  |
| Describe |  |
| Reset all configurations of CTL100H. |  |
| Parameter | Describe |
| None |  |
| Return value | Describe |
| [SUCCESS]System and network will reset to default config, it will take about 40 seconds, and RS232 will disable during this time, please wait... | Reset successfully |
| Example |  |
| TELNET login CTL100H Reset all configuration, enter the command: SET RESET ALL Return: Sure to RESET system and network to default settings? Type "Yes" after next prompt to confirm... Enter yes Reset all configuration, enter the command: [SUCCESS]System and network will reset to default config, it will take about 40 seconds, and RS232 will disable during this time, please wait... |  |

RX Control Module API Reference

Set RX ID number

| API interface |  |
| --- | --- |
| SET DEC [dec] ID [id] |  |
| Describe |  |
| Set RX ID number. |  |
| Parameter | Describe |
| dec | [001... 762]: RX ID number |
| id | [001... 762]: Target ID number |
| Return value | Describe |
| [SUCCESS]Set decoder 001 ID to 760. | Set RX1 ID number to 760 |
| [ERROR]Decoder 100 does not exist. | RX100 does not exist |
| Example |  |
| TELNET login CTL100H Set RX1 ID to 760, enter the command: SET DEC 1 ID 760 return: [SUCCESS]Set decoder 001 ID to 760. |  |

Set RX name

| API interface |  |
| --- | --- |
| SET DEC [dec] NAME [name] |  |
| Describe |  |
| Set RX name. |  |
| Parameter | Describe |
| dec | [001... 762]: RX ID number |
| name | Name, with a maximum length of 16 bytes |
| Return value | Describe |
| [SUCCESS]Set decoder 001 name: TEST1. | Set RX1 name to TEST 1 |
| Example |  |
| TELNET login CTL100H Set RX1 alias as TEST 1, enter the command: SET DEC 1 NAME TEST return: [SUCCESS]Set decoder 001 name:TEST1. |  |

Set RX routing

| API interface |  |
| --- | --- |
| SET DEC [dec] SWITCH [enc] ALL |  |
| Describe |  |
| Set RX Video/IR/RS-232/USB routing |  |
| Parameter | Describe |
| dec | [001... 762]: RX ID number 0: All RXs |
| enc | [001... 762]: TX ID number 0: Cancel routing |
| Return value | Describe |
| [SUCCESS]Set decoder 001 from encoder 003. | Set RX1 Video/IR/RS-232/USB signal routing to TX3 |
| Example |  |
| TELNET login CTL100H Enter the command: SET DEC 1 SWITCH 3 ALL return: [SUCCESS]Set decoder 001 from encoder 003. Enter the command: SET DEC 1 SWITCH 0 ALL： return: [SUCCESS]Set decoder 001 no source. |  |

Lock RX video routing

| API interface |  |
| --- | --- |
| SET DEC [dec] SWITCH [enc] VIDEO |  |
| Describe |  |
| Lock the RX video signal routing. |  |
| Parameter | Describe |
| dec | [001... 762]: RX ID number 0: All RXs |
| enc | [001... 762]: TX ID number 0: Unlock |
| Return value | Describe |
| [SUCCESS]Set decoder 001 video from encoder 003. | Set RX1 to lock the video signal routing to TX3 |
| Example |  |
| TELNET login CTL100H Enter the command: SET DEC 1 SWITCH 3 VIDEO return: [SUCCESS]Set decoder 001 video from encoder 003. Enter the command: SET DEC 1 SWITCH 0 VIDEO return: [SUCCESS]Set decoder 001 video follow SET DEC SWITCH ALL command select. |  |

Lock RX IR routing

| API interface |  |
| --- | --- |
| SET DEC [dec] SWITCH [enc] IR |  |
| Describe |  |
| Lock the RX IR signal routing. |  |
| Parameter | Describe |
| dec | [001…762]: RX ID number 0: All RXs |
| enc | [001...762]: 0: Unlock |
| Return value | Describe |
| [SUCCESS]Set decoder 001 IR from encoder 003. | Set RX1 to lock IR signal routing to TX3 |
| Example |  |
| TELNET login CTL100H Enter the command: SET DEC 1 SWITCH 3 IR return: [SUCCESS]Set decoder 001 IR from encoder 003. Enter the command: SET DEC 1 SWITCH 0 IR return: [SUCCESS]Set decoder 001 IR follow SET DEC SWITCH ALL command select. |  |

Lock RX RS-232 routing

| API interface |  |
| --- | --- |
| SET DEC [dec] SWITCH [enc] RS232 |  |
| Describe |  |
| Lock the RX RS-232 signal routing. |  |
| Parameter | Describe |
| dec | [001…762]: RX ID number 0: All RXs |
| enc | [001...762]: TX ID number 0: Unlock |
| Return value | Describe |
| [SUCCESS]Set decoder 001 RS232 from encoder 003. | Set RX1 to lock RS-232 signal routing to TX3 |
| Example |  |
| TELNET login CTL100H Enter the command: SET DEC 1 SWITCH 3 RS232 return: [SUCCESS]Set decoder 001 RS232 from encoder 003. Enter the command: SET DEC 1 SWITCH 0 RS232 return: [SUCCESS]Set decoder 001 serial follow SET DEC SWITCH ALL command select. |  |

Lock RX USB routing

| API interface |  |
| --- | --- |
| SET DEC [dec] SWITCH [enc] USB |  |
| Describe |  |
| Lock the RX USB signal routing. |  |
| Parameter | Describe |
| dec | [001... 762]: RX ID number 0: All RXs |
| enc | [001... 762]: TX ID number 0: Unlock |
| Return value | Describe |
| [SUCCESS]Set decoder 001 USB from encoder 003. | Set RX1 to lock USB signal routing to TX3 |
| Example |  |
| TELNET login CTL100H Enter the command: SET DEC 1 SWITCH 3 USB return: [SUCCESS]Set decoder 001 USB from encoder 003. Enter the command: SET DEC 1 SWITCH 0 USB return: [SUCCESS]Set decoder 001 USB follow SET DEC SWITCH ALL command select. |  |

Control the RX power light to flash

| API interface |  |
| --- | --- |
| SET DEC [dec] LED ON/OFF SET DEC [dec] LED ON 90 |  |
| Describe |  |
| Control the RX power light to flash. |  |
| Parameter | Describe |
| dec | [001... 762]: RX ID number 0: All RXs |
| ON | The power light is flashing |
| OFF | The power light is always on |
| ON 90 | The power light flashes for 90 seconds and then remains on |
| Return value | Describe |
| [SUCCESS]Flash power LED on decoder 001. | Flash RX1 power light |
| Example |  |
| TELNET login CTL100H Flash power light, enter the command: SET DEC 1 LED ON return: [SUCCESS]Flash power LED on decoder 001. The power light is always on. Enter the command: SET DEC 1 LED OFF return: [SUCCESS]Disable flash power LED on decoder 001. Flash power light for 90 seconds, enter the command: SET DEC 1 LED ON 90 return: [SUCCESS]Flash power LED on decoder 001 and keep 90 seconds. |  |

Control the digital tube on the front panel of RX

| API interface |  |
| --- | --- |
| SET DEC [dec] FPLED 0/9 |  |
| Describe |  |
| Control the digital tube on the front panel of RX to turn on/off. |  |
| Parameter | Describe |
| dec | [001…762]: RX ID number 0: All RXs |
| 0 | The digital tube is always on |
| 9 | The digital tube lights up for 90 seconds before turning off |
| Return value | Describe |
| [SUCCESS]Set decoder 001 LED always on. | Control the digital tube to be always on. |
| Example |  |
| TELNET login CTL100H Control the RX1 digital tube to remain on, enter the command: SET DEC 1 FPLED 0 return: [SUCCESS]Set decoder 001 LED always on. Control the RX1 digital tube to turn on for 90 seconds and then turn off. Enter the command: SET DEC 1 FPLED 9 return: [SUCCESS]Set decoder 001 LED 90 seconds auto off. |  |

Set RX OSD on/off

| API interface |  |
| --- | --- |
| SET DEC [dec] OUTPUT OSD ON/OFF |  |
| Describe |  |
| Turn on/off RX OSD |  |
| Parameter | Describe |
| dec | [001…762]: RX ID number 0: All RXs |
| ON | Turn on OSD |
| OFF | Turn off OSD |
| Return value | Describe |
| [SUCCESS]Show OSD on decoder 001. | Turn on OSD |
| Example |  |
| TELNET login CTL100H Turn on OSD and enter the command: SET DEC 1 OUTPUT OSD ON return: [SUCCESS]Show OSD on decoder 001. Turn off OSD, enter the command: SET DEC 1 OUTPUT OSD OFF return: [SUCCESS]Hide OSD on decoder 001. |  |

Set the front panel button on/off

| API interface |  |
| --- | --- |
| SET DEC [dec] BUTTON ON/OFF |  |
| Describe |  |
| Turn on/off RX front panel button |  |
| Parameter | Describe |
| dec | [001…762]: RX ID number 0: All RXs |
| ON | Enable button |
| OFF | Disable button |
| Return value | Describe |
| [SUCCESS]Set decoder 003 front panel button on. | Enable button |
| Example |  |
| TELNET login CTL100H Enable button, enter the command: SET DEC 1 BUTTON ON return: [SUCCESS]Set decoder 003 front panel button on. |  |

Set RX HDMI OUTPUT on/off

| API interface |  |
| --- | --- |
| SET DEC [dec] OUTPUT ON/OFF |  |
| Describe |  |
| Turn on/off RX HDMI OUTPUT. |  |
| Parameter | Describe |
| dec | [001…762]:RX ID number 0: All RXs |
| ON | Turn on HDMI OUTPUT |
| OFF | Turn off HDMI OUTPUT |
| Return value | Describe |
| [SUCCESS]Set decoder 001 output on. | Turn on HDMI OUTPUT |
| Example |  |
| TELNET login CTL100H Turn on HDMI OUTPUT, enter the command： SET DEC 1 OUTPUT ON return： [SUCCESS]Set decoder 001 output on. Turn off HDMI OUTPUT, enter the command： SET DEC 1 OUTPUT OFF return： [SUCCESS]Set decoder 001 output off. |  |

Set RX HDMI OUTPUT MUTE

| API interface |  |
| --- | --- |
| SET DEC [dec] OUTPUT MUTE ON/OFF |  |
| Describe |  |
| Set RX HDMI OUTPUT mute. |  |
| Parameter | Describe |
| dec | [001…762]:RX ID number 0: All RXs |
| ON | Set HDMI OUTPUT mute on |
| OFF | Set HDMI OUTPUT mute off |
| Return value | Describe |
| [SUCCESS]Set decoder 001 output mute on. | Set HDMI OUTPUT mute on |
| Example |  |
| TELNET login CTL100H Set HDMI OUTPUT mute on, enter the command: SET DEC 1 OUTPUT MUTE ON return： [SUCCESS]Set decoder 001 output mute on. Set HDMI OUTPUT mute off, enter the command: SET DEC 1 OUTPUT MUTE OFF return： [SUCCESS]Set decoder 001 output mute off. |  |

Set RX HDMI OUTPUT pause

| API interface |  |
| --- | --- |
| SET DEC [dec] OUTPUT PAUSE ON/OFF |  |
| Describe |  |
| Pause RX HDMI output. |  |
| Return value | Describe |
| dec | [001…762]: RX ID number 0: All RXs |
| ON | Pause HDMI OUTPUT |
| OFF | Restore HDMI OUTPUT |
| Return value | Describe |
| [SUCCESS]Set decoder 001 pause on. | Pause RX1 HDMI OUTPUT |
| Example |  |
| TELNET login CTL100H Pause RX1 HDMI OUTPUT, enter the command: SET DEC 1 OUTPUT PAUSE ON return: [SUCCESS]Set decoder 001 pause on. Turn off HDMI OUTPUT PAUSE, enter the command: SET DEC 1 OUTPUT PAUSE OFF return: [SUCCESS]Set decoder 001 pause off. |  |

Set RX output resolution

| API interface |  |
| --- | --- |
| SET DEC [dec] OUTPUT RESOLUTION [res] |  |
| Describe |  |
| Set RX output resolution to res |  |
| Parameter | Describe |
| dec | [001…762]: RX ID number 0: All RXs |
| res | 00: Bypass 01: 1080p@60 02: 1080p@50 03: 1080p@30 04: 1080p@25 05: 1080p@24 06: 720@p60 07: 720p@50 08: 576p@50 09: 480p@60 10: 640x480@60 11: 800x600@60 12: 1024x768@60 13: 1280x800@60 14: 1280x1024@60 15: 1366x768@60 16: 1440x900@60 17: 1600x1200@60 18: 1680x1050@60 19: 1920x1200@60 |
| Return value | Describe |
| [SUCCESS]Set decoder 001 resolution to 1080P@60Hz. | Set RX1 output resolution to 1080P60 |
| Example |  |
| TELNET login CTL100H Set the output resolution of RX1 to 1080P60, enter the command: SET DEC 1 OUTPUT RESOLUTION 1 return: [SUCCESS]Set decoder 001 resolution to 1080P@60Hz. |  |

Set RX screen rotation

| API interface |  |
| --- | --- |
| SET DEC [dec] OUTPUT ROTATE [rtt] |  |
| Describe |  |
| Set RX screen rotation. |  |
| Parameter | Describe |
| dec | [001…762]: RX ID number 0: All RXs |
| rtt | 0:0° 1:90° 2:180° 3:270° |
| Return value | Describe |
| [SUCCESS]Set decoder 001 rotate 90 degree. | Set RX1 screen to rotate 90 degrees |
| Example |  |
| TELNET login CTL100H Set the RX1 screen to rotate 90 degrees, enter the command: SET DEC 1 OUTPUT ROTATE 1 return: [SUCCESS]Set decoder 001 rotate 90 degree. |  |

Set IR on/off

| API interface |  |
| --- | --- |
| SET DEC [dec] IR ON/OFF |  |
| Describe |  |
| Set the IR on/off on the rear panel of RX. |  |
| Parameter | Describe |
| dec | [001…762]: RX ID number 0: All RXs |
| ON | Turn on IR |
| OFF | Turn off IR |
| Return value | Describe |
| [SUCCESS]Send decoder 001 IR data ON. | Turn on IR |
| Example |  |
| TELNET login CTL100H Set the IR on, enter the command: SET DEC 1 IR ON return: [SUCCESS]Send decoder 001 IR data ON. |  |

Set RX IR level

| API interface |  |
| --- | --- |
| SET DEC [dec] IR VOL 5V/12V |  |
| Describe |  |
| Set the RX IR level. |  |
| Parameter | Describe |
| dec | [001…762]: RX ID number 0: All RXs |
| 5V | Use 5V IR cable |
| 12V | Use 12V IR cable |
| Return value | Describe |
| [SUCCESS]Set decoder 001 IR voltage 5V. | Set RX1 to use 5V IR cable |
| Example |  |
| TELNET login CTL100H Set RX1 to use 5V IR cable, enter the command: SET DEC 1 IR VOL 5V： return: [SUCCESS]Set decoder 001 IR voltage 5V. |  |

Set RX transmission mode

| API interface |  |
| --- | --- |
| SET DEC [dec] STREAM UNICAST/MULTICAST |  |
| Describe |  |
| Set RX transmission mode. |  |
| Parameter | Describe |
| dec | [001…762]: RX ID number 0: All RXs |
| UNICAST | Unicast Mode |
| MULTICAST | Multicast mode |
| Return value | Describe |
| [SUCCESS]Set decoder 001 multicast on. | Set RX1 to use multicast mode |
| Example |  |
| TELNET login CTL100H Set RX1 to use multicast mode, enter the command: SET DEC 1 STREAM MULTICAST return: [SUCCESS]Set decoder 001 multicast on. |  |

Set RX standby time

| API interface |  |
| --- | --- |
| SET DEC [dec] OUTPUT LOST [time] |  |
| Describe |  |
| Set RX standby time, turn off output when there is no signal timeout. |  |
| Parameter | Describe |
| dec | [001…762]: RX ID number 0: All RXs |
| time | - [0... 60]: timeout, in minutes 0: Never standby |
| Return value | Describe |
| [SUCCESS]Set decoder 001 video lost timeout to 1 minutes. | Set RX1 no signal timeout for 1 minute to turn off output |
| Example |  |
| TELNET login CTL100H Set RX1 to disable output after 1 minute of no signal timeout. Enter the command: SET DEC 1 OUTPUT LOST 1 return: [SUCCESS]Set decoder 001 video lost timeout to 1 minutes. |  |

Set RX image output mode

| API interface |  |
| --- | --- |
| SET DEC [dec] MODE MX/VW |  |
| Describe |  |
| Set RX image output mode, only valid for the decoders in Video Wall. |  |
| Parameter | Describe |
| dec | [001…762]: RX ID number 0: All RXs |
| MX | MX mode output (displaying all images) |
| VW | VW mode output (displaying partial images) |
| Return value | Describe |
| [SUCCESS]Set decoder 001 to matrix mode. | Set RX1 MX mode output |
| Example |  |
| TELNET login CTL100H Set RX1 MX mode output, enter the command: SET DEC 1 MODE MX return: [SUCCESS]Set decoder 001 to matrix mode. |  |

Set RX serial port parameters

| API interface |  |
| --- | --- |
| SET DEC [dec] GUEST ON/OFF BR [br] BIT [bit] |  |
| Describe |  |
| Set RX serial port parameters |  |
| Parameter | Describe |
| dec | [001…762]: RX ID number 0: All RXs |
| ON | Enable serial port Guest mode |
| OFF | Disable the serial port Guest mode |
| br | [0:300 1:600 2:1200 3:2400 4:4800 5:9600] [6:19200 7:38400 8:57600 9:115200] |
| bit | Data Bits + Parity + Stop Bits example: 8n1 Data Bits=[5...8], Parity=[n o e], Stop Bits=[1..2] |
| Return value | Describe |
| [SUCCESS]Set serial guest mode config done. | Successfully set RX serial port parameters |
| Example |  |
| TELNET login CTL100H Set RX1 to enable serial port Guest mode, baud rate 115200, 8-bit data bits, no checksum, 1-bit stop bit, enter the command: SET DEC 1 GUEST ON BR 9 BIT 8N1 return: [SUCCESS]Set serial guest mode config done. |  |

Activate RX serial port Guest mode

| API interface |  |
| --- | --- |
| SET DEC [dec] GUEST |  |
| Describe |  |
| Activate RX serial port Guest mode, only valid when the serial port parameter is set to GUEST ON |  |
| Parameter | Describe |
| dec | [001... 762]: RX ID number |
| Return value | Describe |
| None |  |
| Example |  |
| TELNET login CTL100H Activate RX1 serial port Guest mode, enter the command: SET DEC 1 GUEST |  |

Exit RX serial port Guest mode

| API interface |  |
| --- | --- |
| EXITGUEST |  |
| Describe |  |
| After starting RX serial port Guest mode, send EXITGUEST to exit Guest mode |  |
| Parameter | Describe |
| None |  |
| Return value | Describe |
| None |  |
| Example |  |
| TELNET login CTL100H Exit RX1 serial port Guest mode, enter the command: EXITGUEST |  |

Set RX IP mode

| API interface |  |
| --- | --- |
| SET DEC [dec] IPMODE DHCP/STATIC |  |
| Describe |  |
| Set the IP mode of RX |  |
| Parameter | Describe |
| dec | [001…762]: RX ID number 0: All RXs |
| DHCP | Dynamic IP |
| STATIC | Static IP |
| Return value | Describe |
| [SUCCESS]Set encoder 001 ip mode to dhcp. Use "SET DEC xx NETWORK REBOOT" command to apply new config!!! | Set successfully, RX needs to be restarted for it to take effect |
| Example |  |
| TELNET login CTL100H Set RX1 to dynamic IP mode, enter the command: SET DEC 1 IPMODE DHCP return: [SUCCESS]Set encoder 001 ip mode to dhcp. Use "SET DEC xx NETWORK REBOOT" command to apply new config!!! |  |

Set RX IP address

| API interface |  |
| --- | --- |
| SET DEC [dec] STATIC IP [ip] |  |
| Describe |  |
| Set the IP address of RX, which is only valid when IP MODE is set to STATIC. |  |
| Parameter | Describe |
| dec | [001…762]: RX ID number |
| ip | IP address, such as 169.254.10.10 |
| Return value | Describe |
| [SUCCESS]Set decoder 001 IP address to 169.254.020.006. Use "SET DEC xx NETWORK REBOOT" command to apply new config!!! | Set successfully, RX needs to be restarted for it to take effect |
| Example |  |
| TELNET login CTL100H Set the IP of RX1 to 169.254.20.6, enter the command: SET DEC 1 STATIC IP 169.254.20.6 return: [SUCCESS]Set decoder 001 IP address to 169.254.020.006. Use "SET DEC xx NETWORK REBOOT" command to apply new config!!! |  |

Set RX subnet mask

| API interface |  |
| --- | --- |
| SET DEC [dec] STATIC MASK [mask] |  |
| Describe |  |
| Set the subnet mask for RX, which is only valid when IP MODE is set to STATIC. |  |
| Parameter | Describe |
| dec | [001…762]: RX ID number 0: All RXs |
| mask | Subnet mask, such as 255.255.0.0 |
| Return value | Describe |
| [SUCCESS]Set encoder 001 subnet mask address to 255.255.000.000. Use "SET DEC xx NETWORK REBOOT" command to apply new config!!! | Set successfully, RX needs to be restarted for it to take effect |
| Example |  |
| TELNET login CTL100H Set the subnet mask of RX1 to 255.255.0.0, enter the command: SET DEC 1 STATIC MASK 255.255.0.0 return: [SUCCESS]Set encoder 001 subnet mask address to 255.255.000.000. Use "SET DEC xx NETWORK REBOOT" command to apply new config!!! |  |

Set RX gateway address

| API interface |  |
| --- | --- |
| SET DEC [dec] STATIC GATEWAY [gw] |  |
| Describe |  |
| Set the gateway address of RX, which is only valid when IP MODE is set to STATIC. |  |
| Parameter | Describe |
| dec | [001…762]: RX ID number 0: All RXs |
| gw | Gateway address, such as 169.254.0.1 |
| Return value | Describe |
| [SUCCESS]Set encoder 001 gateway address to 169.254.000.001. Use "SET DEC xx NETWORK REBOOT" command to apply new config!!! | Set successfully, RX needs to be restarted for it to take effect |
| Example |  |
| TELNET login CTL100H Set the gateway address of RX1 to 169.254.0.1, enter the command: SET DEC 1 STATIC GATEWAY 169.254.0.1 return： [SUCCESS]Set encoder 001 gateway address to 169.254.000.001. Use "SET DEC xx NETWORK REBOOT" command to apply new config!!! |  |

Set RX network reboot

| API interface |  |
| --- | --- |
| SET DEC [dec] NETWORK REBOOT |  |
| Describe |  |
| Set RX network reboot |  |
| Parameter | Describe |
| dec | [001…762]: RX ID number 0: All RXs |
| Return value | Describe |
| [SUCCESS]Set decoder 001 reboot and apply all the new config. | Set successfully, RX needs to be restarted for it to take effect |
| Example |  |
| TELNET login CTL100H Reboot the network of RX1 and enter the command: SET DEC 1 NETWORK REBOOT return: [SUCCESS]Set decoder 001 reboot and apply all the new config. |  |

Remove RX from the system

| API interface |  |
| --- | --- |
| SET DEC [dec] DELETE |  |
| Describe |  |
| Delete RX |  |
| Parameter | Describe |
| dec | [001…762]: RX ID number 0: All RXs |
| Return value | Describe |
| [SUCCESS]Delete decoder 001 done. | Delete RX1 successfully |
| Example |  |
| TELNET login CTL100H Delete RX1, enter the command: SET DEC 1 DELETE return: [SUCCESS]Delete decoder 001 done. |  |

Reboot RX

| API interface |  |
| --- | --- |
| SET DEC [dec] REBOOT |  |
| Describe |  |
| Reboot RX |  |
| Parameter | Describe |
| dec | [001…762]: RX ID number 0: All RXs |
| Return value | Describe |
| [SUCCESS]Set decoder 001 reboot and apply all the new config. | Reboot successfully |
| Example |  |
| TELNET login CTL100H Reboot RX1 and enter the command: SET DEC 1 REBOOT return: [SUCCESS]Set decoder 001 reboot and apply all the new config. |  |

Reset RX

| API interface |  |
| --- | --- |
| SET DEC [dec] RESET |  |
| Describe |  |
| Reset RX |  |
| Describe |  |
| dec | [001…762]: RX ID number 0: All RXs |
| Return value | Describe |
| [SUCCESS]Set decoder 001 reset to default setting. | Reset successful |
| Example |  |
| TELNET login CTL100H Reset RX1, enter the command: SET DEC 1 RESET return: [SUCCESS]Set decoder 001 reset to default setting. |  |

Obtain RX status information

| API interface |  |
| --- | --- |
| GET DEC [dec] STATUS |  |
| Describe |  |
| Obtain the status information of RX, without the parameter dec, to obtain the status information of all RX, i.e. GET DEC STATION and GET DEC 0 STATION. |  |
| Parameter | Describe |
| dec | [001…762]: RX ID number 0: All RXs |
| Return value | Describe |
| Return RX status information | Include version number, network information, and other status information |
| Example |  |
| TELNET login CTL100H Obtain the status information of RX1, enter the command: GET DEC 3 STATUS return： ================================================================ IP Control Box CTL100H Decoder Info FW Version: 2.03.19 Out Net HPD Ver Mode Res Rotate Name 003 On Off 1.10.03 MX 01 1 Decoder 003 >>Fr Vid/IR_/Ser/USB MCast 001 000/000/000/000 On >>ASPECT OSP IR BTN LED SGEn/Br/Bit Maintain SNK On On 9 Off /9/8n1 >>Video Mute Pause Auto VideoLostTimeout On Off Off On 0 >>MAC 6c:df:fb:07:cf:c6 GET DEC 3 STATUS return： ================================================================ IP Control Box CTL100H Decoder Info FW Version: 2.03.19 Out Net HPD Ver Mode Res Rotate Name 003 On Off 1.10.03 MX 01 1 Decoder 003 >>Fr Vid/IR_/Ser/USB MCast 001 000/000/000/000 On >>ASPECT OSP IR BTN LED SGEn/Br/Bit Maintain SNK On On 9 Off /9/8n1 >>Video Mute Pause Auto VideoLostTimeout On Off Off On 0 >>MAC 6c:df:fb:07:cf:c6 >>IP GW SM 169.254.020.003 169.254.008.001 255.255.000.000 ================================================================ |  |

Set RX preset IP mode

| API interface |  |
| --- | --- |
| SET DEC PRESET IPMODE [mode] |  |
| Describe |  |
| Set the preset IP mode for RX, and when adding RX to the system, assign the RX's IP based on this preset mode |  |
| Parameter | Describe |
| mode | 0:AUTOIP 1:DHCP 2: STATIC |
| Return value | Describe |
| [SUCCESS]Set decoder preset IP to static mode. | RX preset IP mode is set to static successfully |
| Example |  |
| TELNET login CTL100H Set the RX preset IP mode to static, enter the command: SET DEC PRESET IPMODE 2 return: [SUCCESS]Set decoder preset IP to static mode. |  |

Set RX preset IP starting address

| API interface |  |
| --- | --- |
| SET DEC PRESET START IP [ip] |  |
| Describe |  |
| Set the IP starting address for RX preset |  |
| Parameter | Describe |
| ip | IP address, such as 169.254.10.10 |
| Return value | Describe |
| [SUCCESS]Set decoder preset IP min 172.016.010.001. | Set the IP starting address for RX preset to 172.16.10.1 |
| Example |  |
| TELNET login CTL100H Set the RX preset IP starting address to 172.16.10.1, enter the command: SET DEC PRESET START IP 172.16.10.1 return: [SUCCESS]Set decoder preset IP min 172.016.010.001. |  |

Set RX preset IP end address

| API interface |  |
| --- | --- |
| SET DEC PRESET END IP [ip] |  |
| Describe |  |
| Set the RX preset IP end address, which should be greater than the starting address and in the same network segment. |  |
| Parameter | Describe |
| ip | IP address, such as 169.254.20.10 |
| Return value | Describe |
| [SUCCESS]Set decoder preset IP max 172.016.010.200. | Set the IP end address for RX preset to 172.16.10.200 |
| Example |  |
| TELNET login CTL100H Set the IP end address for RX preset to 172.16.10.200, enter the command: SET DEC PRESET END IP 172.16.10.200 return: [SUCCESS]Set decoder preset IP max 172.016.010.200. |  |

Set RX preset subnet mask

| API interface |  |
| --- | --- |
| SET DEC PRESET SM [mask] |  |
| Describe |  |
| Set RX preset subnet mask |  |
| Parameter | Describe |
| mask | Subnet mask, such as 255.255.0.0 |
| Return value | Describe |
| [SUCCESS]Set decoder preset netmask 255.255.000.000. | Set the subnet mask for RX preset to 255.255.0.0 |
| Example |  |
| TELNET login CTL100H Set the subnet mask for RX preset to 255.255.0.0, enter the command: SET DEC PRESET SM 255.255.0.0 return: [SUCCESS]Set decoder preset netmask 255.255.000.000. |  |

Set RX preset gateway address

| API interface |  |
| --- | --- |
| SET DEC PRESET GW [gw] |  |
| Describe |  |
| Set RX preset gateway address |  |
| Parameter | Describe |
| gw | Gateway address, such as 169.254.0.1 |
| Return value | Describe |
| [SUCCESS]Set decoder preset gateway 172.016.010.001. | Set the RX preset gateway address to 172.16.10.1 |
| Example |  |
| TELNET login CTL100H Set the RX preset gateway address to 172.16.10.1, enter the command: SET DEC PRESET GW 172.16.10.1 return: [SUCCESS]Set decoder preset gateway 172.016.010.001. |  |

Save RX preset configuration

| API interface |  |
| --- | --- |
| SET DEC PRESET APPLY |  |
| Describe |  |
| Save the preset configuration of RX. After setting the preset IP mode and other settings, you need to call APPLY to save it. |  |
| Parameter | Describe |
| Return value | Describe |
| [SUCCESS]Set decoder preset IP done. | Save RX preset configuration successfully |
| Example |  |
| TELNET login CTL100H Save RX preset configuration, enter the command: SET DEC PRESET APPLY return: [SUCCESS]Set decoder preset IP done. |  |

Set RX HDCP mode

| API interface |  |
| --- | --- |
| SET DEC [dec] OSP SNK/SRC/OFF/H14/H22 |  |
| Describe |  |
| Set RX HDCP mode, hide command, default value is SNK. |  |
| Parameter | Describe |
| dec | [001…762]: RX ID number 0: All RXs |
| SRC | Follow the video source |
| SNK | Follow the backend TV |
| OFF | Turn off HDCP |
| H14 | Force HDCP1.4 |
| H22 | Force HDCP2.2 |
| Return value | Describe |
| [SUCCESS]Set decoder 001 osp mode SNK. | Set RX1 HDCP mode to follow the backend TV |
| Example |  |
| TELNET login CTL100H Set RX1 HDCP mode to follow the backend TV, enter the command: SET DEC 1 OSP SNK return: [SUCCESS]Set decoder 001 osp mode SNK. |  |

TX Control Module API Reference

Set TX ID number

| API interface |  |
| --- | --- |
| SET ENC [enc] ID [id] |  |
| Describe |  |
| Set TX ID number. |  |
| Parameter | Describe |
| enc | [001…762]: TX ID number |
| id | [001…762]: Target ID number |
| Return value | Describe |
| [SUCCESS]Set encoder 001 ID to 760. | Set TX1 ID number to 760 |
| [ERROR]Encoder 100 does not exist. | TX100 does not exist |
| Example |  |
| TELNET login CTL100H Set TX1 ID to 760, enter the command: SET ENC 1 ID 760 return: [SUCCESS]Set encoder 001 ID to 760. |  |

Set TX name

| API interface |  |
| --- | --- |
| SET ENC [enc] NAME [name] |  |
| Describe |  |
| Set TX name. |  |
| Parameter | Describe |
| enc | [001…762]: TX ID number |
| name | Name, with a maximum length of 16 bytes |
| Return value | Describe |
| [SUCCESS]Set encoder 001 name:TEST1. | Set TX1 name as TEST 1 |
| Example |  |
| TELNET login CTL100H Set TX1 alias as TEST 1, enter the command: SET ENC 1 NAME TEST1 return: [SUCCESS]Set encoder 001 name:TEST1. |  |

Control the TX power light to flash

| API interface |  |
| --- | --- |
| SET ENC [enc] LED ON/OFF SET ENC [enc] LED ON 90 |  |
| Describe |  |
| Control the TX power light to flash. |  |
| Parameter | Describe |
| enc | [001…762]: TX ID number 0: All TX |
| ON | The power light is flashing |
| OFF | The power light is always on |
| ON 90 | The power light flashes for 90 seconds and then remains on |
| Return value | Describe |
| [SUCCESS]Flash power LED on encoder 001. | Flash TX1 power light |
| Example |  |
| TELNET login CTL100H Flash power light, enter the command: SET ENC 1 LED ON return: [SUCCESS]Flash power LED on encoder 001. The power light is always on. Enter the command: SET ENC 1 LED OFF return: [SUCCESS]Disable flash power LED on encoder 001. Flash power light for 90 seconds, enter the command: SET ENC 1 LED ON 90 return: [SUCCESS]Flash power LED on encoder 001 and keep 90 seconds. |  |

Control TX front panel digital tube

| API interface |  |
| --- | --- |
| SET ENC [enc] FPLED 0/9 |  |
| Describe |  |
| Control the digital tube on the TX front panel to turn on/off. |  |
| Parameter | Describe |
| enc | [001…762]: TX ID number 0: All TX |
| 0 | The digital tube is always on |
| 9 | The digital tube lights up for 90 seconds before turning off |
| Return value | Describe |
| [SUCCESS]Set encoder 001 LED always on. | Control the digital tube to stay on continuously |
| Example |  |
| TELNET login CTL100H Control the RX1 digital tube to remain on, enter the command: SET DEC 1 FPLED 0 return: [SUCCESS]Set encoder 001 LED always on. Control the RX1 digital tube to turn on for 90 seconds and then turn off. Enter the command: SET DEC 1 FPLED 9 return: [SUCCESS]Set encoder 001 LED 90 seconds auto off. |  |

Set TX audio source

| API interface |  |
| --- | --- |
| SET ENC [enc] AUDIO INPUT HDMI/ANA |  |
| Describe |  |
| Set up TX audio source. |  |
| Parameter | Describe |
| enc | [001…762]: TX ID number 0: All TX |
| HDMI | Audio from HDMI IN |
| ANA | Audio from analog input AUDIO IN L/R |
| Return value | Describe |
| [SUCCESS]Set encoder 001 audio select hdmi. | Set TX1 audio source to HDMI |
| Example |  |
| TELNET login CTL100H Set TX1 audio source to HDMI, enter the command: SET ENC 1 AUDIO INPUT HDMI return: [SUCCESS]Set encoder 001 audio select hdmi. |  |

Set TX EDID

| API interface |  |
| --- | --- |
| SET ENC [enc] EDID DEFAULT [edid] |  |
| Describe |  |
| Set TX EDID |  |
| Parameter | Describe |
| enc | [001…762]: TX ID number 0: All TX |
| edid | 00: HDMI 1080p@60Hz, Audio 2CH PCM 01: HDMI 720p@60Hz , Audio 2CH PCM 02: DVI 1280x1024@60Hz, Audio None 03: DVI 1920x1080@60Hz, Audio None 04: DVI 1920x1200@60Hz, Audio None 05: HDMI 1920x1200p@60Hz, Audio 2CH PCM 06: Copy EDID 07: User EDID 1 08: User EDID 2 |
| Return value | Describe |
| [SUCCESS]Set encoder 001 edid to HDMI 1080p@60Hz, Audio 2CH PCM. | Set TX1 EDID to 1080p@60Hz, Audio 2CH PCM |
| Example |  |
| TELNET login CTL100H Set TX1 EDID to 1080p@60Hz, Audio 2CH PCM, enter the command： SET ENC 1 EDID DEFAULT 0 return： [SUCCESS]Set encoder 001 edid to HDMI 1080p@60Hz, Audio 2CH PCM. |  |

Set TX to copy RX EDID

| API interface |  |
| --- | --- |
| SET ENC [enc] EDID COPY [dec] |  |
| Describe |  |
| Set TX to copy RX's EDID |  |
| Parameter | Describe |
| enc | [001…762]: TX ID number 0: All TX |
| dec | [001...762]: RX ID number |
| Return value | Describe |
| [SUCCESS]Copy decoder 002 edid to encoder 001. | Set TX1 to copy RX2's EDID |
| Example |  |
| TELNET login CTL100H Set TX1 to copy the EDID of RX2, enter the command: SET ENC 1 EDID COPY 2 return: [SUCCESS]Copy decoder 002 edid to encoder 001. |  |

Set TX IR level

| API interface |  |
| --- | --- |
| SET ENC [enc] IR VOL 5V/12V |  |
| Describe |  |
| Set TX IR level. |  |
| Parameter | Describe |
| enc | [001…762]: TX ID number 0: All TX |
| 5V | Use 5V IR cable |
| 12V | Use 12V IR cable |
| Return value | Describe |
| [SUCCESS]Set encoder 001 IR voltage 5V. | Set TX1 to use 5V IR cable |
| Example |  |
| TELNET login CTL100H Set TX1 to use 5V IR cable, enter the command: SET ENC 1 IR VOL 5V return: [SUCCESS]Set encoder 001 IR voltage 5V. |  |

Set TX encoding bandwidth

| API interface |  |
| --- | --- |
| SET ENC [enc] STREAM BITRATE [rate] |  |
| Describe |  |
| Set the TX encoding bandwidth. |  |
| Parameter | Describe |
| enc | [001…762]: TX ID number 0: All TX |
| rate | 0: 1Mb 1: 4Mb 2: 8Mb 3: 16Mb 4: 20Mb |
| Return value | Describe |
| [SUCCESS]Set encoder 001 bandwidth 8Mb. | Set TX1 encoding bandwidth to 8Mbps |
| Example |  |
| TELNET login CTL100H Set TX1 encoding bandwidth to 8Mbps, enter the command: SET ENC 1 STREAM BITRATE 2 return: [SUCCESS]Set encoder 001 bandwidth 8Mb. |  |

Set TX mainstream encoding parameters

| API interface |  |
| --- | --- |
| SET ENC [enc] MAINSTREAM E [type] A ON/OFF |  |
| Describe |  |
| Set TX mainstream encoding parameters. |  |
| Parameter | Describe |
| enc | [001…762]: TX ID number 0: All TX |
| type | 0: h264 1: h265 |
| ON | Add audio |
| OFF | Remove audio |
| Return value | Describe |
| [SUCCESS]Set encoder 001 main stream encode type H265 audio on. | Set TX1 mainstream H265 encoding and add audio |
| Example |  |
| TELNET login CTL100H Set TX1 mainstream H265 encoding, add audio, enter the command: SET ENC 1 MAINSTREAM E 1 A ON return: [SUCCESS]Set encoder 001 main stream encode type H265 audio on. |  |

Set TX secondary stream encoding parameters

| API interface |  |
| --- | --- |
| SET ENC [enc] SUBSTREAM E [type] H [sh] V [sv] B [rate] A ON/OFF |  |
| Describe |  |
| Set TX secondary stream encoding parameters. |  |
| Parameter | Describe |
| enc | [001…762]: TX ID number 0: All TX |
| type | 0: h264 1: h265 |
| sh | [320...640]: Must be an even number |
| sv | [180...540]: Must be an even number |
| rate | 0: 1Mb 1: 4Mb 2: 8Mb 3: 16Mb 4: 20Mb |
| ON | Add audio |
| OFF | Remove audio |
| Return value | Describe |
| [SUCCESS]Set encoder 001 sub stream encode type H265 width 640 height 540 bandwidth 8Mb audio on. | Set TX1 sub stream H265 encoding, width 640, height 540, 8Mb bandwidth, add audio |
| Example |  |
| TELNET login CTL100H Set TX1 sub stream H265 encoding, width 640, height 540, 8Mb bandwidth, add audio, enter the command: SET ENC 1 SUBSTREAM E 1 H 640 V 540 B 2 A ON return: [SUCCESS]Set encoder 001 sub stream encode type H265 width 640 height 540 bandwidth 8Mb audio on. |  |

Set TX audio encoding format

| API interface |  |
| --- | --- |
| SET ENC [enc] AUDIO FORMAT PCM/AAC |  |
| Describe |  |
| Set the TX audio encoding format. |  |
| Parameter | Describe |
| enc | [001…762]: TX ID number 0: All TX |
| PCM | Audio PCM encoding format |
| AAC | Audio AAC encoding format |
| Return value | Describe |
| [SUCCESS]Set encoder 001 audio format AAC. | Set TX1 audio to be encoded in AAC format |
| Example |  |
| TELNET login CTL100H Set TX1 audio to be encoded in AAC format, enter the command: SET ENC 1 AUDIO FORMAT AAC return: [SUCCESS]Set encoder 001 audio format AAC. |  |

Set TX serial port parameters

| API interface |  |
| --- | --- |
| SET ENC [enc] GUEST ON/OFF BR [br] BIT [bit] |  |
| Describe |  |
| Set TX serial port parameters |  |
| Parameter | Describe |
| enc | [001…762]: TX ID number 0: All TX |
| ON | Enable serial port Guest mode |
| OFF | Disable the serial port Guest mode |
| br | [0:300 1:600 2:1200 3:2400 4:4800 5:9600] [6:19200 7:38400 8:57600 9:115200] |
| bit | Data Bits + Parity + Stop Bits example: 8n1 Data Bits=[5...8], Parity=[n o e], Stop Bits=[1..2] |
| Return value | Describe |
| [SUCCESS]Set serial guest mode config done. | Successfully set TX serial port parameters |
| Example |  |
| TELNET login CTL100H Set TX1 to enable serial port Guest mode, baud rate 115200, 8-bit data bits, no checksum, 1-bit stop bit, enter the command: SET ENC 1 GUEST ON BR 9 BIT 8N1 return: [SUCCESS]Set serial guest mode config done. |  |

Activate TX serial port Guest mode

| API interface |  |
| --- | --- |
| SET ENC [enc] GUEST |  |
| Describe |  |
| Activate TX serial port Guest mode, only effective when the serial port parameter is set to GUEST ON |  |
| Parameter | Describe |
| enc enc | [001…762]: TX ID号 [001... 762]: TX ID number |
| Return value | Describe |
| None |  |
| Example |  |
| TELNET login CTL100H Enable TX1 serial port Guest mode, enter the command: SET ENC 1 GUEST |  |

Exit TX serial port Guest mode

| API interface |  |
| --- | --- |
| EXITGUEST |  |
| Describe |  |
| After starting the TX serial port Guest mode, send EXITGUEST to exit Guest mode |  |
| Parameter | Describe |
| None |  |
| Return value | Describe |
| None |  |
| Example |  |
| TELNET login CTL100H Exit TX1 serial port Guest mode, enter the command: EXITGUEST |  |

Set TX IP mode

| API interface |  |
| --- | --- |
| SET ENC [enc] IPMODE DHCP/STATIC |  |
| Describe |  |
| Set the IP mode of TX |  |
| Parameter | Describe |
| enc | [001…762]: TX ID number 0: All TX |
| DHCP | Dynamic IP |
| STATIC | Static IP |
| Return value | Describe |
| [SUCCESS]Set encoder 001 ip mode to dhcp. Use "SET ENC xx NETWORK REBOOT" command to apply new config!!! | Set successfully, TX network needs to be restarted for it to take effect |
| Example |  |
| TELNET login CTL100H Set TX1 to dynamic IP mode, enter the command: SET ENC 1 IPMODE DHCP return: [SUCCESS]Set encoder 001 ip mode to dhcp. Use "SET ENC xx NETWORK REBOOT" command to apply new config!!! |  |

Set TX IP address

| API interface |  |
| --- | --- |
| SET ENC [enc] STATIC IP [ip] |  |
| Describe |  |
| Set the IP address of TX, which is only valid when IP MODE is set to STATIC. |  |
| Parameter | Describe |
| enc | [001…762]: TX ID number |
| ip | IP address, such as 169.254.10.10 |
| Return value | Describe |
| [SUCCESS]Set encoder 001 IP address to 169.254.020.006. Use "SET ENC xx NETWORK REBOOT" command to apply new config!!! | Set successfully, TX network needs to be restarted for it to take effect |
| Example |  |
| TELNET login CTL100H Set the IP of TX1 to 169.254.20.6, enter the command: SET ENC 1 STATIC IP 169.254.20.6 return: [SUCCESS]Set encoder 001 IP address to 169.254.020.006. Use "SET ENC xx NETWORK REBOOT" command to apply new config!!! |  |

Set TX subnet mask

| API interface |  |
| --- | --- |
| SET ENC [enc] STATIC MASK [mask] |  |
| Describe |  |
| Set the subnet mask for TX, which is only valid when IP MODE is set to STATIC. |  |
| Parameter | Describe |
| enc | [001…762]: TX ID number 0: All TX |
| mask | Subnet mask, such as 255.255.0.0 |
| Return value | Describe |
| [SUCCESS]Set encoder 001 subnet mask address to 255.255.000.000. Use "SET ENC xx NETWORK REBOOT" command to apply new config!!! | Set successfully, TX network needs to be restarted for it to take effect |
| Example |  |
| TELNET login CTL100H Set the subnet mask of TX1 to 255.255.0.0, enter the command: SET ENC 1 STATIC MASK 255.255.0.0 return: [SUCCESS]Set encoder 001 subnet mask address to 255.255.000.000. Use "SET ENC xx NETWORK REBOOT" command to apply new config!!! |  |

Set TX gateway address

| API interface |  |
| --- | --- |
| SET ENC [enc] STATIC GATEWAY [gw] |  |
| Describe |  |
| Set the gateway address of TX, which is only valid when IP MODE is set to STATIC. |  |
| Parameter | Describe |
| enc | [001…762]: TX ID number 0: All TX |
| gw | Gateway address, such as 169.254.0.1 |
| Return value | Describe |
| [SUCCESS]Set encoder 001 gateway address to 169.254.000.001. Use "SET ENC xx NETWORK REBOOT" command to apply new config!!! | Set successfully, TX network needs to be restarted for it to take effect |
| Example |  |
| TELNET login CTL100H Set the gateway address of TX to 169.254.0.1, enter the command: SET ENC 1 STATIC GATEWAY 169.254.0.1 return: [SUCCESS]Set encoder 001 gateway address to 169.254.000.001. Use "SET ENC xx NETWORK REBOOT" command to apply new config!!! |  |

Set TX network reboot

| API interface |  |
| --- | --- |
| SET ENC [enc] NETWORK REBOOT |  |
| Describe |  |
| Set TX's network reboot |  |
| Parameter | Describe |
| enc | [001…762]: TX ID number 0: All TX |
| Return value | Describe |
| [SUCCESS]Set encoder 001 reboot and apply all the new config. | Set successfully |
| Example |  |
| TELNET login CTL100H Reboot the network of TX1 and enter the command: SET ENC 1 NETWORK REBOOT return: [SUCCESS]Set encoder 001 reboot and apply all the new config. |  |

Remove TX from the system

| API interface |  |
| --- | --- |
| SET ENC [enc] DELETE |  |
| Describe |  |
| Delete TX |  |
| Parameter | Describe |
| enc | [001…762]: TX ID number 0: All TX |
| Return value | Describe |
| [SUCCESS]Delete encoder 001 done. | Delete TX1 successfully |
| Example |  |
| TELNET login CTL100H Delete TX1, enter the command: SET ENC 1 DELETE return: [SUCCESS]Delete encoder 001 done. |  |

Reboot TX

| API interface |  |
| --- | --- |
| SET ENC [enc] REBOOT |  |
| Describe |  |
| Reboot TX |  |
| Parameter | Describe |
| enc | [001…762]: TX ID number 0: All TX |
| Return value | Describe |
| [SUCCESS]Set encoder 001 reboot and apply all the new config. | Reboot successfully |
| Example |  |
| TELNET login CTL100H Reboot TX1 and enter the command: SET ENC 1 REBOOT return: [SUCCESS]Set encoder 001 reboot and apply all the new config. |  |

Reset TX

| API interface |  |
| --- | --- |
| SET ENC [enc] RESET |  |
| Describe |  |
| 复位TX Reset TX |  |
| Parameter | Describe |
| enc | [001…762]: TX ID number 0: All TX |
| Return value | Describe |
| [SUCCESS]Set encoder 001 reset to default setting. | Reset successfully |
| Example |  |
| TELNET login CTL100H Reset TX1, enter the command: SET ENC 1 RESET return: [SUCCESS]Set encoder 001 reset to default setting. |  |

Obtain TX status information

| API interface |  |
| --- | --- |
| GET ENC [enc] STATUS |  |
| Describe |  |
| Obtain the status information of TX, without the parameter enc, to obtain the status information of all TX, i.e. GET ENC STATUS and GET ENC 0 STATUS. |  |
| Parameter | Describe |
| enc | [001…762]: TX ID number 0: All TX |
| Return value | Describe |
| Return TX status information | Include version number, network information, and other status information |
| Example |  |
| TELNET login CTL100H： Obtain the status information of TX1, enter the command: GET ENC 1 STATUS return： ================================================================ IP Control Box CTL100H Encoder Info FW Version: 2.03.19 In Net Sig Ver EDID Aud MCast Name 001 On Off 2.00.06 DF000 HDMI On Encoder 001 >>LED SGEn/Br/Bit 9 Off /9/8n1 >>MAC 6c:df:fb:08:5e:b9 >>IP GW SM 169.254.010.001 169.254.008.001 255.255.000.000 ================================================================ |  |

Set TX preset IP mode

| API interface |  |
| --- | --- |
| SET ENC PRESET IPMODE [mode] |  |
| Describe |  |
| Set the IP mode for TX preset, and assign TX's IP based on this preset mode when adding TX to the system |  |
| Parameter | Describe |
| mode | 0:AUTOIP 1:DHCP 2:STATIC |
| Return value | Describe |
| [SUCCESS]Set encoder preset IP to static mode. | Set TX preset IP mode to static successfully |
| Example |  |
| TELNET login CTL100H Set the TX preset IP mode to static, enter the command: SET ENC PRESET IPMODE 2 return: [SUCCESS]Set encoder preset IP to static mode. |  |

Set TX preset IP starting address

| API interface |  |
| --- | --- |
| SET ENC PRESET START IP [ip] |  |
| Describe |  |
| Set the IP starting address for TX preset |  |
| Parameter | Describe |
| ip | IP address, such as 169.254.10.10 |
| Return value | Describe |
| [SUCCESS]Set encoder preset IP min 172.016.010.001. | Set the IP starting address for TX preset to 172.16.10.1 |
| Example |  |
| TELNET login CTL100H 设置TX预设的IP起始地址为172.16.10.1，输入命令： Set the IP starting address for TX preset to 172.16.10.1, enter the command: SET ENC PRESET START IP 172.16.10.1 return: [SUCCESS]Set encoder preset IP min 172.016.010.001. |  |

Set TX preset IP end address

| API interface |  |
| --- | --- |
| SET ENC PRESET END IP [ip] |  |
| Describe |  |
| Set the IP end address for TX preset, which should be greater than the starting address and within the same network segment. |  |
| Parameter | Describe |
| ip | IP address, such as 169.254.20.10 |
| Return value | Describe |
| [SUCCESS]Set encoder preset IP max 172.016.010.200. | Set the IP end address for TX preset to 172.16.10.200 |
| Example |  |
| TELNET login CTL100H Set the IP end address for TX preset to 172.16.10.200, enter the command: SET ENC PRESET END IP 172.16.10.200 return: [SUCCESS]Set encoder preset IP max 172.016.010.200. |  |

Set TX preset subnet mask

| API interface |  |
| --- | --- |
| SET ENC PRESET SM [mask] |  |
| Describe |  |
| Set the subnet mask for TX preset |  |
| Parameter | Describe |
| mask | Subnet mask, such as 255.255.0.0 |
| Return value | Describe |
| [SUCCESS]Set encoder preset netmask 255.255.000.000. | Set the subnet mask for TX preset to 255.255.0.0 |
| Example |  |
| TELNET login CTL100H Set the subnet mask for TX preset to 255.255.0.0, enter the command: SET ENC PRESET SM 255.255.0.0 return: [SUCCESS]Set encoder preset netmask 255.255.000.000. |  |

Set TX preset gateway address

| API interface |  |
| --- | --- |
| SET ENC PRESET GW [gw] |  |
| Describe |  |
| Set the gateway address for TX preset |  |
| Parameter | Describe |
| gw | Gateway address, such as 169.254.0.1 |
| Return value | Describe |
| [SUCCESS]Set encoder preset gateway 172.016.010.001. | Set the gateway address for TX preset to 172.16.10.1 |
| Example |  |
| TELNET login CTL100H Set the gateway address for TX preset to 172.16.10.1, enter the command: SET ENC PRESET GW 172.16.10.1 return: [SUCCESS]Set encoder preset gateway 172.016.010.001. |  |

Save TX preset configuration

| API interface |  |
| --- | --- |
| SET ENC PRESET APPLY |  |
| Describe |  |
| Save the preset configuration of TX, after setting the preset IP mode and other settings, you need to call APPLY to save it. |  |
| Parameter | Describe |
| Return value | Describe |
| [SUCCESS]Set encoder preset IP done. [SUCCESS]Set encoder preset IP done. | Save TX preset configuration successfully |
| Example |  |
| TELNET login CTL100H Save TX preset configuration, enter the command: SET ENC PRESET APPLY return: [SUCCESS]Set encoder preset IP done. |  |

Video Wall Module API Reference

Create a video wall

| API interface |  |
| --- | --- |
| CREATE WALL HANDLE [hdl] |  |
| Describe |  |
| Create a video wall |  |
| Parameter | Describe |
| hdl | [01...09]: Video Wall ID Number |
| Return value | Describe |
| [SUCCESS]Create video wall 1. | Create a video wall with ID 1 |
| Example |  |
| TELNET login CTL100H Create a video wall with ID 1, enter the command: CREATE WALL HANDLE 1 return: [SUCCESS]Create video wall 1. |  |

Delete video wall

| API interface |  |
| --- | --- |
| DELETE WALL HANDLE [hdl] |  |
| Describe |  |
| Delete video wall |  |
| Parameter | Describe |
| hdl | [01...09]: Video Wall ID Number |
| Return value | Describe |
| [SUCCESS]Delete videowall 1. | Delete Video Wall 1 |
| Example |  |
| TELNET login CTL100H Delete Video Wall 1, enter the command: DELETE WALL HANDLE 1 return: [SUCCESS]Delete videowall 1. |  |

Change the name of the video wall

| API interface |  |
| --- | --- |
| SET WALL [hdl] NAME [name] |  |
| Describe |  |
| Change the name of the video wall |  |
| Parameter | Describe |
| hdl | [01...09]: Video Wall ID Number |
| name | Video wall name, maximum 16 characters |
| Return value | Describe |
| [SUCCESS]Rename video wall 1: VW1. | Rename Video Wall 1 as VW1 |
| Example |  |
| TELNET login CTL100H Rename Video Wall 1 as VW1, enter the command: SET WALL 1 NAME VW1 return: [SUCCESS]Rename video wall 1: VW1. |  |

Set the size of the video wall

| API interface |  |
| --- | --- |
| SET WALL [hdl] C [c] R [r] |  |
| Describe |  |
| Set the size of the video wall. |  |
| Parameter | Describe |
| hdl | [01...09]: Video Wall ID Number |
| c | Number of video wall columns |
| r | Number of video wall rows |
| Return value | Describe |
| [SUCCESS]Create video wall 1: NULL. | Successfully set up video wall 1 |
| Example |  |
| TELNET login CTL100H Set up a 2x2 video wall, enter the command: SET WALL 1 C 2 R 2 return: [SUCCESS]Create video wall 1: NULL. |  |

Assign RX to Video Wall

| API interface |  |
| --- | --- |
| SET WALL [hdl] DEC [dec] H [h] V [v] |  |
| Describe |  |
| Assign RX to Video Wall |  |
| Parameter | Describe |
| hdl | [01...09]: Video Wall ID Number |
| dec | [001...762]: RX ID number |
| h | Column number of video wall |
| v | Line number of the video wall |
| Return value | Describe |
| [SUCCESS]Assign decoder 001 to video wall 1. | Assign RX1 to Video Wall 1 |
| Example |  |
| TELNET login CTL100H Assign RX1 to the first row and column of Video Wall 1, enter the command: SET WALL 1 DEC 1 H 1 V 1 return: [SUCCESS]Assign decoder 001 to video wall 1. |  |

Create a video wall preset

| API interface |  |
| --- | --- |
| CREATE WALL [hdl] PRESET [prs] |  |
| Describe |  |
| Create a video wall preset. A preset will be created automatically when creating a new video wall. |  |
| Parameter | Describe |
| hdl | [01...09]: Video Wall ID Number |
| prs | [01...09]: Preset ID Number |
| Return value | Describe |
| [SUCCESS]Create preset 2: NULL. | Create preset successfully |
| Example |  |
| TELNET login CTL100H Create preset 2 for video wall 1, enter the command: CREATE WALL 1 PRESET 2 return: [SUCCESS]Create preset 2: NULL. |  |

Delete video wall preset

| API interface |  |
| --- | --- |
| DELETE WALL [hdl] PRESET [prs] |  |
| Describe |  |
| Delete the video wall preset. |  |
| Parameter | Describe |
| hdl | [01...09]: Video Wall ID Number |
| prs | [01...09]: Preset ID Number |
| Return value | Describe |
| [SUCCESS]Delete preset: Preset 2. | Delete preset successfully |
| Example |  |
| TELNET login CTL100H 视频墙1删除预设2，输入命令： Delete Preset 2 of Video Wall 1, enter the command: DELETE WALL 1 PRESET 2 return: [SUCCESS]Delete preset: Preset 2. |  |

Modify the preset name of the video wall

| API interface |  |
| --- | --- |
| SET WALL [hdl] PRESET [prs] NAME [name] |  |
| Describe |  |
| Modify the preset name of the video wall |  |
| Parameter | Describe |
| hdl | [01...09]: Video Wall ID Number |
| prs | [01...09]: Preset ID Number |
| name | Preset name, supporting up to 16 characters |
| Return value | Describe |
| [SUCCESS]Rename preset 1: TEST1. | Change the preset name of the video wall to TEST 1 |
| Example |  |
| TELNET login CTL100H Change the name of video wall 1 preset 1 to TEST 1, enter the command: SET WALL 1 PRESET 1 NAME TEST1 return: [SUCCESS]Rename preset 1: TEST1. |  |

Activate video wall preset

| API interface |  |
| --- | --- |
| APPLY WALL [hdl] PRESET [prs] |  |
| Describe |  |
| Activate the video wall preset. |  |
| Parameter | Describe |
| hdl | [01...09]: Video Wall ID Number |
| prs | [01...09]: Preset ID Number |
| Return value | Describe |
| [SUCCESS]Apply preset: Preset 1. | Startup preset successfully |
| Example |  |
| TELNET login CTL100H Activate Video Wall 1 Preset 1, enter the command: APPLY WALL 1 PRESET 1 return: [SUCCESS]Apply preset: Preset 1. |  |

Set video wall preset grouping

| API interface |  |
| --- | --- |
| SET WALL [hdl] PRESET [prs] CLASS [cls] H [h] V [v] |  |
| Describe |  |
| Set the video wall preset grouping, and when creating the preset, all RXs will be in Class A by default. |  |
| Parameter | Describe |
| hdl | [01...09]: Video Wall ID Number |
| prs | [01...09]: Preset ID Number |
| cls | [A...G]: Group ID number |
| h | Column number of video wall |
| v | Line number of the video wall |
| Return value | Describe |
| [SUCCESS]Done. | Group creation successfully |
| Example |  |
| TELNET login CTL100H Set the screen of the first row and second column of Video Wall 1 to preset Class B, enter the command: SET WALL 1 PRESET 1 CLASS B H 2 V 1 return: [SUCCESS]Done. |  |

Set the signal source for the preset grouping of the video wall

| API interface |  |
| --- | --- |
| SET WALL [hdl] PRESET [prs] CLASS [cls] SOURCE [enc] |  |
| Describe |  |
| Set the signal source for the preset grouping of the video wall. |  |
| Parameter | Describe |
| hdl | [01...09]: Video Wall ID Number |
| prs | [01...09]: Preset ID Number |
| cls | [A...G]: Class ID number |
| enc | [001...762]: TX ID number 0: Cancel routing |
| Return value | Describe |
| [SUCCESS]Done. | Set successfully |
| Example |  |
| TELNET login CTL100H Set the signal source for Class B of Video Wall 1 to TX1. Enter the command: SET WALL 1 PRESET 1 CLASS B SOURCE 1 return: [SUCCESS]Done. |  |

Set the preset matrix group for the video wall

| API interface |  |
| --- | --- |
| SET WALL [hdl] PRESET [prs] MATRIX H [h] V [v] |  |
| Describe |  |
| Set the preset matrix group for the video wall, with the RX in matrix mode. |  |
| Parameter | Describe |
| hdl | [01...09]: Video Wall ID Number |
| prs | [01...09]: Preset ID Number |
| h | Column numbers of video wall |
| v | Line numbers of the video wall |
| Return value | Describe |
| [SUCCESS]Done. | Successfully set matrix group |
| Example |  |
| TELNET login CTL100H Set the screen of the first row and second column of Video Wall 1 as the preset matrix group, enter the command: SET WALL 1 PRESET 1 MATRIX H 2 V 1 return: [SUCCESS]Done. |  |

Set the signal source for the preset matrix group of the video wall

| API interface |  |
| --- | --- |
| SET WALL [hdl] PRESET [prs] MATRIX H [h] V [v] SOURCE [enc] |  |
| Describe |  |
| Set the signal source for the preset matrix group of the video wall. |  |
| Parameter | Describe |
| hdl | [01...09]: Video Wall ID Number |
| prs | [01...09]: Preset ID Number |
| h | Column number of video wall |
| v | Line number of the video wall |
| enc | [001...762]: TX ID number 0: Cancel routing |
| Return value | Describe |
| [SUCCESS]Done. | Set successfully |
| Example |  |
| TELNET login CTL100H Set the signal source for the screen in the first row and second column of the preset matrix group of Video Wall 1 to TX1. Enter the command: SET WALL 1 PRESET 1 MATRIX H 2 V 1 SOURCE 1 return: [SUCCESS]Done. |  |

Set the width bezel of the video wall screen

| API interface |  |
| --- | --- |
| SET WALL [hdl] H [h] V [v] WIDTH BEZEL BW [b] IW [i] |  |
| Describe |  |
| Set the width bezel of the video wall screen. |  |
| Parameter | Describe |
| hdl | [01...09]: Video Wall ID Number |
| h | Column number of video wall |
| v | Line number of the video wall |
| b | [100...1000]: Original image width |
| i | [100...1000]: Visible image width, i cannot be greater than b |
| Return value | Describe |
| [SUCCESS]Done. | Set successfully |
| Example |  |
| TELNET login CTL100H Set the screen width bezel of the first row and first column of Video Wall 1 to be cropped by 10%, (BW - IW)/BW=10%, enter the command: SET WALL 1 H 1 V 1 WIDTH BEZEL BW 1000 IW 900 return: [SUCCESS]Done. |  |

Set the height bezel of the video wall screen

| API interface |  |
| --- | --- |
| SET WALL [hdl] H [h] V [v] HEIGHT BEZEL BH [b] IH [i] |  |
| Describe |  |
| Set the height bezel of the video wall screen. |  |
| Parameter | Describe |
| hdl | [01...09]: Video Wall ID Number |
| h | Column number of video wall |
| v | Line number of the video wall |
| b | [100...1000]: Original image height |
| i | [100...1000]: Visible image height, i cannot be greater than b |
| Return value | Describe |
| [SUCCESS]Done. | Set successfully |
| Example |  |
| TELNET login CTL100H Set the screen height bezel of the first row and first column of Video Wall 1 to be cropped by 10%, (BH - IH)/BH=10%, enter the command: SET WALL 1 H 1 V 1 HEIGHT BEZEL BH 1000 IH 900 return: [SUCCESS]Done. |  |

Get the status of the video wall

| API interface |  |
| --- | --- |
| GET WALL [hdl] STATUS |  |
| Describe |  |
| Get the status of the video wall |  |
| Parameter | Describe |
| hdl | [01...09]: Video Wall ID Number |
| Return value | Describe |
| Print video wall status information |  |
| Example |  |
| TELNET login CTL100H Get the status information of Video Wall 1, enter the command: GET WALL 1 STATUS return: ================================================================ IP Control Box CTL100H Video Wall Info FW Version: 2.03.19 VW Col Row CfgSel Name 01 02 02 01 Video Wall 1 OutID 003 --- --- --- CFG Name 01 Preset 1 Group FromIn Screen A 001 H01V01 H02V01 H01V02 H02V02 ================================================================ |  |

System Management Module API Reference

Device Search

| API interface |  |
| --- | --- |
| SEARCH |  |
| Describe |  |
| Search for online devices |  |
| Parameter | Describe |
| None |  |
| Return value | Describe |
| Return the information of the current system's online device |  |
| Example |  |
| TELNET login CTL100H Search online devices, enter the command： SEARCH return： [SUCCESS]More device in network will take more time to finish scan, please wait...done. ================================================================ Scan Device Result Info ==New Encoder None ==System Control Encoder Index IP MAC ID 001 169.254.010.001 6c:df:fb:08:5e:b9 001 ==New Decoder None ==System Control Decoder Index IP MAC ID 001 169.254.020.003 6c: df:fb:07:cf:c6 003 ================================================================ |  |

View device search results

| API interface |  |
| --- | --- |
| GET SEARCH STATUS |  |
| Describe |  |
| View device search results |  |
| Parameter | Describe |
| None |  |
| Return value | Describe |
| Return the information of current system's online devices |  |
| Example |  |
| TELNET login CTL100H View the device search results, enter the command: GET SEARCH STATUS return： ================================================================ Scan Device Result Info ==New Encoder None ==System Control Encoder Index IP MAC ID 001 169.254.010.001 6c:df:fb:08:5e:b9 001 ==New Decoder None ==System Control Decoder Index IP MAC ID 001 169.254.020.003 6c:df:fb:07:cf:c6 003 ================================================================ |  |

Clear device search results

| API interface |  |
| --- | --- |
| SEARCH RESET |  |
| Describe |  |
| Clear device search results |  |
| Parameter | Describe |
| None |  |
| Return value | Describe |
| [SUCCESS]Reset search info. | Clear device search results |
| Example |  |
| TELNET login CTL100H Clear the device search results and enter the command: SEARCH RESET return: [SUCCESS]Reset search info. |  |

Automatically add new devices to the system

| API interface |  |
| --- | --- |
| ADD AUTO ALL |  |
| Describe |  |
| Automatically add new devices to the system. Before calling this interface, you need to call the SEARCH interface to search for online devices. |  |
| Parameter | Describe |
| None |  |
| Return value | Describe |
| [SUCCESS]Add scan index 001 device to decoder 003. [SUCCESS]Add scan index 002 device to decoder 004. [SUCCESS]Add scan index 003 device to decoder 002. [SUCCESS]Add scan index 004 device to decoder 001. | Add new devices successfully |
| Example |  |
| TELNET login CTL100H Automatically add new devices to the system, enter the command: ADD AUTO ALL return： [SUCCESS]Add scan index 001 device to decoder 003. [SUCCESS]Add scan index 002 device to decoder 004. [SUCCESS]Add scan index 003 device to decoder 002. [SUCCESS]Add scan index 004 device to decoder 001. |  |

Add a new TX device to the system

| API interface |  |
| --- | --- |
| ADD DEV [dev] ENC [enc] |  |
| Describe |  |
| Add a new TX device to the system. Before calling this interface, you need to call the SEARCH interface to search for online devices. |  |
| Parameter | Describe |
| dev | Index of New Encoder in SEARCH results |
| enc | [001...762]: TX ID number 0: The system automatically assigns IDs |
| Return value | Describe |
| [SUCCESS]Add scan index 001 device to encoder 004. | Add the TX device with Index 1 from the New Encoder to the system and assign ID 4 |
| Example |  |
| TELNET login CTL100H Add the TX device with Index 1 from the New Encoder to the system and assign ID 4. Enter the command: ADD DEV 1 ENC 4 return: [SUCCESS]Add scan index 001 device to encoder 004. |  |

Add a new RX device to the system

| API interface |  |
| --- | --- |
| ADD DEV [dev] DEC [dec] |  |
| Describe |  |
| Add a new RX device to the system. Before calling this interface, you need to call the SEARCH interface to search for online devices. |  |
| Parameter | Describe |
| dev | Index of New Decoder in SEARCH results |
| dec | [001...762]: RX ID number 0: The system automatically assigns IDs |
| Return value | Describe |
| [SUCCESS]Add scan index 001 device to decoder 001. | Add the RX device with Index 1 from the New Decoder to the system and assign ID 1 |
| Example |  |
| TELNET login CTL100H Add the RX device with Index 1 from the New Decoder to the system and assign ID 1. Enter the command: ADD DEV 1 DEC 1 return: [SUCCESS]Add scan index 001 device to decoder 001. |  |

Clear existing devices from the system

| API interface |  |
| --- | --- |
| ADD DEV RESET |  |
| Describe |  |
| Clearing existing devices in the system will also clear information related to the devices, such as VW. |  |
| Parameter | Describe |
| None |  |
| Return value | Describe |
| [SUCCESS]Reset all Encoder/Decoder/Videowall/Search configuration. | Clear the existing devices and related configurations in the system |
| Example |  |
| TELNET login CTL100H Clear the existing devices in the system and enter the command: ADD DEV RESET return: [SUCCESS]Reset all Encoder/Decoder/Videowall/Search configuration. |  |

CTL100H Network Configuration API Reference

Set CTL100H IP mode

| API interface |  |
| --- | --- |
| SET NETWORK [lan] DHCP ON/OFF |  |
| Describe |  |
| Set the IP mode for CTL100H Control LAN port and Video LAN port. |  |
| Parameter | Describe |
| lan | LAN1: Video LAN port LAN2: Control LAN port |
| ON/OFF | ON: Enable DHCP OFF: Disable DHCP and use static IP |
| Return value | Describe |
| [SUCCESS]Set lan2 DHCP to on. Use "SET NETWORK REBOOT" command or repower device to apply new config!!! | Set the Control LAN port to DHCP mode |
| Example |  |
| TELNET login CTL100H Set the Control LAN port to DHCP mode and enter the command: SET NETWORK LAN2 DHCP ON return: [SUCCESS]Set lan2 DHCP to on. Use "SET NETWORK REBOOT" command or repower device to apply new config!!! |  |

Set CTL100H IP address

| API interface |  |
| --- | --- |
| SET NETWORK [lan] STATIC IP [ip] |  |
| Describe |  |
| Set the IP addresses of CTL100H Control LAN port and Video LAN port, which is only valid when the network port is set to static IP mode. |  |
| Parameter | Describe |
| lan | LAN1: VIDEO LAN port LAN2: CONTROL LAN port |
| ip | IP address, such as 169.254.20.10 |
| Return value | Describe |
| [SUCCESS]Set lan2 IP address to 192.168.070.040. Use "SET NETWORK REBOOT" command or repower device to apply new config!!! | Set the IP address of CONTROL LAN port to 192.168.70.40 |
| Example |  |
| TELNET login CTL100H Set the IP address of the CONTROL LAN port to192.168.70.40, enter the command: SET NETWORK LAN2 STATIC IP 192.168.70.40 return： [SUCCESS]Set lan2 IP address to 192.168.070.040. Use "SET NETWORK REBOOT" command or repower device to apply new config!!! |  |

Set CTL100H gateway address

| API interface |  |
| --- | --- |
| SET NETWORK [lan] STATIC GATEWAY [gw] |  |
| Describe |  |
| Set the gateway addresses for CTL100H Control LAN port and Video LAN port, which is only valid when the network port is set to static IP mode. |  |
| Parameter | Describe |
| lan | LAN1: Video LAN port LAN2: Control LAN port |
| gw | Gateway address, such as 169.254.0.1 |
| Return value | Describe |
| [SUCCESS]Set lan2 gateway address to 192.168.070.001. Use "SET NETWORK REBOOT" command or repower device to apply new config!!! | Set the gateway address for the Control LAN port to 192.168.70.1 |
| Example |  |
| TELNET login CTL100H Set the gateway address for the Control LAN port to 192.168.70.1, enter the command: SET NETWORK LAN2 STATIC GATEWAY 192.168.70.1 NET RB return: [SUCCESS]Set lan2 gateway address to 192.168.070.001. Use "SET NETWORK REBOOT" command or repower device to apply new config!!! |  |

Set CTL100H subnet mask

| API interface |  |
| --- | --- |
| SET NETWORK [lan] STATIC MASK [mask] |  |
| Describe |  |
| Set the subnet mask for CTL100H Control LAN port and Video LAN port, which is only valid when the network port is set to static IP mode. |  |
| Parameter | Describe |
| lan | LAN1: Video LAN port LAN2: Control LAN port |
| mask | Subnet mask, such as 255.255.0.0 |
| Return value | Describe |
| [SUCCESS]Set lan2 subnet mask address to 255.255.255.000. Use "SET NETWORK REBOOT" command or repower device to apply new config!!! | Set the subnet mask of Control LAN port to 255.255.255.0 |
| Example |  |
| TELNET login CTL100H Set the subnet mask of Control LAN port to 255.255.255.0, enter the command: SET NETWORK LAN2 STATIC MASK 255.255.255.0 return： [SUCCESS]Set lan2 subnet mask address to 255.255.255.000. Use "SET NETWORK REBOOT" command or repower device to apply new config!!! |  |

Reboot CTL100H network

| API interface |  |
| --- | --- |
| SET NETWORK REBOOT |  |
| Describe |  |
| After rebooting the CTL100H network and modifying the network parameters, this API needs to be sent for the configuration to take effect. |  |
| Parameter | Describe |
| None |  |
| Return value | Describe |
| [SUCCESS]Set network reboot and apply new config. | Reboot network successfully |
| Example |  |
| TELNET login CTL100H Reboot the network and enter the command: SET NETWORK REBOOT： return: [SUCCESS]Set network reboot and apply new config. |  |

Set CTL100H TELNET on/off

| API interface |  |
| --- | --- |
| SET NETWORK TELNET ON/OFF |  |
| Describe |  |
| Set the CTL100H TELNET on/off (enabled by default). |  |
| Parameter | Describe |
| ON/OFF | ON：Enable TELNET OFF: Disable TELNET |
| Return value | Describe |
| [SUCCESS]Set telnet port on. | Enable TELNET |
| Example |  |
| TELNET login CTL100H Enable TELNET and enter the command: SET NETWORK TELNET ON return: [SUCCESS]Set telnet port on. |  |

Set CTL100H TELNET port number

| API interface |  |
| --- | --- |
| SET NETWORK TELNET PORT [port] |  |
| Describe |  |
| Set CTL100AL TELNET port number (23 by default). |  |
| Parameter | Describe |
| port | TELNET port number, maximum value 65535 |
| Return value | Describe |
| [SUCCESS]Set telnet port to 0030. | Set the TELNET port number to 30 |
| Example |  |
| TELNET login CTL100H Set the TELNET port number to 30 and enter the command: SET NETWORK TELNET PORT 30 return: [SUCCESS]Set telnet port to 0030. |  |

Set CTL100H SSH on/off

| API interface |  |
| --- | --- |
| SET NETWORK SSH ON/OFF |  |
| Describe |  |
| Set the CTL100H SSH on/off (disabled by default). |  |
| Parameter | Describe |
| ON/OFF | ON: Enable SSH OFF: Disable SSH |
| Return value | Describe |
| [SUCCESS]Set ssh port on. | Enable SSH |
| Example |  |
| TELNET login CTL100H Enable SSH and enter the command: SET NETWORK SSH ON return: [SUCCESS]Set ssh port on. |  |

Set CTL100H SSH port number

| API interface |  |
| --- | --- |
| SET NETWORK SSH PORT [port] |  |
| Describe |  |
| Set CTL100H SSH port number (22 by default). |  |
| Parameter | Describe |
| port | TELNET port number, maximum value 65535 |
| Return value | Describe |
| [SUCCESS]Set ssh port to 0030. | Set SSH port number to 30 |
| Example |  |
| TELNET login CTL100H Set the SSH port number to 30 and enter the command: SET NETWORK SSH PORT 30 return: [SUCCESS]Set ssh port to 0030. |  |

Set CTL100H HTTPS on/off

| API interface |  |
| --- | --- |
| SET NETWORK HTTPS ON/OFF |  |
| Describe |  |
| Set the CTL100H HTTPS on/off (enabled by default). |  |
| Parameter | Describe |
| ON/OFF | ON：Enable HTTPS OFF: Disable HTTPS |
| Return value | Describe |
| [SUCCESS]Set web gui https on. | Enable HTTPS |
| Example |  |
| TELNET login CTL100H Enable HTTPS, enter the command: SET NETWORK HTTPS ON return: [SUCCESS]Set web gui https on. |  |

Set CTL100H WEB GUI on/off

| API interface |  |
| --- | --- |
| SET NETWORK WEB ON/OFF |  |
| Describe |  |
| Set the CTL100H WEB on/off (enabled by default). |  |
| Parameter | Describe |
| ON/OFF | ON：Turn on WEB OFF: Turn off WEB |
| Return value | Describe |
| [SUCCESS]Set web gui on. | Turn on WEB |
| Example |  |
| TELNET login CTL100H Turn on WEB and enter the command: SET NETWORK WEB ON return: [SUCCESS]Set web gui on. |  |

Modify CTL100H domain name

| API interface |  |
| --- | --- |
| SET NETWORK DNS hostname |  |
| Describe |  |
| Change the domain name of CTL100H, the default domain name is controller.local. |  |
| Parameter | Describe |
| hostname | Domain names only support letters, numbers, and special characters such as - _. |
| Return value | Describe |
| [SUCCESS]Set DNS domain name to test.local. System will restart, please wait... | Set the domain name to test.local |
| Example |  |
| TELNET login CTL100H Set the domain name to test.local and enter the command: SET NETWORK DNS test return: [SUCCESS]Set DNS domain name to test.local. System will restart, please wait... |  |