| 履历页 Resume page |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 序号 Serial number | 更改内容摘要描述 Change the content summary description |  | 版本 edition | 修订人 Revisionist | 修订日期 Revision date |
| 1 one | 新版发行 New Release |  | 1.0 one | 张泽辉 Zhang Zehui | 2024.07.19 2024.07.19 |
| 批 准： approval: 日 期： Date: |  | 审 核： to examine: 日 期： Date: | 制 作： make: 日 期： Date: |  |  |

目录

1. 概述Summary	7

1.1 简介Brief Introduction	7

1.2 使用说明Instructions	7

1.3 重要概念Important Concepts	8

2. CTL100D系统API参考 CTL100D System API Reference	10

2.1 帮助信息Help Information	10

2.2 状态信息输出Status Information Output	10

2.3 配置CTL100D后面板GPIO方向	12

Configure the GPIO direction on the rear panel of CTL100D	12

2.4 配置CTL100D后面板GPIO输出电平	14

Configure the GPIO output level on the rear panel of CTL100D	14

2.5 获取CTL100D后面板GPIO输入电平	15

Obtain the GPIO input level on the rear panel of CTL100D	15

2.6 获取CTL100D后面板GPIO状态信息	16

Obtain GPIO status information on the rear panel of CTL100D	16

2.7 设置CTL100D串口波特率	17

Set CTL100D serial port baud rate	17

2.8 复位CTL100D系统配置	18

Reset CTL100D system configuration	18

2.9 复位CTL100D网络配置	19

Reset CTL100D network configuration	19

2.10 复位CTL100D所有配置	20

Reset all configurations of CTL100D	20

3. RX控制模块API参考 RX Control Module API Reference	22

3.1 设置RX ID号 Set RX ID number	22

3.2 设置RX名称 Set RX name	22

3.3 设置RX路由Set RX routing	23

3.4 锁定RX VIDEO路由Lock RX video routing	24

3.5 锁定RX AUDIO路由Lock RX audio routing	26

3.6 锁定RX IR路由Lock RX IR routing	27

3.7 锁定RX RS-232路由Lock RX RS-232 routing	28

3.8 锁定RX USB路由Lock RX USB routing	29

3.9 控制RX DEC灯闪烁Control RX DEC light flashing	30

3.10 设置RX OSD开关 Set RX OSD switch	32

3.11 设置RX HDMI OUTPUT开关 Set RX HDMI OUTPUT switch	33

3.12 设置RX HDMI OUTPUT MUTE Set RX HDMI OUTPUT MUTE	34

3.13 设置RX输出分辨率 Set RX output resolution	35

3.14 设置RX IR电平 Set RX IR level	37

3.15 设置RX图像输出模式 Set RX image output mode	38

3.16 发送CEC指令给RX（Guest mode） Send CEC command to RX (Guest mode)	39

3.17 设置RX串口参数 Set RX serial port parameters	40

3.18 启动RX串口Guest mode Activate RX serial port Guest mode	41

3.19 退出RX串口Guest mode Exit RX serial port Guest mode	42

3.20 从系统中删除RX Remove RX from the system	43

3.21 重启RX Reboot RX	43

3.22 复位RX Reset RX	44

3.23 获取RX状态信息 Obtain RX status information	45

3.24 设置RX预设IP模式 Set RX preset IP mode	47

3.25 设置RX预设IP起始地址 Set RX preset start IP	48

3.26 设置RX预设IP结束地址 Set RX preset end IP	49

3.27 设置RX预设子网掩码 Set RX preset subnet mask	50

3.28 设置RX预设网关地址 Set RX preset gateway address	51

3.29 保存RX预设配置 Save RX preset configuration	52

3.30 设置RX图像拉伸 Set RX image stretching	53

3.31 设置RX显示模式 Set RX display mode	54

3.32 切换RX为编码器模式 Switch RX to encoder mode	55

4. TX控制模块API参考 TX Control Module API Reference	56

4.1 设置TX ID号 Set TX ID number	56

4.2 设置TX名称 Set TX Name	57

4.3 控制TX ENC灯闪烁 Control the TX ENC LED to flash	58

4.4 设置TX音频源 Set TX audio source	59

4.5 设置TX EDID Set TX EDID	60

4.6 设置TX拷贝RX EDID Set TX to copy RX EDID	62

4.7 设置TX IR电平 Set TX IR level	63

4.8 发送CEC指令给TX（Guest mode） Send CEC command to TX (Guest mode)	64

4.9 设置TX串口参数 Set TX serial port parameters	65

4.10 启动TX串口Guest mode Activate TX serial port Guest mode	66

4.11 退出TX串口Guest mode Exit TX serial port Guest mode	67

4.12 从系统中删除TX Remove TX from the system	68

4.13 重启TX Reboot TX	68

4.14 复位TX Reset TX	69

4.15 获取TX状态信息 Obtain TX status information	70

4.16 设置TX预设IP模式 Set TX preset IP mode	72

4.17 设置TX预设IP起始地址 Set TX preset start IP	73

4.18 设置TX预设IP结束地址 Set TX preset end IP	74

4.19 设置TX预设子网掩码 Set TX preset subnet mask	75

4.20 设置TX预设网关地址 Set TX preset gateway address	76

4.21 保存TX预设配置 Save TX preset configuration	77

4.22 切换TX为解码器模式 Switch TX to decoder mode	77

5. TX SS模块API参考 TX SS module API reference	79

5.1 获取TX SS状态信息 Obtain TX SS status information	79

5.2 获取TX SS主流URL Obtain mainstream URL for TX SS	80

5.3 获取TX SS次流URL Get secondary stream URL for TX SS	81

5.4 获取TX SS抓图URL Get TX SS snapshot URL	82

5.5 设置TX SS主流参数 Set mainstream parameters for TX SS	83

5.6 设置TX SS次流参数 Set TX SS secondary stream parameters	85

5.7 设置TX SS主流开关 Set TX SS mainstream switch on/off	86

5.8 设置TX SS次流开关 Set TX SS sub stream switch on/off	87

5.9 设置TX SS抓图开关 Set TX SS snapshot switch on/off	88

6. 视频墙模块API参考 Video Wall Module API Reference	90

6.1 创建视频墙 Create a video wall	90

6.2 删除视频墙 Delete video wall	91

6.3 修改视频墙名称 Change the name of the video wall	91

6.4 设置视频墙大小 Set the size of the video wall	92

6.5 视频墙分配RX Assign RX to Video Wall	93

6.6 创建视频墙预设 Create a video wall preset	94

6.7 删除视频墙预设 Delete video wall preset	95

6.8 修改视频墙预设名称 Modify the preset name of the video wall	96

6.9 启动视频墙预设 Apply video wall preset	97

6.10 设置视频墙预设分组 Set video wall preset class	98

6.11 设置视频墙预设分组的信号源 Set the signal source for the preset class of the video wall	99

6.12 设置视频墙预设矩阵组 Set the preset matrix group for the video wall	100

6.13 设置视频墙预设矩阵组的信号源 Set the signal source for the preset matrix group of the video wall	101

6.14 设置视频墙屏幕宽度方向边框 Set the bezel width of the video wall screen	102

6.15 设置视频墙屏幕高度方向边框 Set the bezel height of the video wall screen	103

6.16 获取视频墙状态 Get the status of the video wall	104

7. 多视窗模块API参考 Multiview module API reference	106

7.1 创建多视窗 Create multiview	106

7.2 删除多视窗 Delete multiview	107

7.3 修改多视窗名称 Change the name of multiview	107

7.4 设置多视窗布局 Set up multiview layout	108

7.5 设置多视窗窗口信号源 Set the signal source for multiview	109

7.6 创建画中画布局 Create a multiview PIP layout	110

7.7 删除画中画布局 Delete multiview PIP layout	111

7.8 修改画中画布局名称 Modify the name of the PIP layout	112

7.9 修改画中画布局类型 Modify the layout type of PIP	113

7.10 使能画中画窗口 Enable PIP window	114

8. 矩阵预设模块API参考 Matrix Preset Module API Reference	116

8.1 创建矩阵预设 Create matrix presets	116

8.2 删除矩阵预设 Delete matrix preset	116

8.3 修改矩阵预设名称 Modify the preset name of the matrix	117

8.4 新增矩阵预设条目 Add preset matrix entry	118

8.5 删除矩阵预设条目 Delete matrix preset entry	119

8.6 应用矩阵预设 Apply matrix preset	120

9. 系统管理模块API参考 System Management Module API Reference	122

9.1 设备搜索 Device search	122

9.2 查看设备搜索结果 View device search results	123

9.3 清空设备搜索结果Clear device search results	124

9.4 自动加入新设备到系统Automatically add new devices to the system	125

9.5 加入新TX设备到系统 Add a new TX device to the system	126

9.6 加入新RX设备到系统 Add a new RX device to the system	127

9.7 清空系统中的已有设备Clear existing devices from the system	128

10. CTL100D网络配置API参考 CTL100D Network Configuration API Reference	130

10.1 设置CTL100D IP模式  Set CTL100D IP mode	130

10.2 设置CTL100D IP地址 Set CTL100D IP address	131

10.3 设置CTL100D网关地址 Set CTL100D gateway address	132

10.4 设置CTL100D子网掩码 Set CTL100D subnet mask	133

10.5 重启CTL100D网卡 Reboot CTL100D network	134

10.6 设置CTL100D TELNET开关 Set CTL100D TELNET on/off	135

10.7 设置CTL100D TELNET端口号 Set CTL100D TELNET port number	136

10.8 设置CTL100D SSH开关 Set CTL100D SSH on/off	137

10.9 设置CTL100D SSH端口号 Set CTL100D SSH port number	138

10.10 设置CTL100D HTTPS开关  Set CTL100D HTTPS on/off	138

10.11 设置CTL100D WEB开关  Set CTL100D WEB on/off	139

10.12 修改CTL100D域名 Modify CTL100D domain name	140

概述Summary

简介Brief Introduction

本文档用于介绍基于CTL100D相关API指令，本文档适用于测试工程师、软件工程师。

This document is used to introduce API commands based on CTL100D, and is suitable for testing engineers and software engineers.

使用说明Instructions

在对CTL100D使用API指令前，必须使用TELNET远程登录到相应终端或者使用串口终端，才能进行API指令交互。如下任一方式都可以进入控制终端进行API交互。

- TELNET登录到CTL100D，默认端口号23。

- 使用串口线连接CTL100D后面板串口与PC，在PC打开串口终端工具，选定相应串口号连接后就可以进入控制器终端进行API交互。默认波特率57600,8位数据位，1位停止位，没有校验位。

Before using API commands on CTL100D, it is necessary to remotely log in to the corresponding terminal using TELNET or use a serial port terminal to interact with API commands. Any of the following methods can be used to enter the control terminal for API interaction.

- TELNET login to CTL100D, default port number 23.

- Connect the serial port on the rear panel of CTL100D to the PC using a serial port cable. Open the serial port terminal tool on the PC, select the corresponding serial port number to connect, and then enter the controller terminal for API interaction. The default baud rate is 57600, with 8 data bits, 1 stop bit, and no parity bit.

重要概念Important Concepts

RX1：

下文中出现的RX1指的是ID号为1的RX。

TX1：

下文中出现的TX1指的是ID号为1的TX。

SS:

下文中出现的SS指的是TX的辅流模块。

RX1：

The RX1 mentioned in the following text refers to the RX with ID number 1.

TX1：

The TX1 mentioned in the following text refers to the TX with ID number 1.

SS:

The SS mentioned in the following text refers to the Secondary Stream module of TX.

CTL100D系统API参考 CTL100D System API Reference

帮助信息Help Information

| API接口 API interface |  |
| --- | --- |
| HELP 或者 ? HELP or ? |  |
| 描述 describe |  |
| 打印当前系统支持的API指令 Print API commands supported by the current system |  |
| 参数 parameter | 描述 describe |
| 无None |  |
| 返回值 Return value | 描述 describe |
| HELP信息 HELP Information | 打印HELP信息 Print HELP information |
| 样例 Example |  |
| TELNET登录CTL100D 输入命令： HELP 输入命令： ? TELNET login CTL100D Enter the command: HELP Enter the command: ? |  |

状态信息输出Status Information Output

| API接口 API interface |  |
| --- | --- |
| GET STATUS |  |
| 描述 describe |  |
| 输出CTL100D状态信息与当前工程已添加TX/RX状态信息。 Output CTL100D status information and TX/RX status information added to the current project. |  |
| 参数 parameter | 描述 describe |
| 无None |  |
| 返回值 Return value | 描述 describe |
| 状态信息 status information |  |
| 样例 Example |  |
| TELNET登录CTL100D 输入命令： GET STATUS 返回: TELNET login CTL100D Enter the command： GET STATUS Return: ================================================================ ================================================================ CTL100D Controller Status Info FW Version: 3.00.12 Power IR Baud On On 57600 In EDID IP NET/Sig SNAPURL 001 DF000 169.254.010.001 On /On 169.254.110.1 002 DF000 169.254.010.002 On /On 169.254.110.2 Out FromIn IP NET/HDMI Res Mode 001 002 169.254.020.001 On /On 07 MV 002 002 169.254.020.002 On /On 00 MX LAN DHCP IP Gateway SubnetMask 01_POE Off 169.254.008.100 169.254.008.001 255.255.000.000 02_CTRL On 192.168.006.100 192.168.006.001 255.255.255.000 (static:192.168.006.100 192.168.006.001 255.255.255.000) Telnet SSH2 HTTPS WEB LAN01 MAC LAN02 MAC 23 Off Off On 6C:DF:FB:00:01:8D 6C:DF:FB:00:01:81 Domain Name controller.local ================================================================ API Interface SET GPIO [gpio] DIR IN/OUT describe Configure the GPIO direction on the rear panel of CTL100D parameter describe gpio 1: GPIO1 2: GPIO2 3: GPIO3 LAN DHCP IP Gateway SubnetMask 01_POE Off 169.254.008.100 169.254.008.001 255.255.000.000 02_CTRL On 192.168.006.100 192.168.006.001 255.255.255.000 (static:192.168.006.100 192.168.006.001 255.255.255.000) Telnet SSH2 HTTPS WEB LAN01 MAC LAN02 MAC 23 Off Off On 6C:DF:FB:00:01:8D 6C:DF:FB:00:01:81 Domain Name controller.local ================================================================ ================================================================ |  |

配置CTL100D后面板GPIO方向

  - Configure the GPIO direction on the rear panel of CTL100D

| API接口 API interface |  |
| --- | --- |
| SET GPIO [gpio] DIR IN/OUT |  |
| 描述 describe |  |
| 配置CTL100D后面板GPIO方向 Configure the GPIO direction on the rear panel of CTL100D |  |
| 参数 parameter | 描述 describe |
| gpio | 1: GPIO1 2: GPIO2 3: GPIO3 4: GPIO4 |
| IN | GPIO作输入功能 GPIO for input function |
| OUT | GPIO作输出功能 GPIO for output function |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Set GPIO 01 as input port. | GPIO1配置为输入模式 GPO1 is configured as input mode |
| [SUCCESS]Set GPIO 01 as output port. | GPIO1配置为输出模式 GPO1 is configured as output mode |
| 样例 Example |  |
| TELNET登录CTL100D 配置GPIO1为输入模式，输入命令： SET GPIO 1 DIR IN 返回: [SUCCESS]Set GPIO 01 as input port. 配置GPIO1为输出模式，输入命令： SET GPIO 1 DIR OUT 返回: [SUCCESS]Set GPIO 01 as output port. TELNET login CTL100D Configure GPIO1 as input mode, enter the command: SET GPIO 1 DIR IN return: [SUCCESS]Set GPIO 01 as input port. Configure GPO1 as output mode, enter the command: SET GPIO 1 DIR OUT return: [SUCCESS]Set GPIO 01 as output port. |  |

配置CTL100D后面板GPIO输出电平

  - Configure the GPIO output level on the rear panel of CTL100D

| API接口 API interface |  |
| --- | --- |
| SET GPIO [gpio] LEVEL Low/High |  |
| 描述 describe |  |
| 配置CTL100D后面板GPIO输出电平，本API只对方向为输出的GPIO有作用。 Configure the GPIO output level on the rear panel of CTL100D. This API only works on GPIO with output direction. |  |
| 参数 parameter | 描述 describe |
| gpio | 1: GPIO1 2: GPIO2 3: GPIO3 4: GPIO4 |
| Low/High | Low: 输出低电平 High: 输出高电平 Low: Output low level High: Output high level |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Set GPIO 01 output level 0. | GPO1 output low level GPIO1输出低电平 |
| [SUCCESS]Set GPIO 01 output level 1. | GPO1 output high level GPIO1输出高电平 |
| 样例 Example |  |
| TELNET登录CTL100D 配置GPIO1输出低电平，输入命令： SET GPIO 1 LEVEL Low 返回: [SUCCESS]Set GPIO 01 output level 0. 配置GPIO1输出高电平，输入命令： SET GPIO 1 LEVEL High 返回: [SUCCESS]Set GPIO 01 output level 1. TELNET login CTL100D Configure GPO1 to output low level, enter the command: SET GPIO 1 LEVEL Low return: [SUCCESS]Set GPIO 01 output level 0. Configure GPO1 to output high level, enter the command: SET GPIO 1 LEVEL High return: [SUCCESS]Set GPIO 01 output level 1. |  |

获取CTL100D后面板GPIO输入电平

  - Obtain the GPIO input level on the rear panel of CTL100D

| API接口 API interface |  |
| --- | --- |
| GET GPIO [gpio] LEVEL |  |
| 描述 describe |  |
| 获取CTL100AL后面板GPIO输入电平，本API只对方向为输入的GPIO有作用。 Obtain the GPIO input level on the rear panel of CTL100D. This API only works on GPIO with input direction. |  |
| 参数 parameter | 描述 describe |
| gpio | 1: GPIO1 2: GPIO2 3: GPIO3 4: GPIO4 |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Get GPIO 01 real input level 1. | GPIO1获取输入电平为高电平 GPO1 obtains input level as high level |
| 样例 Example |  |
| TELNET登录CTL100D 获取GPIO1输入电平，输入命令： GET GPIO 1 LEVEL 返回: [SUCCESS]Get GPIO 01 real input level 1. TELNET login CTL100D Get GPO1 input level, enter the command: GET GPIO 1 LEVEL return: [SUCCESS]Get GPIO 01 real input level 1. |  |

获取CTL100D后面板GPIO状态信息

  - Obtain GPIO status information on the rear panel of CTL100D

| API接口 API interface |  |
| --- | --- |
| GET GPIO [gpio] STATUS |  |
| 描述 describe |  |
| 获取CTL100D后面板GPIO状态信息。 Obtain GPIO status information on the rear panel of CTL100D. |  |
| 参数 parameter | 描述 describe |
| gpio | 可选参数，没指定时代表获取所有GPIO状态Optional parameter, when not specified, represents obtaining all GPIO statuses 1: GPIO1 2: GPIO2 3: GPIO3 4: GPIO4 |
| 返回值 Return value | 描述 describe |
| 返回GPIO状态信息 Return GPIO status information |  |
| 样例 Example |  |
| TELNET登录CTL100D 获取GPIO1状态信息，输入命令： GET GPIO 1 STATUS 返回: TELNET login CTL100D Get GPO1 status information, enter the command: GET GPIO 1 STATUS return: ================================================================ ================================================================ CTL100D Controller GPIO Info FW Version: 3.00.12 GPIO DIR Set Get 01 In - 1 ================================================================ ================================================================ |  |

设置CTL100D串口波特率

  - Set CTL100D serial port baud rate

| API接口 API interface |  |
| --- | --- |
| SET RS232BAUDRATE [a] |  |
| 描述 describe |  |
| 设置CTL100AL串口波特率为a，出厂默认为57600 Set the baud rate of the CTL100D serial port to a, with a factory default of 57600 |  |
| 参数 parameter | 描述 describe |
| a | [0:115200 1:57600, 2:38400, 3:19200, 4:9600] |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Set RS232 Baud Rate to 115200bps. | 设置波特率为57600成功 Set baud rate to 57600 successfully |
| 样例 Example |  |
| TELNET登录CTL100D 设置串口波特率为115200，输入命令： SET RS232BAUDRATE 0 返回: [SUCCESS]Set RS-232 Baud Rate to 115200bps. 设置串口波特率为57600，输入命令： SET RS232BAUDRATE 1 返回: [SUCCESS]Set RS-232 Baud Rate to 57600bps. TELNET login CTL100D Set the serial port baud rate to 115200, enter the command: SET RS232BAUDRATE 0 return: [SUCCESS]Set RS-232 Baud Rate to 115200bps. Set the serial port baud rate to 57600, enter the command: SET RS232BAUDRATE 1 return: [SUCCESS]Set RS-232 Baud Rate to 57600bps. |  |

复位CTL100D系统配置

  - Reset CTL100D system configuration

| API接口 API interface |  |
| --- | --- |
| SET RESET |  |
| 描述 describe |  |
| 复位系统配置信息，清空已加入系统的设备。 Reset system configuration information and clear devices that have been added to the system. |  |
| 参数 parameter | 描述 describe |
| 无 None |  |
| 返回值 Return value | 描述 describe |
| [SUCCESS]System will reset to default config, it will take about 40 seconds, and RS-232 will disable during this time, please wait... | 复位成功 Reset successfully |
| 样例 Example |  |
| TELNET登录CTL100D 复位系统配置，输入命令： SET RESET 返回: Sure to RESET system to default settings? Type "Yes" after next prompt to confirm... 输入yes 返回: [SUCCESS]System will reset to default config, it will take about 40 seconds, and RS-232 will disable during this time, please wait... TELNET login CTL100D Reset the system configuration, enter the command: SET RESET return Sure to RESET system to default settings? Type "Yes" after next prompt to confirm... Enter yes return: [SUCCESS]System will reset to default config, it will take about 40 seconds, and RS-232 will disable during this time, please wait... |  |

复位CTL100D网络配置

  - Reset CTL100D network configuration

| API接口 API interface |  |
| --- | --- |
| SET RESET NETWORK |  |
| 描述 describe |  |
| 复位CTL100D网络配置信息。 Reset CTL100D network configuration. |  |
| 参数 parameter | 描述 describe |
| 无 None |  |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Network will reset to default config, it will take about 40 seconds, and RS232 will disable during this time, please wait... | 复位成功 Reset successfully |
| 样例 Example |  |
| TELNET登录CTL100D 复位系统配置，输入命令： SET RESET NETWORK 返回: Sure to RESET network config to default settings? Type "Yes" after next prompt to confirm... 输入yes 返回: [SUCCESS]Network will reset to default config, it will take about 40 seconds, and RS232 will disable during this time, please wait... TELNET login CTL100D Reset the network configuration, enter the command: SET RESET NETWORK return: Sure to RESET network config to default settings? Type "Yes" after next prompt to confirm... Enter yes return: [SUCCESS]Network will reset to default config, it will take about 40 seconds, and RS232 will disable during this time, please wait... |  |

复位CTL100D所有配置

  - Reset all configurations of CTL100D

| API接口 API interface |  |
| --- | --- |
| SET RESET ALL |  |
| 描述 describe |  |
| 复位CTL100D所有配置信息。 Reset CTL100D all configuration. |  |
| 参数 parameter | 描述 describe |
| 无 None |  |
| 返回值 Return value | 描述 describe |
| [SUCCESS]System and network will reset to default config, it will take about 40 seconds, and RS232 will disable during this time, please wait... | 复位成功 Reset successfully |
| 样例 Example |  |
| TELNET登录CTL100D 复位系统配置，输入命令： SET RESET ALL 返回: Sure to RESET system and network to default settings? Type "Yes" after next prompt to confirm... 输入yes 返回: [SUCCESS]System and network will reset to default config, it will take about 40 seconds, and RS232 will disable during this time, please wait... TELNET login CTL100D Reset all configuration, enter the command: SET RESET ALL return: Sure to RESET system and network to default settings? Type "Yes" after next prompt to confirm... Enter yes return: [SUCCESS]System and network will reset to default config, it will take about 40 seconds, and RS232 will disable during this time, please wait... |  |

RX控制模块API参考 RX Control Module API Reference

设置RX ID号 Set RX ID number

| API接口 API interface |  |
| --- | --- |
| SET DEC [dec] ID [id] |  |
| 描述 describe |  |
| 设置RX ID号。 Set the RX ID number. |  |
| 参数 parameter | 描述 describe |
| dec | [001…762]: RX ID号 [001…762]: RX ID number |
| id | [001…762]: 目标ID号 [001…762]: Target ID number |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Set decoder 001 ID to 760. | 设置RX1 ID号为760 Set RX1 ID to 760. |
| [ERROR]Decoder 100 does not exist. [ERROR]Decoder 100 does not exist. | RX100不存在 RX100 does not exist |
| 样例 Example |  |
| TELNET登录CTL100D RX1 ID设置为760，输入命令： SET DEC 1 ID 760 返回: [SUCCESS]Set decoder 001 ID to 760. TELNET login CTL100D Set RX1 ID to 760, enter the command: SET DEC 1 ID 760 return: [SUCCESS]Set decoder 001 ID to 760. |  |

设置RX名称 Set RX name

| API接口 API interface |  |
| --- | --- |
| SET DEC [dec] NAME [name] |  |
| 描述 describe |  |
| 设置RX名称。 Set RX name. |  |
| 参数 parameter | 描述 describe |
| dec | [001…762]: RX ID号 [001... 762]: RX ID number |
| name | 名字，最大长度为16个字节 Name, with a maximum length of 16 bytes |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Set decoder 001 name:TEST1. | 设置RX1名字为TEST1 Set RX1 name to TEST 1 |
| 样例 Example |  |
| TELNET登录CTL100D 设置RX1别名为TEST1, 输入命令： SET DEC 1 NAME TEST1 返回： [SUCCESS]Set decoder 001 name:TEST1. TELNET login CTL100D Set RX1 name as TEST 1, enter the command: SET DEC 1 NAME TEST1 return: [SUCCESS]Set decoder 001 name:TEST1. |  |

设置RX路由Set RX routing

| API接口 API interface |  |
| --- | --- |
| SET DEC [dec] SWITCH [enc] ALL |  |
| 描述 describe |  |
| 设置RX VIDEO/AUDIO/IR/RS-232/USB/路由 Set RX Video/AUDIO/IR/RS-232/USB/routing |  |
| 参数 parameter | 描述 describe |
| dec | [001…762]: RX ID号 0: 所有RX [001…762]: RX ID number 0: All RXs |
| enc | [001...254]: TX ID号 0: 取消路由 [001... 254]: TX ID number 0: Cancel routing |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Set decoder 001 from encoder 003. | 设置RX1 VIDEO/AUDIO/IR/RS-232/USB/信号路由到TX3 Set RX1 video/audio/IR/RS-232/USB signal routing to TX3 |
| 样例 Example |  |
| TELNET登录CTL100D 输入命令： SET DEC 1 SWITCH 3 ALL 返回： [SUCCESS]Set decoder 001 from encoder 003. 输入命令： SET DEC 1 SWITCH 0 ALL 返回： [SUCCESS]Set decoder 001 no source. TELNET login CTL100D Enter the command: SET DEC 1 SWITCH 3 ALL return: [SUCCESS]Set decoder 001 from encoder 003. Enter the command: SET DEC 1 SWITCH 0 ALL return: [SUCCESS]Set decoder 001 no source. |  |

锁定RX VIDEO路由Lock RX video routing

| API接口 API interface |  |
| --- | --- |
| SET DEC [dec] SWITCH [enc] VIDEO |  |
| 描述 describe |  |
| 锁定RX VIDEO信号路由。 Lock the RX Video signal routing. |  |
| parameter | 描述 describe |
| dec dec | [001…762]: RX ID号 0: 所有RX [001…762]: RX ID number 0: All RXs |
| enc | [001...254]: TX ID号 0: 取消锁定 [001...254]: TX ID number 0: Cancel the lock |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Set decoder 001 video from encoder 003. | 设置RX1锁定视频信号路由到TX3 Set RX1 video from encoder TX3. |
| 样例 Example |  |
| TELNET登录CTL100D 输入命令： SET DEC 1 SWITCH 3 VIDEO 返回： [SUCCESS]Set decoder 001 video from encoder 003. 输入命令： SET DEC 1 SWITCH 0 VIDEO 返回： [SUCCESS]Set decoder 001 video follow SET DEC SWITCH ALL command select. TELNET login CTL100D Enter the command: SET DEC 1 SWITCH 3 VIDEO Return: [SUCCESS]Set decoder 001 video from encoder 003. Enter the command: SET DEC 1 SWITCH 0 VIDEO Return: [SUCCESS]Set decoder 001 video follow SET DEC SWITCH ALL command select. |  |

锁定RX AUDIO路由Lock RX audio routing

| API接口 API interface |  |
| --- | --- |
| SET DEC [dec] SWITCH [enc] AUDIO |  |
| 描述 describe |  |
| 锁定RX AUDIO信号路由。 Lock the RX AUDIO signal routing. |  |
| 参数 parameter | 描述 describe |
| dec | [001…762]: RX ID号 0: 所有RX [001... 762]: RX ID number 0: All RXs |
| enc | [001...254]: TX ID号 0: 取消锁定 [001... 254]: TX ID number 0: Unlock |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Set decoder 001 audio from encoder 003. | 设置RX1锁定音频信号路由到TX3 Set RX1 to lock audio signal routing to TX3. |
| 样例 Example |  |
| TELNET登录CTL100D 输入命令： SET DEC 1 SWITCH 3 AUDIO 返回： [SUCCESS]Set decoder 001 audio from encoder 003. 输入命令： SET DEC 1 SWITCH 0 AUDIO 返回： [SUCCESS]Set decoder 001 audio follow SET DEC SWITCH ALL command select. TELNET login CTL100D Enter the command: SET DEC 1 SWITCH 3 AUDIO return: [SUCCESS]Set decoder 001 audio from encoder 003. Enter the command: SET DEC 1 SWITCH 0 AUDIO return: [SUCCESS]Set decoder 001 audio follow SET DEC SWITCH ALL command select. |  |

锁定RX IR路由Lock RX IR routing

| API接口 API interface |  |
| --- | --- |
| SET DEC [dec] SWITCH [enc] IR |  |
| 描述 describe |  |
| 锁定RX IR信号路由。 Lock the RX IR signal routing. |  |
| 参数 parameter | 描述 describe |
| dec | [001…762]: RX ID号 0: 所有RX [001... 762]: RX ID number 0: All RXs |
| enc | [001...254]: TX ID号 0: 取消锁定 [001... 254]: TX ID number 0: Unlock |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Set decoder 001 IR from encoder 003. | 设置RX1锁定IR信号路由到TX3 Set RX1 to lock IR signal routing to TX3. |
| 样例 Example |  |
| TELNET登录CTL100D 输入命令： SET DEC 1 SWITCH 3 IR 返回： [SUCCESS]Set decoder 001 IR from encoder 003. 输入命令： SET DEC 1 SWITCH 0 IR 返回： [SUCCESS]Clear decoder 001 IR routing. TELNET login CTL100D Enter the command: SET DEC 1 SWITCH 3 IR return: [SUCCESS]Set decoder 001 IR from encoder 003. Enter the command: SET DEC 1 SWITCH 0 IR return: [SUCCESS]Clear decoder 001 IR routing. |  |

锁定RX RS-232路由Lock RX RS-232 routing

| API接口 API interface |  |
| --- | --- |
| SET DEC [dec] SWITCH [enc] RS232 |  |
| 描述 describe |  |
| 锁定RX RS-232信号路由。 Lock the RX RS-232 signal routing. |  |
| 参数 parameter | 描述 describe |
| dec | [001…762]: RX ID号 0: 所有RX [001... 762]: RX ID number 0: All RXs |
| enc | [001...254]: TX ID号 0: 取消锁定 [001... 254]: TX ID number 0: Unlock |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Set decoder 001 RS232 from encoder 003. | 设置RX1锁定RS-232信号路由到TX3 Set RX1 to lock RS-232 signal routing to TX3. |
| 样例 Example |  |
| TELNET登录CTL100D 输入命令： SET DEC 1 SWITCH 3 RS232 返回： [SUCCESS]Set decoder 001 RS232 from encoder 003. 输入命令： SET DEC 1 SWITCH 0 RS232 返回： [SUCCESS]Clear decoder 001 serial routing. TELNET login CTL100D Enter the command: SET DEC 1 SWITCH 3 RS232 return: [SUCCESS]Set decoder 001 RS232 from encoder 003. Enter the command: SET DEC 1 SWITCH 0 RS232 return: [SUCCESS]Clear decoder 001 serial routing. |  |

锁定RX USB路由Lock RX USB routing

| API接口 API interface |  |
| --- | --- |
| SET DEC [dec] SWITCH [enc] USB |  |
| 描述 describe |  |
| 锁定RX USB信号路由。 Lock the RX USB signal routing. |  |
| 参数 parameter | 描述 describe |
| dec | [001…762]: RX ID号 0: 所有RX [001... 762]: RX ID number 0: All RXs |
| enc | [001...254]: TX ID号 0: 取消锁定 [001... 254]: TX ID number 0: Unlock |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Set decoder 001 USB from encoder 003. | 设置RX1锁定USB信号路由到TX3 Set RX1 to lock USB signal routing to TX3. |
| 样例 Example |  |
| TELNET登录CTL100D 输入命令： SET DEC 1 SWITCH 3 USB 返回： [SUCCESS]Set decoder 001 USB from encoder 003. 输入命令： SET DEC 1 SWITCH 0 USB 返回： [SUCCESS]Clear decoder 001 USB routing. TELNET login CTL100D Enter the command: SET DEC 1 SWITCH 3 USB return: [SUCCESS]Set decoder 001 USB from encoder 003. Enter the command: SET DEC 1 SWITCH 0 USB return: [SUCCESS]Clear decoder 001 USB routing. |  |

控制RX DEC灯闪烁Control RX DEC light flashing

| API接口 API interface |  |
| --- | --- |
| SET DEC [dec] LED ON/OFF SET DEC [dec] LED ON 90 |  |
| 描述 describe |  |
| 控制RX DEC灯闪烁。 Control the RX DEC light to flash. |  |
| 参数 parameter | 描述 describe |
| dec | [001…762]: RX ID号 0: 所有RX [001... 762]: RX ID number 0: All RXs |
| ON | DEC灯闪烁 DEC LED flashing |
| OFF | DEC灯常亮 DEC LED is steady on |
| ON 90 | DEC灯闪烁90秒后常亮 The DEC LED flashes for 90 seconds and then remains on |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Flash DEC LED on decoder 001. | 闪烁RX1 DEC灯 Set RX1 DEC light to flash. |
| 样例 Example |  |
| TELNET登录CTL100D 闪烁DEC灯，输入命令： SET DEC 1 LED ON 返回： [SUCCESS]Flash DEC LED on decoder 001. DEC灯常亮，输入命令： SET DEC 1 LED OFF 返回： [SUCCESS]Disable flash DEC LED on decoder 001. 闪烁DEC灯90秒，输入命令： SET DEC 1 LED ON 90 返回： [SUCCESS]Flash DEC LED on decoder 001 and keep 90 seconds. TELNET login CTL100D Set DEC LED to flash, enter the command: SET DEC 1 LED ON return: [SUCCESS]Flash DEC LED on decoder 001. DEC LED stays on, enter the command: SET DEC 1 LED OFF return: [SUCCESS]Disable flash DEC LED on decoder 001. Flash DEC LED for 90 seconds, enter command: SET DEC 1 LED ON 90 return: [SUCCESS]Flash DEC LED on decoder 001 and keep 90 seconds. |  |

设置RX OSD开关 Set RX OSD switch

| API接口 API interface |  |
| --- | --- |
| SET DEC [dec] OUTPUT OSD ON/OFF |  |
| 描述 describe |  |
| 开关RX OSD Turn on/off RX OSD |  |
| 参数 parameter | 描述 describe |
| dec | [001…762]: RX ID号 0: 所有RX [001... 762]: RX ID number 0: All RXs |
| ON | 打开OSD Turn on OSD |
| OFF OFF | 关闭OSD Turn off OSD |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Show OSD on decoder 001. | 打开OSD Turn on OSD |
| 样例 Example |  |
| TELNET登录CTL100D 打开OSD，输入命令： SET DEC 1 OUTPUT OSD ON 返回： [SUCCESS]Show OSD on decoder 001. 关闭OSD，输入命令： SET DEC 1 OUTPUT OSD OFF 返回： [SUCCESS]Hide OSD on decoder 001. TELNET login CTL100D Turn on OSD and enter the command: SET DEC 1 OUTPUT OSD ON return: [SUCCESS]Show OSD on decoder 001. Turn off OSD, enter the command: SET DEC 1 OUTPUT OSD OFF return: [SUCCESS]Hide OSD on decoder 001. |  |

设置RX HDMI OUTPUT开关 Set RX HDMI OUTPUT switch

| API接口 API interface |  |
| --- | --- |
| SET DEC [dec] OUTPUT ON/OFF |  |
| 描述 describe |  |
| 开关RX HDMI OUTPUT。 Turn on/off RX HDMI OUTPUT. |  |
| 参数 parameter | 描述 describe |
| dec | [001…762]: RX ID号 0: 所有RX [001... 762]: RX ID number 0: All RXs |
| ON | 打开HDMI OUTPUT Turn on HDMI OUTPUT |
| OFF | 关闭HDMI OUTPUT Turn off HDMI OUTPUT |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Set decoder 001 output on. | 打开HDMI OUTPUT Turn on HDMI OUTPUT |
| 样例 Example |  |
| TELNET登录CTL100D 打开HDMI OUTPUT，输入命令： SET DEC 1 OUTPUT ON 返回： [SUCCESS]Set decoder 001 on. 关闭HDMI OUTPUT，输入命令： SET DEC 1 OUTPUT OFF 返回： [SUCCESS]Set decoder 001 off. TELNET login CTL100D Turn on HDMI OUTPUT and enter the command: SET DEC 1 OUTPUT ON return: [SUCCESS]Set decoder 001 on. Turn off HDMI OUTPUT, enter the command: SET DEC 1 OUTPUT OFF return: [SUCCESS]Set decoder 001 off. |  |

设置RX HDMI OUTPUT MUTE
Set RX HDMI OUTPUT MUTE

| API接口 API interface |  |
| --- | --- |
| SET DEC [dec] OUTPUT MUTE ON/OFF |  |
| 描述 describe |  |
| 设置RX HDMI OUTPUT MUTE（黑屏）。 Set RX HDMI OUTPUT MUTE (black screen). |  |
| 参数 parameter | 描述 describe |
| dec | [001…762]: RX ID号 0: 所有RX [001... 762]: RX ID number 0: All RXs |
| ON | 打开HDMI OUTPUT MUTE Set HDMI OUTPUT MUTE on |
| OFF | 关闭HDMI OUTPUT MUTE Set HDMI OUTPUT MUTE off |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Set decoder 001 mute on. | 打开HDMI OUTPUT MUTE Set HDMI OUTPUT MUTE on |
| 样例 Example |  |
| TELNET登录CTL100D 打开HDMI OUTPUT MUTE，输入命令： SET DEC 1 OUTPUT MUTE ON 返回： [SUCCESS]Set decoder 001 mute on. 关闭HDMI OUTPUT MUTE，输入命令： SET DEC 1 OUTPUT MUTE OFF 返回： [SUCCESS]Set decoder 001 mute off. TELNET login CTL100D Set HDMI OUTPUT MUTE on, and enter the command: SET DEC 1 OUTPUT MUTE ON return: [SUCCESS]Set decoder 001 mute on. Set HDMI OUTPUT MUTE off, enter the command: SET DEC 1 OUTPUT MUTE OFF return: [SUCCESS]Set decoder 001 mute off. |  |

设置RX输出分辨率
Set RX output resolution

| API接口 API interface |  |
| --- | --- |
| SET DEC [dec] OUTPUT RESOLUTION [res] |  |
| 描述 describe |  |
| 设置RX输出分辨率为res Set RX output resolution to res |  |
| 参数 parameter | 描述 describe |
| dec dec | [001…762]: RX ID号 0: 所有RX [001... 762]: RX ID number 0: All RXs |
| res | 00: Bypass 01: 1280x720@50Hz 02: 1280x720@60Hz 03: 1920x1080@24Hz 04: 1920x1080@25Hz 05: 1920x1080@30Hz 06: 1920x1080@50Hz 07: 1920x1080@60Hz 08: 3840x2160@24Hz 09: 3840x2160@25Hz 10: 3840x2160@30Hz 11: 3840x2160@50Hz 12: 3840x2160@60Hz 13: 4096x2160@24Hz 14: 4096x2160@25Hz 15: 4096x2160@30Hz 16: 4096x2160@50Hz 17: 4096x2160@60Hz 18: 1280x768@60Hz 19: 1360x768@60Hz 20: 1680x1050@60Hz 21: 1920x1200@60Hz |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Set decoder 001 resolution to 1920x1080@60Hz. | 设置RX1输出分辨率为1080P60 Set RX1 output resolution to 1080P60 |
| 样例 Example |  |
| TELNET登录CTL100D 设置RX1输出分辨率为1080P60, 输入命令： SET DEC 1 OUTPUT RESOLUTION 7 返回： [SUCCESS]Set decoder 001 resolution to 1920x1080@60Hz. TELNET login CTL100D Set the output resolution of RX1 to 1080P60, input the command: SET DEC 1 OUTPUT RESOLUTION 7 return: [SUCCESS]Set decoder 001 resolution to 1920x1080@60Hz. |  |

设置RX IR电平
Set RX IR level

| API接口 API interface |  |
| --- | --- |
| SET DEC [dec] IR VOL [vol] |  |
| 描述 describe |  |
| 设置RX IR电平。 Set the RX IR level. |  |
| 参数 parameter | 描述 describe |
| dec | [001…762]: RX ID号 0: 所有RX [001... 762]: RX ID number 0: All RXs |
| vol | 0: 使用标准5V的IR线材 1: 使用12V的IR线材 2: 使用非标5V的IR线材 0: Use standard 5V IR cable 1: Use 12V IR cable 2: Use non-standard 5V IR cable |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Set decoder 001 ir voltage standard 5V. | 设置RX1使用5V的IR线材 Set RX1 to use 5V IR cable |
| 样例 Example |  |
| TELNET登录CTL100D 设置RX1使用5V的IR线材，输入命令： SET DEC 1 IR VOL 5V 返回： [SUCCESS]Set decoder 001 ir voltage standard 5V. TELNET login CTL100D Set RX1 to use 5V IR cable, enter the command: SET DEC 1 IR VOL 5V return: [SUCCESS]Set decoder 001 ir voltage standard 5V. |  |

设置RX图像输出模式
Set RX image output mode

| API接口 API interface |  |
| --- | --- |
| SET DEC [dec] MODE MX/VW |  |
| 描述 describe |  |
| 设置RX图像输出模式，只有Video Wall中的解码器才有效。 Set RX image output mode, only the decoder in Video Wall is valid. |  |
| 参数 parameter | 描述 describe |
| dec | [001…762]: RX ID号 0: 所有RX [001... 762]: RX ID number 0: All RXs |
| MX | MX模式输出（显示全部图像） Matrix mode output (displaying all images) |
| VW | VW模式输出（显示部分图像） Video wall mode output (displaying partial images) |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Set decoder 001 to matrix mode. | 设置RX1 MX模式输出 Set RX1 MX mode output |
| 样例 Example |  |
| TELNET登录CTL100D 设置RX1 MX模式输出，输入命令： SET DEC 1 MODE MX 返回： [SUCCESS]Set decoder 001 to matrix mode. TELNET login CTL100D Set RX1 MX mode output, input command: SET DEC 1 MODE MX return: [SUCCESS]Set decoder 001 to matrix mode. |  |

发送CEC指令给RX（Guest mode）
Send CEC command to RX (Guest mode)

| API接口 API interface |  |
| --- | --- |
| SET DEC [dec] CEC COMMAND [cmd] SET DEC [dec] CEC COMMAND [cmd] |  |
| 描述 describe |  |
| 发送CEC指令给RX（Guest mode） Send CEC command to RX (Guest mode) |  |
| 参数 parameter | 描述 describe |
| dec | [001…762]: RX ID号 0: 所有RX [001... 762]: RX ID number 0: All RXs |
| cmd | POWERON/POWEROFF/VIDEOLEFT/VIDEORIGHT/ VIDEOUP/VIDEODOWN/VIDEOENTER/VIDEOMENU/ VIDEOBACK/BACKWARD/FORWARD/PLAY/ VIDEOREW/VIDEOFF/PAUSE/VIDEOSTOP/ VOLUMEDOWN/VOLUMEUP/MUTE |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Decoder 1 CEC command power on. | 发送CEC指令成功 Send CEC command successfully |
| 样例 Example |  |
| TELNET登录CTL100D 输入命令： SET DEC 1 CEC COMMAND POWERON 返回： [SUCCESS]Decoder 1 CEC command power on. TELNET login CTL100D Input command: SET DEC 1 CEC COMMAND POWERON return: [SUCCESS]Decoder 1 CEC command power on. |  |

设置RX串口参数
Set RX serial port parameters

| API接口 API interface |  |
| --- | --- |
| SET DEC [dec] GUEST ON/OFF BR [br] BIT [bit] |  |
| 描述 describe |  |
| 设置RX串口参数 Set RX serial port parameters |  |
| 参数 parameter | 描述 describe |
| dec | [001…762]: RX ID号 0: 所有RX [001... 762]: RX ID number 0: All RXs |
| ON | 开启串口Guest mode Set serial port Guest mode on |
| OFF | 关闭串口Guest mode Set serial port Guest mode off |
| br | [0:2400 1:4800 2:9600] [3:19200 4:38400 5:57600 6:115200] |
| bit | Data Bits + Parity + Stop Bits example: 8n1 Data Bits=[5...8], Parity=[n o e], Stop Bits=[1..2] |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Set serial guest mode config done. | 设置RX串口参数成功 Successfully set RX serial port parameters |
| 样例 Example |  |
| TELNET登录CTL100D 设置RX1开启串口Guest mode，波特率115200，8位数据位，无校验位，1位停止位，输入命令： SET DEC 1 GUEST ON BR 9 BIT 8N1 返回： [SUCCESS]Set serial guest mode config done. TELNET login CTL100D Set RX1 to enable serial port Guest mode, baud rate 115200, 8-bit data bits, no checksum, 1-bit stop bit, input command: SET DEC 1 GUEST ON BR 9 BIT 8N1 return: [SUCCESS]Set serial guest mode config done. |  |

启动RX串口Guest mode
Activate RX serial port Guest mode

| API接口 API interface |  |
| --- | --- |
| SET DEC [dec] GUEST |  |
| 描述 describe |  |
| 启动RX串口Guest mode，只有当串口参数设置为GUEST ON时才有效 Activate RX serial port Guest mode, only valid when the serial port parameter is set to GUEST ON |  |
| 参数 parameter | 描述 describe |
| dec | [001…762]: RX ID号 [001... 762]: RX ID number |
| 返回值 Return value | 描述 describe |
| 无 none |  |
| 样例 Example |  |
| TELNET登录CTL100D 启动RX1串口Guest mode，输入命令： SET DEC 1 GUEST TELNET login CTL100D Activate RX1 serial port Guest mode, enter the command: SET DEC 1 GUEST |  |

退出RX串口Guest mode
Exit RX serial port Guest mode

| API接口 API interface |  |
| --- | --- |
| EXITGUEST |  |
| 描述 describe |  |
| 启动RX串口Guest mode后，发送EXITGUEST退出Guest mode After starting RX serial port Guest mode, send EXITGUEST to exit Guest mode |  |
| 参数 parameter | 描述 describe |
| 无 none |  |
| 返回值 Return value | 描述 describe |
| 无 nothing |  |
| 样例 Example |  |
| TELNET登录CTL100D 退出RX1串口Guest mode，输入命令： EXITGUEST TELNET login CTL100D Exit RX1 serial port Guest mode, enter the command: EXITGUEST |  |

从系统中删除RX
Remove RX from the system

| API接口 API interface |  |
| --- | --- |
| SET DEC [dec] DELETE |  |
| 描述 describe |  |
| 删除RX Delete RX |  |
| 参数 parameter | 描述 describe |
| dec | [001…762]: RX ID号 0: 所有RX [001... 762]: RX ID number 0: All RXs |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Delete decoder 001 done. | 删除RX1成功 Delete RX1 successfully |
| 样例 Example |  |
| TELNET登录CTL100D 删除RX1, 输入命令： SET DEC 1 DELETE 返回： [SUCCESS]Delete decoder 001 done. TELNET login CTL100D Delete RX1, enter the command: SET DEC 1 DELETE return: [SUCCESS]Delete decoder 001 done. |  |

重启RX
Reboot RX

| API接口 API interface |  |
| --- | --- |
| SET DEC [dec] REBOOT |  |
| 描述 describe |  |
| 重启RX Reboot RX |  |
| 参数 parameter | 描述 describe |
| dec | [001…762]: RX ID号 0: 所有RX [001... 762]: RX ID number 0: All RXs |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Set decoder 001 reboot and apply all the new config. | 重启成功 Reboot successful |
| 样例 Example |  |
| TELNET登录CTL100D 重启RX1，输入命令： SET DEC 1 REBOOT 返回： [SUCCESS]Set decoder 001 reboot and apply all the new config. TELNET login CTL100D Reboot RX1 and enter the command: SET DEC 1 REBOOT return: [SUCCESS]Set decoder 001 reboot and apply all the new config. |  |

复位RX
Reset RX

| API接口 API interface |  |
| --- | --- |
| SET DEC [dec] RESET |  |
| 描述 describe |  |
| 复位RX Reset RX |  |
| 参数 parameter | 描述 describe |
| dec | [001…762]: RX ID号 0: 所有RX [001... 762]: RX ID number 0: All RXs |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Set decoder 001 reset to default setting. | 复位成功 Reset successful |
| 样例 Example |  |
| TELNET登录CTL100D 复位RX1，输入命令： SET DEC 1 RESET 返回： [SUCCESS]Set decoder 001 reset to default setting. TELNET login CTL100D Reset RX1, enter the command: SET DEC 1 RESET return: [SUCCESS]Set decoder 001 reset to default setting. |  |

获取RX状态信息
Obtain RX status information

| API接口 API interface |  |
| --- | --- |
| GET DEC [dec] STATUS |  |
| 描述 describe |  |
| 获取RX的状态信息，不带参数dec时为获取所有RX的状态信息，即GET DEC STATUS同GET DEC 0 STATUS。 Obtain the status information of RX, without the parameter dec, to obtain the status information of all RX, i.e. GET DEC STATION and GET DEC 0 STATUS. |  |
| 参数 parameter | 描述 describe |
| dec | [001…762]: RX ID号 0: 所有RX [001... 762]: RX ID number 0: All RXs |
| 返回值 Return value | 描述 describe |
| 返回RX状态信息 Return RX status information | 包含版本号，网络信息等状态 Include version number, network information, and other status information |
| 样例 Example |  |
| TELNET登录CTL100D 获取RX1状态信息，输入命令： GET DEC 1 STATUS 返回： TELNET login CTL100D Obtain the status information of RX1, enter the command: GET DEC 1 STATUS return: ================================================================ ================================================================ CTL100D Controller Decoder Info FW Version: 3.00.12 Out Net HPD Ver Mode Res Name 001 On On 1.5.0.12 MX 07 Decoder 001 >>Fast Fr Vid/Aud/IR_/Ser/USB/CEC On 000 001/000/000/000/000/000 >>CEC Stretch IR BTN LED SGEn/Baudrate/Bit Off Off On On 9 On /115200 /8n1 >>Video Mute Auto IM MAC On On On Static 6c:df:fb:00:36:0d >>IP GW SM 169.254.020.001 000.000.000.000 255.255.000.000 ================================================================ |  |

设置RX预设IP模式
Set RX preset IP mode

| API接口 API interface |  |
| --- | --- |
| SET DEC PRESET IPMODE [mode] |  |
| 描述 describe |  |
| 设置RX预设的IP模式，在加入RX到系统时会根据此预设模式来分配RX的IP Set the preset IP mode for RX, and when adding RX to the system, assign the RX's IP based on this preset mode |  |
| 参数 parameter | 描述 describe |
| mode | 0:AUTOIP 1:DHCP 2:STATIC |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Set dec preset ip mode to static mode. Use "SET DEC PRESET APPLY" command to apply new config!!!. | 设置成功，RX预设IP模式为static RX preset IP mode is set to static successfully |
| 样例 Example |  |
| TELNET登录CTL100D 设置RX预设IP模式为static，输入命令： SET DEC PRESET IPMODE 2 返回： [SUCCESS]Set dec preset ip mode to static mode. Use "SET DEC PRESET APPLY" command to apply new config!!!. TELNET login CTL100D Set the RX preset IP mode to static, enter the command: SET DEC PRESET IPMODE 2 return: [SUCCESS]Set dec preset ip mode to static mode. Use "SET DEC PRESET APPLY" command to apply new config!!!. |  |

设置RX预设IP起始地址
Set RX preset start IP

| API接口 API interface |  |
| --- | --- |
| SET DEC PRESET START IP [ip] |  |
| 描述 describe |  |
| 设置RX预设的IP起始地址 Set the preset start IP for RX |  |
| 参数 parameter | 描述 describe |
| ip | IP地址，如169.254.10.10 IP address, such as 169.254.10.10 |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Set decoder preset IP min 172.016.010.001. | 设置RX预设的IP起始地址为172.16.10.1 Set the preset start IP for RX to 172.16.10.1 |
| 样例 Example |  |
| TELNET登录CTL100D 设置RX预设的IP起始地址为172.16.10.1，输入命令： SET DEC PRESET START IP 172.16.10.1 返回： [SUCCESS]Set decoder preset IP min 172.016.010.001. Use "SET DEC PRESET APPLY" command to apply new config!!!. TELNET login CTL100D Set the RX preset start IP to 172.16.10.1, enter the command: SET DEC PRESET START IP 172.16.10.1 return: [SUCCESS]Set decoder preset IP min 172.016.010.001. Use "SET DEC PRESET APPLY" command to apply new config!!!. |  |

设置RX预设IP结束地址
Set RX preset end IP

| API接口 API interface |  |
| --- | --- |
| SET DEC PRESET END IP [ip] |  |
| 描述 describe |  |
| 设置RX预设的IP结束地址，结束地址应大于起始地址，且在同一个网段中。 Set the RX preset end IP, which should be greater than the start IP, and in the same network segment. |  |
| 参数 parameter | 描述 describe |
| ip | IP地址，如169.254.20.10 IP address, such as 169.254.20.10 |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Set decoder preset IP max 172.016.010.200. | 设置RX预设的IP结束地址为172.16.10.200 Set the preset end IP for RX to 172.16.10.200 |
| 样例 Example |  |
| TELNET登录CTL100D 设置RX预设的IP结束地址为172.16.10.200，输入命令： SET DEC PRESET END IP 172.16.10.200 返回： [SUCCESS]Set decoder preset IP max 172.016.010.200. Use "SET DEC PRESET APPLY" command to apply new config!!!. TELNET login CTL100D Set the preset end IP for RX to 172.16.10.200, enter the command: SET DEC PRESET END IP 172.16.10.200 return: [SUCCESS]Set decoder preset IP max 172.016.010.200. Use "SET DEC PRESET APPLY" command to apply new config!!!. |  |

设置RX预设子网掩码
Set RX preset subnet mask

| API接口 API interface |  |
| --- | --- |
| SET DEC PRESET SM [mask] |  |
| 描述 describe |  |
| 设置RX预设的子网掩码 Set RX preset subnet mask |  |
| 参数 parameter | 描述 describe |
| mask | 子网掩码，如255.255.0.0 Subnet mask, such as 255.255.0.0 |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Set decoder preset netmask 255.255.000.000. | 设置RX预设的子网掩码为255.255.0.0 Set the preset subnet mask for RX to 255.255.0.0 |
| 样例 Example |  |
| TELNET登录CTL100D 设置RX预设的子网掩码为255.255.0.0，输入命令： SET DEC PRESET SM 255.255.0.0 返回： [SUCCESS]Set decoder preset netmask 255.255.000.000. Use "SET DEC PRESET APPLY" command to apply new config!!!. TELNET login CTL100D Set the preset subnet mask for RX to 255.255.0.0, enter the command: SET DEC PRESET SM 255.255.0.0 return: [SUCCESS]Set decoder preset netmask 255.255.000.000. Use "SET DEC PRESET APPLY" command to apply new config!!!. |  |

设置RX预设网关地址
Set RX preset gateway address

| API接口 API interface |  |
| --- | --- |
| SET DEC PRESET GW [gw] |  |
| 描述 describe |  |
| 设置RX预设的网关地址 Set RX preset gateway address |  |
| 参数 parameter | 描述 describe |
| gw | 网关地址，如169.254.0.1 Gateway address, such as 169.254.0.1 |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Set decoder preset gateway 172.016.010.001. | 设置RX预设的网关地址为172.16.10.1 Set the RX preset gateway address to 172.16.10.1 |
| 样例 Example |  |
| TELNET登录CTL100D 设置RX预设的网关地址为172.16.10.1，输入命令： SET DEC PRESET GW 172.16.10.1 返回： [SUCCESS]Set decoder preset gateway 172.016.010.001. Use "SET DEC PRESET APPLY" command to apply new config!!!. TELNET login CTL100D Set the RX preset gateway address to 172.16.10.1, enter the command: SET DEC PRESET GW 172.16.10.1 return: [SUCCESS]Set decoder preset gateway 172.016.010.001. Use "SET DEC PRESET APPLY" command to apply new config!!!. |  |

保存RX预设配置
Save RX preset configuration

| API接口 API interface |  |
| --- | --- |
| SET DEC PRESET APPLY |  |
| 描述 describe |  |
| 保存RX的预设配置，上面的预设IP模式等设置完成后，需要调用APPLY来保存。 Save the preset configuration of RX. After setting the preset IP mode and other settings, you need to call APPLY to save it. |  |
| 参数 parameter | 描述 describe |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Set decoder preset IP done. | 保存RX预设配置完成 Save RX preset configuration successfully |
| 样例 Example |  |
| TELNET登录CTL100D 保存RX预设配置，输入命令： SET DEC PRESET APPLY 返回： [SUCCESS]Set decoder preset IP done. TELNET login CTL100D Save RX preset configuration, enter command: SET DEC PRESET APPLY return: [SUCCESS]Set decoder preset IP done. |  |

设置RX图像拉伸
Set RX image stretching

| API接口 API interface |  |
| --- | --- |
| SET DEC [dec] STRETCH ON/OFF |  |
| 描述 describe |  |
| 设置RX画面拉伸 Set RX screen stretching |  |
| 参数 parameter | 描述 describe |
| dec | [001…762]: RX ID号 0: 所有RX [001... 762]: RX ID number 0: All RXs |
| ON | 图像拉伸 Image stretching |
| OFF | 保持原比例 Maintain the original proportion |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Set decoder 001 stretch on. | 设置RX图像拉伸成功 Successfully set RX image stretching |
| 样例 Example |  |
| TELNET登录CTL100D 保存RX预设配置，输入命令： SET DEC 1 STRETCH ON 返回： [SUCCESS]Set decoder 001 stretch on. TELNET login CTL100D Save RX preset configuration, enter command: SET DEC 1 STRETCH ON return: [SUCCESS]Set decoder 001 stretch on. |  |

设置RX显示模式
Set RX display mode

| API接口 API interface |  |
| --- | --- |
| SET DEC [dec] DISPLAY MODE [playmode] |  |
| 描述 describe |  |
| 设置RX显示模式 Set RX display mode |  |
| 参数 parameter | 描述 describe |
| dec | [001…762]: RX ID号 0: 所有RX [001... 762]: RX ID number 0: All RXs |
| playmode | 0: Fast Switch 1: Genlock |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Set decoder 001 display mode FastSwitch. | 设置RX图像输出为无缝切换模式 Set RX image output to seamless switching mode |
| 样例 Example |  |
| TELNET登录CTL100D 保存RX预设配置，输入命令： SET DEC 1 DISPLAY MODE 0 返回： [SUCCESS]Set decoder 001 display mode FastSwitch. TELNET login CTL100D Save RX preset configuration, enter command: SET DEC 1 DISPLAY MODE 0 return: [SUCCESS]Set decoder 001 display mode FastSwitch. |  |

切换RX为编码器模式
Switch RX to encoder mode

| API接口 API interface |  |
| --- | --- |
| SET DEC [dec] TXMODE |  |
| 描述 describe |  |
| 设置RX切换到编码器模式 Set RX to switch to encoder mode |  |
| 参数 parameter | 描述 describe |
| dec | [001…762]: RX ID号 0: 所有RX [001... 762]: RX ID number 0: All RXs |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Set Decoder 1 to Encoder Mode. | 设置RX切换到编码器模式 Set RX to switch to encoder mode |
| 样例 Example |  |
| TELNET登录CTL100D 设置RX切换到编码器模式，输入命令： SET DEC 1 TXMODE 返回： [SUCCESS]Set Decoder 1 to Encoder Mode. TELNET login CTL100D Set RX to switch to encoder mode, enter the command: SET DEC 1 TXMODE return: [SUCCESS]Set Decoder 1 to Encoder Mode. |  |

TX控制模块API参考
TX Control Module API Reference

设置TX ID号
Set TX ID number

| API接口 API interface |  |
| --- | --- |
| SET ENC [enc] ID [id] |  |
| 描述 describe |  |
| 设置TX ID号。 Set TX ID number. |  |
| 参数 parameter | 描述 describe |
| enc | [001…254]: TX ID号 [001... 254]: TX ID number |
| id | [001…254]: 目标ID号 [001... 254]: Target ID number |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Set encoder 001 ID to 221. | 设置TX1 ID号为221 Set TX1 ID number to 221 |
| [ERROR]Encoder 100 does not exist. | TX100不存在 TX100 does not exist |
| 样例 Example |  |
| TELNET登录CTL100D TX1 ID设置为160，输入命令： SET ENC 1 ID 160 返回: [SUCCESS]Set encoder 001 ID to 160. TELNET login CTL100D Set TX1 ID to 160, enter the command: SET ENC 1 ID 160 return: [SUCCESS]Set encoder 001 ID to 160. |  |

设置TX名称
Set TX Name

| API接口 API interface |  |
| --- | --- |
| SET ENC [enc] NAME [name] |  |
| 描述 describe |  |
| 设置TX名称。 Set TX name. |  |
| 参数 parameter | 描述 describe |
| enc | [001…254]: TX ID号 [001... 254]: TX ID number |
| name | 名字，最大长度为16个字节 Name, with a maximum length of 16 bytes |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Set encoder 001 name:TEST1. | 设置TX1名字为TEST1 Set TX1 name as TEST 1 |
| 样例 Example |  |
| TELNET登录CTL100D 设置TX1别名为TEST1, 输入命令： SET ENC 1 NAME TEST1 返回： [SUCCESS]Set encoder 001 name:TEST1. TELNET login CTL100D Set TX1 name as TEST 1, enter the command: SET ENC 1 NAME TEST1 return: [SUCCESS]Set encoder 001 name:TEST1. |  |

控制TX ENC灯闪烁
Control the TX ENC LED to flash

| API接口 API interface |  |
| --- | --- |
| SET ENC [enc] LED ON/OFF SET ENC [enc] LED ON 90 |  |
| 描述 describe |  |
| 控制TX ENC灯闪烁。 Control the TX ENC LED to flash. |  |
| 参数 parameter | 描述 describe |
| enc | [001…254]: TX ID号 0: 所有TX [001... 254]: TX ID number 0: All TX |
| ON | ENC灯闪烁 ENC LED flashing |
| OFF | ENC灯常亮 ENC LED is steady on |
| ON 90 | ENC灯闪烁90秒后常亮 The ENC LED flashes for 90 seconds and then remains on |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Flash ENC LED on encoder 001. | 闪烁TX1ENC灯 Set TX1ENC LED flash |
| 样例 Example |  |
| TELNET登录CTL100D 闪烁电源灯，输入命令： SET ENC 1 LED ON 返回： [SUCCESS]Flash ENC LED on encoder 001. 电源灯常亮，输入命令： SET ENC 1 LED OFF 返回： [SUCCESS]Disable flash ENC LED on encoder 001. 闪烁电源灯90秒，输入命令： SET ENC 1 LED ON 90 返回： [SUCCESS]Flash ENC LED on encoder 001 and keep 90 seconds. TELNET login CTL100D Power LED flashing, enter the command: SET ENC 1 LED ON return: [SUCCESS]Flash ENC LED on encoder 001. Power LED is steady on, enter the command: SET ENC 1 LED OFF return: [SUCCESS]Disable flash ENC LED on encoder 001. Power LED flashing for 90s, enter the command: SET ENC 1 LED ON 90 return: [SUCCESS]Flash ENC LED on encoder 001 and keep 90 seconds. |  |

设置TX音频源
Set TX audio source

| API接口 API interface |  |
| --- | --- |
| SET ENC [enc] SWITCH [dev] AUDIO SELECT [achn] |  |
| 描述 describe |  |
| 设置TX模拟输出音频。 Set up TX analog output audio. |  |
| 参数 parameter | 描述 describe |
| enc | [001…254]: TX ID号 0: 所有TX [001... 254]: TX ID number 0: All TXs |
| dev | [001…254]: TX ID号 [001...762]: RX ID号 0: 输出TX自身HDMI音频源 [001... 254]: TX ID number [001... 762]: RX ID number 0: Output TX's own HDMI audio source |
| achn | DECODER: 输出RX L/R in音频 LR: 输出TX L/R in音频 DECODER: Output RX L/R in audio LR: Output TX L/R in audio |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Set encoder 001 L/R audio out from encoder 002. | 设置TX1模拟输出音频源为encoder 002模拟输入 Set TX1 analog output audio source to encoder 002 analog input |
| 样例 Example |  |
| TELNET登录CTL100D 设置TX1模拟输出音频源为encoder 001 L/R in，输入命令： SET ENC 1 SWITCH 2 AUDIO SELECT LR 返回： [SUCCESS]Set encoder 001 L/R audio out from encoder 002. 特别的，设置TX1输出自身HDMI音频源 SET ENC 1 SWITCH 0 AUDIO [SUCCESS]Set encoder 001 L/R audio out to hdmi mode. TELNET login CTL100D Set TX1 analog output audio source to encoder 001 L/R in, input command: SET ENC 1 SWITCH 2 AUDIO SELECT LR return: [SUCCESS]Set encoder 001 L/R audio out from encoder 002. In particular, set TX1 to output its own HDMI audio source SET ENC 1 SWITCH 0 AUDIO [SUCCESS]Set encoder 001 L/R audio out to hdmi mode. |  |

设置TX EDID
Set TX EDID

| API接口 API interface |  |
| --- | --- |
| SET ENC [enc] EDID DEFAULT [edid] |  |
| 描述 describe |  |
| 设置TX EDID Set TX EDID |  |
| 参数 parameter | 描述 describe |
| enc | [001…254]: TX ID号 0: 所有TX [001…254]: TX ID number 0: All TXs |
| edid | 00: 1080P 20CH(Default) 01: 1080P 51CH 02: 1080P 71CH 03: 1080I 20CH 04: 1080I 51CH 05: 1080I 71CH 06: 1080P 3D 20CH 07: 1080P 3D 51CH 08: 1080P 3D 71CH 09: 4K2K30_444 20CH 10: 4K2K30_444 51CH 11: 4K2K30_444 71CH 12: 4K2K60_420 20CH 13: 4K2K60_420 51CH 14: 44K2K60_420 71CH 15: 4K2K60_444 20CH 16: 4K2K60_444 51CH 17: 4K2K60_444 71CH 18: 4K2K60_444 20CH HDR 19: 4K2K60_444 51CH HDR 20: 4K2K60_444 71CH HDR 22: User EDID 1 23: User EDID 2 |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Set encoder 001 edid with default edid 1080P 20CH. | 设置TX1 EDID为1080P 20CH Set TX1 EDID to 1080P 20CH |
| 样例 Example |  |
| TELNET登录CTL100D 设置TX1 EDID为1080P 20CH，输入命令： SET ENC 1 EDID DEFAULT 0 返回： [SUCCESS]Set encoder 001 edid with default edid 1080P 20CH. TELNET login CTL100D Set TX1 EDID to 1080P 20CH, enter the command: SET ENC 1 EDID DEFAULT 0 return: [SUCCESS]Set encoder 001 edid with default edid 1080P 20CH. |  |

设置TX拷贝RX EDID
Set TX to copy RX EDID

| API接口 API interface |  |
| --- | --- |
| SET ENC [enc] EDID COPY [dec] |  |
| 描述 describe |  |
| 设置TX拷贝RX的EDID Set TX to copy RX's EDID |  |
| 参数 parameter | 描述 describe |
| enc | [001…254]: TX ID号 0: 所有TX [001... 254]: TX ID number 0: All TXs |
| dec | [001...762]: RX ID号 [001... 762]: RX ID number |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Copy decoder 002 edid to encoder 01. | 设置TX1拷贝RX2的EDID Set TX1 to copy RX2's EDID |
| 样例 Example |  |
| TELNET登录CTL100D 设置TX1拷贝RX2的EDID，输入命令： SET ENC 1 EDID COPY 2 返回： [SUCCESS]Copy decoder 002 edid to encoder 01. TELNET login CTL100D Set TX1 to copy the EDID of RX2, enter the command: SET ENC 1 EDID COPY 2 return: [SUCCESS]Copy decoder 002 edid to encoder 01. |  |

设置TX IR电平
Set TX IR level

| API接口 API interface |  |
| --- | --- |
| SET ENC [enc] IR VOL [vol] |  |
| 描述 describe |  |
| 设置TX IR电平。 Set TX IR level. |  |
| 参数 parameter | 描述 describe |
| enc | [001…254]: TX ID号 0: 所有TX [001... 254]: TX ID number 0: All TXs |
| vol | 0: 使用标准5V的IR线材 1: 使用12V的IR线材 2: 使用非标5V的IR线材 0: Use standard 5V IR cable 1: Use 12V IR cable 2: Use non-standard 5V IR wire |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Set encoder 001 ir voltage standard 5V. | 设置TX1使用5V的IR线材 Set TX1 to use 5V IR cable |
| 样例 Example |  |
| TELNET登录CTL100D 设置TX1使用5V的IR线材，输入命令： SET ENC 1 IR VOL 0 返回： [SUCCESS]Set encoder 001 ir voltage standard 5V. TELNET login CTL100D Set TX1 to use 5V IR cable, enter the command: SET ENC 1 IR VOL 0 return: [SUCCESS]Set encoder 001 ir voltage standard 5V. |  |

发送CEC指令给TX（Guest mode）
Send CEC command to TX (Guest mode)

| API接口 API interface |  |
| --- | --- |
| SET ENC [enc] CEC COMMAND [cmd] |  |
| 描述 describe |  |
| 发送CEC指令给TX（Guest mode） Send CEC command to TX（Guest mode） |  |
| 设置TX IR电平。 Set TX IR level. | 设置TX IR电平。 Set TX IR level. |
| dec | [001…254]: TX ID号 0: 所有TX [001…254]: TX ID number 0: All TXs |
| cmd | POWERON/POWEROFF/VIDEOLEFT/VIDEORIGHT/ VIDEOUP/VIDEODOWN/VIDEOENTER/VIDEOMENU/ VIDEOBACK/BACKWARD/FORWARD/PLAY/ VIDEOREW/VIDEOFF/PAUSE/VIDEOSTOP/ VOLUMEDOWN/VOLUMEUP/MUTE |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Encoder 1 cec command power on. | 发送CEC指令成功 Send CEC command successfully |
| 样例 Example |  |
| TELNET登录CTL100D 输入命令： SET ENC 1 CEC COMMAND POWERON 返回： [SUCCESS]Encoder 1 cec command power on. TELNET login CTL100D Enter the command: SET ENC 1 CEC COMMAND POWERON return: [SUCCESS]Encoder 1 cec command power on. |  |

设置TX串口参数
Set TX serial port parameters

| API接口 API interface |  |
| --- | --- |
| SET ENC [enc] GUEST ON/OFF BR [br] BIT [bit] |  |
| 描述 describe |  |
| 设置TX串口参数 Set TX serial port parameters |  |
| 参数 parameter | 描述 describe |
| enc | [001…254]: TX ID号 0: 所有TX [001... 254]: TX ID number 0: All TXs |
| ON | 开启串口Guest mode Enable serial port Guest mode |
| OFF | 关闭串口Guest mode Disable the serial port Guest mode |
| br | [0:2400 1:4800 2:9600] [3:19200 4:38400 5:57600 6:115200] |
| bit | Data Bits + Parity + Stop Bits example: 8n1 Data Bits=[5...8], Parity=[n o e], Stop Bits=[1..2] |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Set serial guest mode config done. | 设置TX串口参数成功 Successfully set TX serial port parameters |
| 样例 Example |  |
| TELNET登录CTL100D 设置TX1开启串口Guest mode，波特率115200，8位数据位，无校验位，1位停止位，输入命令： SET ENC 1 GUEST ON BR 9 BIT 8N1 返回： [SUCCESS]Set serial guest mode config done. TELNET login CTL100D Set TX1 to enable serial port Guest mode, baud rate 115200, 8-bit data bits, no checksum, 1-bit stop bit, enter command: SET ENC 1 GUEST ON BR 9 BIT 8N1 return: [SUCCESS]Set serial guest mode config done. |  |

启动TX串口Guest mode
Activate TX serial port Guest mode

| API接口 API interface |  |
| --- | --- |
| SET ENC [enc] GUEST |  |
| 描述 describe |  |
| 启动TX串口Guest mode，只有当串口参数设置为GUEST ON时才有效 Activate TX serial port Guest mode, only effective when the serial port parameter is set to GUEST ON |  |
| 参数 parameter | 描述 describe |
| enc | [001…254]: TX ID号 [001... 254]: TX ID number |
| 返回值 Return value | 描述 describe |
| 无 none |  |
| 样例 Example |  |
| TELNET登录CTL100D 启动TX1串口Guest mode，输入命令： SET ENC 1 GUEST TELNET login CTL100D Activate TX1 serial port Guest mode, enter the command: SET ENC 1 GUEST |  |

退出TX串口Guest mode
Exit TX serial port Guest mode

| API接口 API interface |  |
| --- | --- |
| EXITGUEST |  |
| 描述 describe |  |
| 启动TX串口Guest mode后，发送EXITGUEST退出Guest mode After starting the TX serial port Guest mode, send EXITGUEST to exit Guest mode |  |
| 参数 parameter | 描述 describe |
| 无 none |  |
| 返回值 Return value | 描述 describe |
| 无 none |  |
| 样例 Example |  |
| TELNET登录CTL100D 退出TX1串口Guest mode，输入命令： EXITGUEST TELNET login CTL100D Exit TX1 serial port Guest mode, enter the command: EXITGUEST |  |

从系统中删除TX
Remove TX from the system

| API接口 API interface |  |
| --- | --- |
| SET ENC [enc] DELETE |  |
| 描述 describe |  |
| 删除TX Delete TX |  |
| 参数 parameter | 描述 describe |
| enc | [001…254]: TX ID号 0: 所有TX [001... 254]: TX ID number 0: All TXs |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Delete encoder 001 done. | 删除TX1成功 Delete TX1 successfully |
| 样例 Example |  |
| TELNET登录CTL100D 删除TX1, 输入命令： SET ENC 1 DELETE 返回： [SUCCESS]Delete encoder 001 done. TELNET login CTL100D Delete TX1, enter the command: SET ENC 1 DELETE return: [SUCCESS]Delete encoder 001 done. |  |

重启TX
Reboot TX

| API接口 API interface |  |
| --- | --- |
| SET ENC [enc] REBOOT |  |
| 描述 describe |  |
| 重启TX Reboot TX |  |
| 参数 parameter | 描述 describe |
| enc | [001…254]: TX ID号 0: 所有TX [001... 254]: TX ID number 0: All TXs |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Set encoder 001 reboot and apply all the new config. | 重启成功 Reboot successful |
| 样例 Example |  |
| TELNET登录CTL100D 重启TX1，输入命令： SET ENC 1 REBOOT 返回： [SUCCESS]Set encoder 001 reboot and apply all the new config. TELNET login CTL100D Reboot TX1 and enter the command: SET ENC 1 REBOOT return: [SUCCESS]Set encoder 001 reboot and apply all the new config. |  |

复位TX
Reset TX

| API接口 API interface |  |
| --- | --- |
| SET ENC [enc] RESET |  |
| 描述 describe |  |
| 复位TX Reset TX |  |
| 参数 parameter | 描述 describe |
| enc | [001…254]: TX ID号 0: 所有TX [001... 254]: TX ID number 0: All TXs |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Set encoder 001 reset to default setting. | 复位成功 Reset successful |
| 样例 Example |  |
| TELNET登录CTL100D 复位TX1，输入命令： SET ENC 1 RESET 返回： [SUCCESS]Set encoder 001 reset to default setting. TELNET login CTL100D Reset TX1, enter the command: SET ENC 1 RESET return: [SUCCESS]Set encoder 001 reset to default setting. |  |

获取TX状态信息
Obtain TX status information

| API接口 API interface |  |
| --- | --- |
| GET ENC [enc] STATUS |  |
| 描述 describe |  |
| 获取TX的状态信息，不带参数enc时为获取所有TX的状态信息，即GET ENC STATUS同GET ENC 0 STATUS。 Obtain the status information of TX, without the parameter enc, to obtain the status information of all TXs, i.e. GET ENC STATUS and GET ENC 0 STATUS. |  |
| 参数 parameter | 描述 describe |
| enc | [001…254]: TX ID号 0: 所有TX [001... 254]: TX ID number 0: All TX |
| 返回值 Return value | 描述 describe |
| 返回TX状态信息 Return TX status information | 包含版本号，网络信息等状态 Include version number, network information, and other status information |
| 样例 Example |  |
| TELNET登录CTL100D 获取TX1状态信息，输入命令： GET ENC 1 STATUS 返回： TELNET login CTL100D Obtain the status information of TX1, enter the command: GET ENC 1 STATUS return: ================================================================ ================================================================ CTL100D Controller Encoder Info FW Version: 3.00.12 In Net Sig Ver EDID Aud Name 001 On On 1.5.0.12 CP002 Auto Encoder 001 >>CEC LED SGEn/Baudrate/Bit Off 9 Off /115200 /8n1 Off 9 Off /115200 /8n1 >>IM MAC Static 6c:df:fb:00:a9:5f >>IP GW SM 169.254.010.001 000.000.000.000 255.255.000.000 ================================================================ |  |

设置TX预设IP模式
Set TX preset IP mode

| API接口 API interface |  |
| --- | --- |
| SET ENC PRESET IPMODE [mode] |  |
| 描述 describe |  |
| 设置TX预设的IP模式，在加入TX到系统时会根据此预设模式来分配TX的IP Set the preset IP mode for TX, and assign TX's IP based on this preset mode when adding TX to the system |  |
| 参数 parameter | 描述 describe |
| mode | 0:AUTOIP 1:DHCP 2:STATIC |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Set enc preset ipmode to static mode. Use "SET ENC PRESET APPLY" command to apply new config!!!. | 设置成功，TX预设IP模式为static Set TX preset IP mode to static successfully |
| 样例 Example |  |
| TELNET登录CTL100D 设置TX预设IP模式为static，输入命令： SET ENC PRESET IPMODE 2 返回： [SUCCESS]Set enc preset ip mode to static mode. Use "SET ENC PRESET APPLY" command to apply new config!!!. TELNET login CTL100D Set the TX preset IP mode to static, enter the command: SET ENC PRESET IPMODE 2 return: [SUCCESS]Set enc preset ip mode to static mode. Use "SET ENC PRESET APPLY" command to apply new config!!!. |  |

设置TX预设IP起始地址
Set TX preset start IP

| API接口 API interface |  |
| --- | --- |
| SET ENC PRESET START IP [ip] |  |
| 描述 describe |  |
| 设置TX预设的IP起始地址 Set the preset start IP for TX |  |
| 参数 parameter | 描述 describe |
| ip | IP地址，如169.254.10.10 IP address, such as 169.254.10.10 |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Set encoder preset IP min 172.016.010.001. | 设置TX预设的IP起始地址为172.16.10.1 Set the preset start IP for TX to 172.16.10.1 |
| 样例 Example |  |
| TELNET登录CTL100D 设置TX预设的IP起始地址为172.16.10.1，输入命令： SET ENC PRESET START IP 172.16.10.1 返回： [SUCCESS]Set encoder preset IP min 172.016.010.001. Use "SET ENC PRESET APPLY" command to apply new config!!!. TELNET login CTL100D Set the preset start IP for TX to 172.16.10.1, enter the command: SET ENC PRESET START IP 172.16.10.1 return: [SUCCESS]Set encoder preset IP min 172.016.010.001. Use "SET ENC PRESET APPLY" command to apply new config!!!. |  |

设置TX预设IP结束地址
Set TX preset end IP

| API接口 API interface |  |
| --- | --- |
| SET ENC PRESET END IP [ip] |  |
| 描述 describe |  |
| 设置TX预设的IP结束地址，结束地址应大于起始地址，且在同一个网段中。 Set the preset end IP for TX, which should be greater than the start IP and in the same network segment. |  |
| 参数 parameter | 描述 describe |
| ip | IP地址，如169.254.20.10 IP address, such as 169.254.20.10 |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Set encoder preset IP max 172.016.010.200. | 设置TX预设的IP结束地址为172.16.10.200 Set the preset end IP for TX to 172.16.10.200 |
| 样例 Example |  |
| TELNET登录CTL100D 设置TX预设的IP结束地址为172.16.10.200，输入命令： SET ENC PRESET END IP 172.16.10.200 返回： [SUCCESS]Set encoder preset IP max 172.016.010.200. Use "SET ENC PRESET APPLY" command to apply new config!!!. TELNET login CTL100D Set the preset end IP for TX to 172.16.10.200, enter the command: SET ENC PRESET END IP 172.16.10.200 return: [SUCCESS]Set encoder preset IP max 172.016.010.200. Use "SET ENC PRESET APPLY" command to apply new config!!!. |  |

设置TX预设子网掩码
Set TX preset subnet mask

| API接口 API interface |  |
| --- | --- |
| SET ENC PRESET SM [mask] |  |
| 描述 describe |  |
| 设置TX预设的子网掩码 Set the preset subnet mask for TX |  |
| 参数 parameter | 描述 describe |
| mask | 子网掩码，如255.255.0.0 Subnet mask, such as 255.255.0.0 |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Set encoder preset netmask 255.255.000.000. | 设置TX预设的子网掩码为255.255.0.0 Set the preset subnet mask for TX to 255.255.0.0 |
| 样例 Example |  |
| TELNET登录CTL100D 设置TX预设的子网掩码为255.255.0.0，输入命令： SET ENC PRESET SM 255.255.0.0 返回： [SUCCESS]Set encoder preset netmask 255.255.000.000. Use "SET ENC PRESET APPLY" command to apply new config!!!. TELNET login CTL100D Set the preset subnet mask for TX to 255.255.0.0, enter the command: SET ENC PRESET SM 255.255.0.0 return: [SUCCESS]Set encoder preset netmask 255.255.000.000. Use "SET ENC PRESET APPLY" command to apply new config!!!. |  |

设置TX预设网关地址
Set TX preset gateway address

| API接口 API interface |  |
| --- | --- |
| SET ENC PRESET GW [gw] |  |
| 描述 describe |  |
| 设置TX预设的网关地址 Set the preset gateway address for TX |  |
| 参数 parameter | 描述 describe |
| gw | 网关地址，如169.254.0.1 Gateway address, such as 169.254.0.1 |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Set encoder preset gateway 172.016.010.001. | 设置TX预设的网关地址为172.16.10.1 Set the preset gateway address for TX to 172.16.10.1 |
| 样例 Example |  |
| TELNET登录CTL100D 设置TX预设的网关地址为172.16.10.1，输入命令： SET ENC PRESET GW 172.16.10.1 返回： [SUCCESS]Set encoder preset gateway 172.016.010.001. Use "SET ENC PRESET APPLY" command to apply new config!!!. TELNET login CTL100D Set the preset gateway address for TX to 172.16.10.1, enter the command: SET ENC PRESET GW 172.16.10.1 return: [SUCCESS]Set encoder preset gateway 172.016.010.001. Use "SET ENC PRESET APPLY" command to apply new config!!!. |  |

保存TX预设配置
Save TX preset configuration

| API接口 API interface |  |
| --- | --- |
| SET ENC PRESET APPLY |  |
| 描述 describe |  |
| 保存TX的预设配置，上面的预设IP模式等设置完成后，需要调用APPLY来保存。 Save the preset configuration of TX, after setting the preset IP mode and other settings, you need to call APPLY to save it. |  |
| 参数 parameter | 描述 describe |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Encoder IP mode set to static ip range: 169.254.10.1-169.254.12.255. | 保存TX预设配置完成 Save TX preset configuration successfully |
| 样例 Example |  |
| TELNET登录CTL100D 保存TX预设配置，输入命令： SET ENC PRESET APPLY 返回： [SUCCESS]Encoder IP mode set to static ip range: 169.254.10.1-169.254.12.255. TELNET login CTL100D Save TX preset configuration, enter command: SET ENC PRESET APPLY return: [SUCCESS]Encoder IP mode set to static ip range: 169.254.10.1-169.254.12.255. |  |

切换TX为解码器模式
Switch TX to decoder mode

| API接口 API interface |  |
| --- | --- |
| SET ENC [enc] RXMODE |  |
| 描述 describe |  |
| 设置TX切换到解码器模式 Set TX to switch to decoder mode |  |
| 参数 parameter | 描述 describe |
| enc | [001…254]: TX ID号 0: 所有TX [001... 254]: TX ID number 0: All TXs |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Set Encoder 1 to Decoder Mode. | 设置TX切换到解码器模式 Set TX to switch to decoder mode |
| 样例 Example |  |
| TELNET登录CTL100D 设置RX切换到编码器模式，输入命令： SET ENC 1 RXMODE 返回： [SUCCESS]Set Encoder 1 to Decoder Mode. TELNET login CTL100D Set RX to switch to encoder mode, enter the command: SET ENC 1 RXMODE return: [SUCCESS]Set Encoder 1 to Decoder Mode. |  |

TX SS模块API参考
TX SS module API reference

获取TX SS状态信息
Obtain TX SS status information

| API接口 API interface |  |
| --- | --- |
| GET ENC [enc] SS STATUS |  |
| 描述 describe |  |
| 获取TX SS的状态信息，不带参数enc时为获取所有TX SS的状态信息，即GET ENC SS STATUS同GET ENC 0 SS STATUS。 Obtain the status information of TX SS, without parameter enc, to obtain the status information of all TX SS, i.e. GET ENC SS STATUS and GET ENC 0 SS STATUS. |  |
| 参数 parameter | 描述 describe |
| enc | [001…254]: TX ID号 0: 所有TX [001... 254]: TX ID number 0: All TXs |
| 返回值 Return value | 描述 describe |
| 返回TX SS状态信息 Return TX SS status information | 包含版本号，网络信息等状态 Include version number, network information, and other status information |
| 样例 Example |  |
| TELNET登录CTL100D 获取TX1 SS状态信息，输入命令： GET ENC 1 SS STATUS 返回： TELNET login CTL100D Obtain the status information of TX1 SS, enter the command: GET ENC 1 SS STATUS return: ================================================================ CTL100D Controller Secondary Stream Info FW Version: 3.00.12 ID WorkMode Version 001 Manual 1.03.16 >>MainStreamURL rtsp://169.254.10.4/live/main/av_stream >>SubStreamURL rtsp://169.254.10.4/live/sub/av_stream >>SnapURL http://169.254.10.4/snap.jpg >>MainStream EncType Res BitRate On h265 1920_1080(auto) 4096 >>SubStream EncType Res BitRate On h264 480_270 1024 >>PicStream On >>IpMode IpAdress MAC Static 169.254.10.4 6C:DF:FB:00:A9:61 ================================================================ |  |

获取TX SS主流URL
Obtain mainstream URL for TX SS

| API接口 API interface |  |
| --- | --- |
| GET ENC [enc] SS MAINSTREAM URL |  |
| 描述 describe |  |
| 获取TX SS的主流URL，SS主流格式为RTSP。 Obtain the mainstream URL of TX SS, which is in RTSP format. |  |
| 参数 parameter | 描述 describe |
| enc | [001…254]: TX ID号 0: 所有TX [001... 254]: TX ID number 0: All TXs |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Encoder 1 mainStream url:rtsp://169.254.10.1/live/main/av_stream. | TX1 SS的主流URL为 rtsp://169.254.10.1/live/main/av_stream The mainstream URL for TX1 SS is rtsp://169.254.10.1/live/main/av_stream |
| 样例 Example |  |
| TELNET登录CTL100D 获取TX1 SS主流URL，输入命令： GET ENC 1 SS MAINSTREAM URL 返回： [SUCCESS]Encoder 1 mainStream url: rtsp://169.254.10.1/live/main/av_stream. TELNET login CTL100D Get the mainstream URL of TX1 SS, enter the command: GET ENC 1 SS MAINSTREAM URL return: [SUCCESS]Encoder 1 mainStream url:rtsp://169.254.10.1/live/main/av_stream. |  |

获取TX SS次流URL
Get secondary stream URL for TX SS

| API接口 API interface |  |
| --- | --- |
| GET ENC [enc] SS SSURL |  |
| 描述 describe |  |
| 获取TX SS的次流URL，SS次流格式为RTSP。 Obtain the secondary stream URL of TX SS, which is in RTSP format. |  |
| 参数 parameter | 描述 describe |
| enc | [001…254]: TX ID号 0: 所有TX [001... 254]: TX ID number 0: All TXs |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Encoder 1 substream url:rtsp://169.254.10.1/live/sub/av_stream. | TX1 SS的次流URL为 rtsp://169.254.10.1/live/sub/av_stream The secondary stream URL for TX1 SS is rtsp://169.254.10.1/live/sub/av_stream |
| 样例 Example |  |
| TELNET登录CTL100D 获取TX1 SS次流URL，输入命令： GET ENC 1 SS SUBSTREAM URL 返回： [SUCCESS]Encoder 1 substream url: rtsp://169.254.10.1/live/sub/av_stream. TELNET login CTL100D Get the TX1 SS secondary stream URL, enter the command: GET ENC 1 SS SUBSTREAM URL return: [SUCCESS]Encoder 1 substream url:rtsp://169.254.10.1/live/sub/av_stream. |  |

获取TX SS抓图URL
Get TX SS snapshot URL

| API接口 API interface |  |
| --- | --- |
| GET ENC [enc] SS SNAPSHOT URL |  |
| 描述 describe |  |
| 获取TX SS的抓图URL。 Obtain the snapshot URL of TX SS. |  |
| 参数 parameter | 描述 describe |
| enc | [001…254]: TX ID号 0: 所有TX [001... 254]: TX ID number 0: All TXs |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Encoder 1 snapshot url:http://169.254.10.1:80/snap.jpg. | TX1 SS的抓图URL为 http://169.254.10.1:80/snap.jpg The snapshot URL for TX1 SS is http://169.254.10.1:80/snap.jpg |
| 样例 Example |  |
| TELNET登录CTL100D 获取TX1 SS抓图URL，输入命令： GET ENC 1 SS SNAPSHOT URL 返回： [SUCCESS]Encoder 1 snapshot url: http://169.254.10.1:80/snap.jpg. TELNET login CTL100D Get the TX1 SS snapshot URL and enter the command: GET ENC 1 SS SNAPSHOT URL return: [SUCCESS]Encoder 1 snapshot url:http://169.254.10.1:80/snap.jpg. |  |

设置TX SS主流参数
Set mainstream parameters for TX SS

| API接口 API interface |  |
| --- | --- |
| SET ENC [enc] SS MAINSTREAMATTR E [type] R [reso] H [mh] V [mv] B [mbit] |  |
| 描述 describe |  |
| 设置TX SS的主流参数。 Set the mainstream parameters of TX SS. |  |
| 参数 parameter | 描述 describe |
| enc | [001…254]: TX ID号 0: 所有TX [001... 254]: TX ID number 0: All TXs |
| type | 0:h264 1:h265 |
| reso | auto: 编码宽高与信号源输入分辨率一致 manual: 自定义编码宽高 Auto: The encoding width and height are consistent with the input resolution of the signal source Manual: Custom encoding width and height |
| mh | 设置编码输出图像宽度，范围: 960~1920，值必须为偶数 Set the encoding output image width, range: 960~1920, the value must be even |
| mv | 设置编码输出图像高度，范围: 540~1080，值必须为偶数 Set the encoding output image height, range: 540~1080, the value must be even |
| mbit | 设置编码比特率（bps） Set encoding bit rate (bps) 0:1Mb 1:2Mb 2:4Mb 3:6Mb 4:8Mb 5:10Mb 6:12Mb 7:16Mb 8:20Mb |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Set encoder 1 secondary stream mainencattr enctype h265 res manual width 960 height 540 bitrate 2048kbps. | 设置成功 Set successfully |
| 样例 Example |  |
| TELNET登录CTL100D 设置TX1 SS的主流编码格式为h265，输出图像大小为960x540，编码比特率为2Mbps，输入命令： SET ENC 2 SS MAINSTREAM ATTR E 1 R MANUAL H 960 V 540 B 1 返回： [SUCCESS]Set encoder 1 secondary stream mainencattr enctype h265 res manual width 960 height 540 bitrate 2048kbps. TELNET login CTL100D Set the mainstream encoding format of TX1 SS to h265, output image size to 960x540, encoding bit rate to 2Mbps, enter the command: SET ENC 2 SS MAINSTREAM ATTR E 1 R MANUAL H 960 V 540 B 1 return: [SUCCESS]Set encoder 1 secondary stream mainencattr enctype h265 res manual width 960 height 540 bitrate 2048kbps. |  |

设置TX SS次流参数
Set TX SS secondary stream parameters

| API接口 API interface |  |
| --- | --- |
| SET ENC [enc] SS SUBSTREAM ATTR E [type] H [sh] V [sv] B [sbit] |  |
| 描述 describe |  |
| 设置TX SS的次流参数。 Set the secondary stream parameters for TX SS. |  |
| 参数 parameter | 描述 describe |
| enc | [001…254]: TX ID号 0: 所有TX [001... 254]: TX ID number 0: All TXs |
| type | 0:h264 1:h265 |
| sh | 设置编码输出图像宽度，范围: 320~960，值必须为偶数 Set the encoding output image width, range: 320~960, the value must be even |
| sv | 设置编码输出图像高度，范围: 180~540，值必须为偶数 Set the encoding output image height, range: 180~540, the value must be even |
| sbit | 设置编码比特率（bps） Set encoding bit rate (bps) 0:128kb 1:256kb 2:512kb 3:1Mb 4:2Mb |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Set encoder 1 secondary stream subencattr enctype h265 width 640 height 480 bitrate 512kbps. | 设置成功 Set successfully |
| 样例 Example |  |
| TELNET登录CTL100D 设置TX1 SS的次流编码格式为h265，输出图像大小为640x480，编码比特率为512kbps，输入命令： SET ENC 1 SS SUBSTREAM ATTR E 1 H 640 V 480 B 2 返回： [SUCCESS]Set encoder 1 secondary stream subencattr enctype h265 width 640 height 480 bitrate 512kbps. TELNET login CTL100D Set the secondary stream encoding format of TX1 SS to h265, output image size to 640x480, encoding bit rate to 512kbps, enter the command: SET ENC 1 SS SUBSTREAM ATTR E 1 H 640 V 480 B 2 return: [SUCCESS]Set encoder 1 secondary stream subencattr enctype h265 width 640 height 480 bitrate 512kbps. |  |

设置TX SS主流开关
Set TX SS mainstream switch on/off

| API接口 API interface |  |
| --- | --- |
| SET ENC [enc] SS MAINSTREAM SWITCH ON/OFF |  |
| 描述 describe |  |
| 设置TX SS的主流开关。 Set the mainstream switch on/off for TX SS. |  |
| 参数 parameter | 描述 describe |
| enc | [001…254]: TX ID号 0: 所有TX [001... 254]: TX ID number 0: All TXs |
| ON | 开启主流 Turn on the mainstream |
| OFF | 关闭主流 Turn off the mainstream |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Set encoder 001 SS main stream off. | 关闭TX1主流 Turn off TX1 mainstream |
| 样例 Example |  |
| TELNET登录CTL100D 关闭TX1主流，输入命令： SET ENC 1 SS MAINSTREAM SWITCH OFF 返回： [SUCCESS]Set encoder 001 SS main stream off. TELNET login CTL100D Turn off TX1 mainstream, enter the command: SET ENC 1 SS MAINSTREAM SWITCH OFF return: [SUCCESS]Set encoder 001 SS main stream off. |  |

设置TX SS次流开关
Set TX SS sub stream switch on/off

| API接口 API interface |  |
| --- | --- |
| SET ENC [enc] SS SUBSTREAM SWITCH ON/OFF |  |
| 描述 describe |  |
| 设置TX SS的次流开关。 Set the sub stream current switch on/off for TX SS. |  |
| 参数 parameter | 描述 describe |
| enc | [001…254]: TX ID号 0: 所有TX [001... 254]: TX ID number 0: All TXs |
| ON | 开启次流 Turn on sub stream |
| OFF | 关闭次流 Turn off sub stream |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Set encoder 001 SS sub stream off. | 关闭TX1次流 Turn off sub stream |
| 样例 Example |  |
| TELNET登录CTL100D 关闭TX1次流，输入命令： SET ENC 1 SS SUBSTREAM SWITCH OFF 返回： [SUCCESS]Set encoder 001 SS sub stream off. TELNET login CTL100D Turn off TX1 sub stream, enter the command: SET ENC 1 SS SUBSTREAM SWITCH OFF return: [SUCCESS]Set encoder 001 SS sub stream off. |  |

设置TX SS抓图开关
Set TX SS snapshot switch on/off

| API接口 API interface |  |
| --- | --- |
| SET ENC [enc] SS SNAPSHOT SWITCH ON/OFF |  |
| 描述 describe |  |
| 设置TX SS的抓图开关。 Set the snapshot switch on/off for TX SS. |  |
| 参数 parameter | 描述 describe |
| enc enc | [001…254]: TX ID号 0: 所有TX [001... 254]: TX ID number 0: All TXs |
| ON | 开启抓图 Turn on snapshot |
| OFF | 关闭抓图 Turn off snapshot |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Set encoder 001 SS picture stream off. | 关闭TX1抓图 Turn off TX1 snapshot |
| 样例 Example |  |
| TELNET登录CTL100D 关闭TX1抓图，输入命令： SET ENC 1 SS SNAPSHOT SWITCH OFF 返回： [SUCCESS]Set encoder 001 SS picture stream off. TELNET login CTL100D Turn off TX1 snapshot, enter the command: SET ENC 1 SS SNAPSHOT SWITCH OFF return: [SUCCESS]Set encoder 001 SS picture stream off. |  |

视频墙模块API参考
Video Wall Module API Reference

创建视频墙
Create a video wall

| API接口 API interface |  |
| --- | --- |
| CREATE WALL HANDLE [hdl] |  |
| 描述 describe |  |
| 创建视频墙 Create a video wall |  |
| 参数 parameter | 描述 describe |
| hdl | [01...09]: 视频墙ID号 [01... 09]: Video Wall ID Number |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Create video wall 1. | 创建视频墙，ID等于1 Create a video wall with ID 1 |
| 样例 Example |  |
| TELNET登录CTL100D 创建视频墙，ID等于1，输入命令： CREATE WALL HANDLE 1 返回： [SUCCESS]Create video wall 1. TELNET login CTL100D Create a video wall with ID 1, enter the command: CREATE WALL HANDLE 1 return: [SUCCESS]Create video wall 1. |  |

删除视频墙
Delete video wall

| API接口 API interface |  |
| --- | --- |
| DELETE WALL HANDLE [hdl] |  |
| 描述 describe |  |
| 删除视频墙 Delete video wall |  |
| 参数 parameter | 描述 describe |
| hdl | [01...09]: 视频墙ID号 [01... 09]: Video Wall ID Number |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Delete videowall 1. | 删除视频墙1 Delete Video Wall 1 |
| 样例 Example |  |
| TELNET登录CTL100D 删除视频墙1，输入命令： DELETE WALL HANDLE 1 返回： [SUCCESS]Delete videowall 1. TELNET login CTL100D Delete Video Wall 1, enter the command: DELETE WALL HANDLE 1 return: [SUCCESS]Delete videowall 1. |  |

修改视频墙名称
Change the name of the video wall

| API接口 API interface |  |
| --- | --- |
| SET WALL [hdl] NAME [name] |  |
| 描述 describe |  |
| 修改视频墙的名称 Change the name of the video wall |  |
| 参数 parameter | 描述 describe |
| hdl | [01...09]: 视频墙ID号 [01... 09]: Video Wall ID Number |
| name | 视频墙名称，最大16个字符 Video wall name, maximum 16 characters |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Rename video wall 1: VW1. | 修改视频墙1的名称为VW1 Change the name of Video Wall 1 to VW1 |
| 样例 Example |  |
| TELNET登录CTL100D 修改视频墙1的名称为VW1，输入命令： SET WALL 1 NAME VW1 返回： [SUCCESS]Rename video wall 1: VW1. TELNET login CTL100D Change the name of Video Wall 1 to VW1, enter the command: SET WALL 1 NAME VW1 return: [SUCCESS]Rename video wall 1: VW1. |  |

设置视频墙大小
Set the size of the video wall

| API接口 API interface |  |
| --- | --- |
| SET WALL [hdl] C [c] R [r] |  |
| 描述 describe |  |
| 设置视频墙的大小。 Set the size of the video wall. |  |
| 参数 parameter | 描述 describe |
| hdl | [01...09]: 视频墙ID号 [01... 09]: Video Wall ID Number |
| c | 视频墙列数 Number of video wall columns |
| r | 视频墙行数 Number of video wall rows |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Create video wall 1. | 设置视频墙1成功 Successfully set up video wall 1 |
| 样例 Example |  |
| TELNET登录CTL100D 设置2x2的视频墙，输入命令： SET WALL 1 C 2 R 2 返回： [SUCCESS]Create video wall 1. TELNET login CTL100D Set up a 2x2 video wall, enter the command: SET WALL 1 C 2 R 2 return: [SUCCESS]Create video wall 1. |  |

视频墙分配RX
Assign RX to Video Wall

| API接口 API interface |  |
| --- | --- |
| SET WALL [hdl] DEC [dec] H [h] V [v] |  |
| 描述 describe |  |
| 视频墙分配RX Assign RX to Video Wall |  |
| 参数 parameter | 描述 describe |
| hdl | [01...09]: 视频墙ID号 [01... 09]: Video Wall ID Number |
| dec | [001...762]: RX ID号 [001... 762]: RX ID number |
| h | 视频墙的列号 Column number of video wall |
| v | 视频墙的行号 Row number of the video wall |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Assign decoder 001 to video wall 1. | 分配RX1到视频墙1上 Assign RX1 to Video Wall 1 |
| 样例 Example |  |
| TELNET登录CTL100D 分配RX1到视频墙1的第1行第1列，输入命令： SET WALL 1 DEC 1 H 1 V 1 返回： [SUCCESS]Assign decoder 001 to video wall 1. TELNET login CTL100D Assign RX1 to the first row and column of Video Wall 1, enter the command: SET WALL 1 DEC 1 H 1 V 1 return: [SUCCESS]Assign decoder 001 to video wall 1. |  |

创建视频墙预设
Create a video wall preset

| API接口 API interface |  |
| --- | --- |
| CREATE WALL [hdl] PRESET [prs] |  |
| 描述 describe |  |
| 创建视频墙预设，新建视频墙时会自动创建预设1 Create a video wall preset, and preset 1 will be created automatically when creating a new video wall |  |
| 参数 parameter | 描述 describe |
| hdl | [01...09]: 视频墙ID号 [01... 09]: Video Wall ID Number |
| prs | [01...09]: 预设ID号 [01... 09]: Preset ID Number |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Create preset 2: NULL. | 创建预设成功 Create preset successfully |
| 样例 Example |  |
| TELNET登录CTL100D 视频墙1创建预设2，输入命令： CREATE WALL 1 PRESET 2 返回： [SUCCESS]Create preset 2: NULL. TELNET login CTL100D Create preset 2 for video wall 1, enter the command: CREATE WALL 1 PRESET 2 return: [SUCCESS]Create preset 2: NULL. |  |

删除视频墙预设
Delete video wall preset

| API接口 API interface |  |
| --- | --- |
| DELETE WALL [hdl] PRESET [prs] |  |
| 描述 describe |  |
| 删除视频墙预设。 Delete the video wall preset. |  |
| 参数 parameter | 描述 describe |
| hdl | [01...09]: 视频墙ID号 [01... 09]: Video Wall ID Number |
| prs | [01...09]: 预设ID号 [01... 09]: Preset ID Number |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Delete preset: Preset 2. | 删除预设成功 Delete preset successfully |
| 样例 Example |  |
| TELNET登录CTL100D 视频墙1删除预设2，输入命令： DELETE WALL 1 PRESET 2 返回： [SUCCESS]Delete preset: Preset 2. TELNET login CTL100D Delete Preset 2 for Video Wall 1, enter the command: DELETE WALL 1 PRESET 2 return: [SUCCESS]Delete preset: Preset 2. |  |

修改视频墙预设名称
Modify the preset name of the video wall

| API接口 API interface |  |
| --- | --- |
| SET WALL [hdl] PRESET [prs] NAME [name] |  |
| 描述 describe |  |
| 修改视频墙预设的名称 Modify the preset name of the video wall |  |
| 参数 parameter | 描述 describe |
| hdl | [01...09]: 视频墙ID号 [01... 09]: Video Wall ID Number |
| prs | [01...09]: 预设ID号 [01... 09]: Preset ID Number |
| name | 预设名称，最大支持16个字符 Preset name, supporting up to 16 characters |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Rename preset 1: TEST1. | 修改视频墙预设的名称为TEST1 Change the preset name of the video wall to TEST 1 |
| 样例 Example |  |
| TELNET登录CTL100D 修改视频墙1预设1的名称为TEST1，输入命令： SET WALL 1 PRESET 1 NAME TEST1 返回： [SUCCESS]Rename preset 1: TEST1. TELNET login CTL100D Change the name of video wall 1 preset 1 to TEST 1, enter the command: SET WALL 1 PRESET 1 NAME TEST1 return: [SUCCESS]Rename preset 1: TEST1. |  |

启动视频墙预设
Apply video wall preset

| API接口 API interface |  |
| --- | --- |
| APPLY WALL [hdl] PRESET [prs] |  |
| 描述 describe |  |
| 启动视频墙预设。 Apply the video wall preset. |  |
| 参数 parameter | 描述 describe |
| hdl | [01...09]: 视频墙ID号 [01... 09]: Video Wall ID Number |
| prs | [01...09]: 预设ID号 [01... 09]: Preset ID Number |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Apply preset: Preset 1. | 启动预设成功 Apply preset successful |
| 样例 Example |  |
| TELNET登录CTL100D 启动视频墙1预设1，输入命令： APPLY WALL 1 PRESET 1 返回： [SUCCESS]Apply preset: Preset 1. TELNET login CTL100D Apply Video Wall 1 Preset 1, enter the command: APPLY WALL 1 PRESET 1 return: [SUCCESS]Apply preset: Preset 1. |  |

设置视频墙预设分组
Set video wall preset class

| API接口 API interface |  |
| --- | --- |
| SET WALL [hdl] PRESET [prs] CLASS [cls] H [h] V [v] |  |
| 描述 describe |  |
| 设置视频墙预设分组，创建预设时默认所有RX都在分组A。 Set the video wall preset class, and when creating the preset, all RXs will be in Class A by default. |  |
| 参数 parameter | 描述 describe |
| hdl | [01...09]: 视频墙ID号 [01... 09]: Video Wall ID Number |
| prs | [01...09]: 预设ID号 [01... 09]: Preset ID Number |
| cls | [A...G]: 分组ID号 [A... G]: Group ID number |
| h | 视频墙的列号 Column number of video wall |
| v | 视频墙的行号 Row number of the video wall |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Done. | 创建分组成功 Class successfully |
| 样例 Example |  |
| TELNET登录CTL100D 设置视频墙1第1行第2列的屏幕为预设1分组B，输入命令： SET WALL 1 PRESET 1 CLASS B H 2 V 1 返回： [SUCCESS]Done. TELNET login CTL100D Set the screen of the first row and second column of Video Wall 1 to preset Class B, enter the command: SET WALL 1 PRESET 1 CLASS B H 2 V 1 return: [SUCCESS]Done. |  |

设置视频墙预设分组的信号源
Set the signal source for the preset class of the video wall

| API接口 API interface |  |
| --- | --- |
| SET WALL [hdl] PRESET [prs] CLASS [cls] SOURCE [enc] |  |
| 描述 describe |  |
| 设置视频墙预设分组的信号源。 Set the signal source for the preset class of the video wall. |  |
| 参数 parameter | 描述 describe |
| hdl | [01...09]: 视频墙ID号 [01... 09]: Video Wall ID Number |
| prs | [01...09]: 预设ID号 [01... 09]: Preset ID Number |
| cls | [A...G]: 分组ID号 [A... G]: Class ID number |
| enc | [001...254]: TX ID号 0: 取消路由 [001... 254]: TX ID number 0: Cancel routing |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Done. | 设置成功 Set successfully |
| 样例 Example |  |
| TELNET登录CTL100D 设置视频墙1预设1分组B的信号源为TX1，输入命令： SET WALL 1 PRESET 1 CLASS B SOURCE 1 返回： [SUCCESS]Done. TELNET login CTL100D Set the signal source for Class B of Video Wall 1 to TX1. Enter the command: SET WALL 1 PRESET 1 CLASS B SOURCE 1 return: [SUCCESS]Done. |  |

设置视频墙预设矩阵组
Set the preset matrix group for the video wall

| API接口 API interface |  |
| --- | --- |
| SET WALL [hdl] PRESET [prs] MATRIX H [h] V [v] |  |
| 描述 describe |  |
| 设置视频墙预设矩阵组，矩阵组中的RX处于矩阵模式。 Set the preset matrix group for the video wall, with the RX in matrix mode. |  |
| 参数 parameter | 描述 describe |
| hdl | [01...09]: 视频墙ID号 [01... 09]: Video Wall ID Number |
| prs | [01...09]: 预设ID号 [01... 09]: Preset ID Number |
| h | 视频墙的列号 Column number of video wall |
| v | 视频墙的行号 Row number of the video wall |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Done. [SUCCESS]Done. | 设置矩阵组成功 Set matrix group successfully |
| 样例 Example |  |
| TELNET登录CTL100D 设置视频墙1第1行第2列的屏幕为预设1矩阵组，输入命令： SET WALL 1 PRESET 1 MATRIX H 2 V 1 返回： [SUCCESS]Done. TELNET login CTL100D Set the screen of the first row and second column of Video Wall 1 as the preset 1 matrix group, enter the command: SET WALL 1 PRESET 1 MATRIX H 2 V 1 return: [SUCCESS]Done. |  |

设置视频墙预设矩阵组的信号源
Set the signal source for the preset matrix group of the video wall

| API接口 API interface |  |
| --- | --- |
| SET WALL [hdl] PRESET [prs] MATRIX H [h] V [v] SOURCE [enc] |  |
| 描述 describe |  |
| 设置视频墙预设矩阵组的信号源。 Set the signal source for the preset matrix group of the video wall. |  |
| 参数 parameter | 描述 describe |
| hdl | [01...09]: 视频墙ID号 [01... 09]: Video Wall ID Number |
| prs | [01...09]: 预设ID号 [01... 09]: Preset ID Number |
| h | 视频墙的列号 Column number of video wall |
| v | 视频墙的行号 Row number of the video wall |
| enc | [001...254]: TX ID号 0: 取消路由 [001... 254]: TX ID number 0: Cancel routing |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Done. | 设置成功 Set successfully |
| 样例 Example |  |
| TELNET登录CTL100D 设置视频墙1预设1矩阵组第1行第2列屏幕的信号源为TX1，输入命令： SET WALL 1 PRESET 1 MATRIX H 2 V 1 SOURCE 1 返回： [SUCCESS]Done. TELNET login CTL100D Set the signal source for the screen in the first row and second column of the preset 1 matrix group of Video Wall 1 to TX1. Enter the command: SET WALL 1 PRESET 1 MATRIX H 2 V 1 SOURCE 1 return: [SUCCESS]Done. |  |

设置视频墙屏幕宽度方向边框
Set the bezel width of the video wall screen

| API接口 API interface |  |
| --- | --- |
| SET WALL [hdl] H [h] V [v] WIDTH BEZEL BW [b] IW [i] |  |
| 描述 describe |  |
| 设置视频墙屏幕宽度方向的边框。 Set the bezel width of the video wall screen. |  |
| 参数 parameter | 描述 describe |
| hdl | [01...09]: 视频墙ID号 [01... 09]: Video Wall ID Number |
| h | 视频墙的列号 Column number of video wall |
| v | 视频墙的行号 Row number of the video wall |
| b b | [100...1000]: 原始图像宽度 [100... 1000]: Original image width |
| i i | [100...1000]: 可见图像宽度，i不能大于b [100... 1000]: Visible image width, i cannot be greater than b |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Done. | 设置成功 Set successfully |
| 样例 Example |  |
| TELNET登录CTL100D 设置视频墙1第1行第1列的屏幕宽度方向裁剪10%，(BW - IW) / BW = 10%，输入命令： SET WALL 1 H 1 V 1 WIDTH BEZEL BW 1000 IW 900 返回： [SUCCESS]Done. TELNET login CTL100D Set the bezel width of the first row and first column of Video Wall 1 to be cropped by 10%, (BW - IW)/BW=10%, enter the command: SET WALL 1 H 1 V 1 WIDTH BEZEL BW 1000 IW 900 return: [SUCCESS]Done. |  |

设置视频墙屏幕高度方向边框
Set the bezel height of the video wall screen

| API接口 API interface |  |
| --- | --- |
| SET WALL [hdl] H [h] V [v] HEIGHT BEZEL BH [b] IH [i] |  |
| 描述 describe |  |
| 设置视频墙屏幕高度方向的边框。 Set the bezel height of the video wall screen. |  |
| 参数 parameter | 描述 describe |
| hdl | [01...09]: 视频墙ID号 [01... 09]: Video Wall ID Number |
| h | 视频墙的列号 Column number of video wall |
| v | 视频墙的行号 Row number of the video wall |
| b | [100...1000]: 原始图像高度 [100... 1000]: Original image height |
| i | [100...1000]: 可见图像高度，i不能大于b [100... 1000]: Visible image height, i cannot be greater than b |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Done. | 设置成功 Set successfully |
| 样例 Example |  |
| TELNET登录CTL100D 设置视频墙1第1行第1列的屏幕高度方向裁剪10%，(BH - IH) / BH = 10%，输入命令： SET WALL 1 H 1 V 1 HEIGHT BEZEL BH 1000 IH 900 返回： [SUCCESS]Done. TELNET login CTL100D Set the bezel height of the first row and first column of Video Wall 1 to crop by 10%, (BH - IH)/BH=10%, enter the command: SET WALL 1 H 1 V 1 HEIGHT BEZEL BH 1000 IH 900 return: [SUCCESS]Done. |  |

获取视频墙状态
Get the status of the video wall

| API接口 API interface |  |
| --- | --- |
| GET WALL [hdl] STATUS |  |
| 描述 describe |  |
| 获取视频墙状态 Get the status of the video wall |  |
| 参数 parameter | 描述 describe |
| hdl | [01...09]: 视频墙ID号 [01... 09]: Video Wall ID Number |
| 返回值 Return value | 描述 describe |
| 打印视频墙状态信息 Print video wall status information |  |
| 样例 Example |  |
| TELNET登录CTL100D 获取视频墙1状态信息，输入命令： GET WALL 1 STATUS 返回： TELNET login CTL100D Get the status information of Video Wall 1, enter the command: GET WALL 1 STATUS return: ================================================================ CTL100D Controller Video Wall Info FW Version: 3.10.08 VW Col Row CfgSel Name 01 02 02 01 Video Wall 1 OutID 001 --- --- --- CFG Name 01 Preset 1 Group FromIn Screen A 001 H01V01 H02V01 H01V02 H02V02 ================================================================ |  |

多视窗模块API参考
Multiview module API reference

创建多视窗
Create multiview

| API接口 API interface |  |
| --- | --- |
| CREATE MULTIVIEW [mhdl] |  |
| 描述 describe |  |
| 创建多视窗 Create multiview |  |
| 参数 parameter | 描述 describe |
| mhdl | [01...100]: 多视窗ID号 [01... 100]: Multiview ID number |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Create multiview 1: NULL. | 创建多视窗，ID等于1 Create multiview with ID 1 |
| 样例 Example |  |
| TELNET登录CTL100D 创建多视窗，ID等于1，输入命令： CREATE MULTIVIEW 1 返回： [SUCCESS]Create multiview 1: NULL. TELNET login CTL100D Create multiview with ID 1, enter the command: CREATE MULTIVIEW 1 return: [SUCCESS]Create multiview 1: NULL. |  |

删除多视窗
Delete multiview

| API接口 API interface |  |
| --- | --- |
| DELETE MULTIVIEW [mhdl] |  |
| 描述 describe |  |
| 删除多视窗 Delete multiview |  |
| 参数 parameter | 描述 describe |
| mhdl | [01...100]: 多视窗ID号 [01... 100]: Multiview ID number |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Delete multiview 1. | 删除多视窗1 Delete multiview 1 |
| 样例 Example |  |
| TELNET登录CTL100D 删除多视窗1，输入命令： DELETE MULTIVIEW 1 返回： [SUCCESS]Delete multiview 1. TELNET login CTL100D Delete multiview 1, enter the command: DELETE MULTIVIEW 1 return: [SUCCESS]Delete multiview 1. |  |

修改多视窗名称
Change the name of multiview

| API接口 API interface |  |
| --- | --- |
| SET MULTIVIEW [mhdl] NAME [name] |  |
| 描述 describe |  |
| 修改多视窗的名称 Change the name of multiview |  |
| 参数 parameter | 描述 describe |
| mhdl | [01...100]: 多视窗ID号 [01... 100]: Multiview ID number |
| name | 多视窗名称，最大16个字符 Multiview name, with maximum 16 characters |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Rename multiview 1: MV1. | 修改多视窗1的名称为MV1 Change the name of multiview 1 to MV1 |
| 样例 Example |  |
| TELNET登录CTL100D 修改多视窗1的名称为MV1，输入命令： SET MULTIVIEW 1 NAME MV1 返回： [SUCCESS]Rename multiview 1: MV1. TELNET login CTL100D Change the name of multiview 1 to MV1, enter the command: SET MULTIVIEW 1 NAME MV1 return: [SUCCESS]Rename multiview 1: MV1. |  |

设置多视窗布局
Set up multiview layout

| API接口 API interface |  |
| --- | --- |
| SET MULTIVIEW [mhdl] LAYOUT [lay] |  |
| 描述 describe |  |
| 设置多视窗布局。 Set up a multiview layout. |  |
| 参数 parameter | 描述 describe |
| mhdl | [01...100]: 多视窗ID号 [01... 100]: Multiview ID number |
| lay | [01...100]: 多视窗布局 [01... 100]: Multiview layout |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Set multiview 1 layout 1. | 设置多视窗1使用布局1 Set up multiview 1 and use layout 1 |
| 样例 Example |  |
| TELNET登录CTL100D 设置多视窗1使用布局1，输入命令： SET MULTIVIEW 1 LAYOUT 1 返回： [SUCCESS]Set multiview 1 layout 1. TELNET login CTL100D Set up multiview 1 and use layout 1. Enter the command: SET MULTIVIEW 1 LAYOUT 1 return: [SUCCESS]Set multiview 1 layout 1. |  |

设置多视窗窗口信号源
Set the signal source for multiview

| API接口 API interface |  |
| --- | --- |
| SET MULTIVIEW [mhdl] WINDOW [w] SOURCE [enc] |  |
| 描述 describe |  |
| 设置多视窗窗口信号源。 Set the signal source for multiview. |  |
| 参数 parameter | 描述 describe |
| mhdl | [01...100]: 多视窗ID号 [01... 100]: Multiview ID number |
| w | [01...n]: 窗口号 [01... n]: Window number |
| enc | [001...254]: TX ID号 0: 取消路由 [001... 254]: TX ID number 0: Cancel routing |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Set multiview 1 window 1 from encoder 1. | 设置多视窗1窗口1信号源TX1 Set signal source TX1 for window 1 of multiview 1 |
| 样例 Example |  |
| TELNET登录CTL100D 设置多视窗1窗口1信号源TX1，输入命令： SET MULTIVIEW 1 LAYOUT 1 返回： [SUCCESS]Set multiview 1 window 1 from encoder 1. TELNET login CTL100D Set signal source TX1 for window 1 of multiview 1, enter the command: SET MULTIVIEW 1 LAYOUT 1 return: [SUCCESS]Set multiview 1 window 1 from encoder 1. |  |

创建画中画布局
Create a multiview PIP layout

| API接口 API interface |  |
| --- | --- |
| CREATE PIP [phdl] |  |
| 描述 describe |  |
| 创建画中画布局 Create a PIP layout |  |
| 参数 parameter | 描述 describe |
| phdl | [26...100]: 画中画布局ID号 [26... 100]: PIP Layout ID Number |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Create multiview PIP layout 27: (null). | 创建画中画布局，ID等于27 Create a PIP layout with ID 27 |
| 样例 Example |  |
| TELNET登录CTL100D 创建画中画布局，ID等于27，输入命令： CREATE PIP 27 返回： [SUCCESS]Create multiview PIP layout 27: (null). TELNET login CTL100D Create a PIP layout with ID 27, enter the command: CREATE PIP 27 return: [SUCCESS]Create multiview PIP layout 27: (null). |  |

删除画中画布局
Delete multiview PIP layout

| API接口 API interface |  |
| --- | --- |
| DELETE PIP [phdl] |  |
| 描述 describe |  |
| 删除画中画布局 Delete PIP layout |  |
| 参数 parameter | 描述 describe |
| phdl | [26...100]: 画中画布局ID号 [26... 100]: PIP Layout ID Number |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Delete multiview PIP layout 27. | 删除画中画布局27 Delete PIP Layout 27 |
| 样例 Example |  |
| TELNET登录CTL100D 删除画中画布局27，输入命令： DELETE PIP 27 返回： [SUCCESS]Delete multiview PIP layout 27. TELNET login CTL100D Delete PIP layout 27, enter the command: DELETE PIP 27 return: [SUCCESS]Delete multiview PIP layout 27. |  |

修改画中画布局名称
Modify the name of the PIP layout

| API接口 API interface |  |
| --- | --- |
| SET PIP [phdl] NAME [name] |  |
| 描述 describe |  |
| 修改画中画布局的名称 Change the name of the PIP layout |  |
| 参数 parameter | 描述 describe |
| phdl | [26...100]: 画中画布局ID号 [26... 100]: PIP Layout ID Number |
| name | 画中画布局名称，最大16个字符 PIP layout name, maximum 16 characters |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Rename multiview PIP layout 27: pip27. | 修改画中画布局27的名称为PIP27 Change the name of PIP Layout 27 to PIP27 |
| 样例 Example |  |
| TELNET登录CTL100D 修改画中画布局27的名称为PIP27，输入命令： SET PIP 27 NAME PIP27 返回： [SUCCESS]Rename multiview PIP layout 27: pip27. TELNET login CTL100D Change the name of PIP Layout 27 to PIP27, enter the command: SET PIP 27 NAME PIP27 return: [SUCCESS]Rename multiview PIP layout 27: pip27. |  |

修改画中画布局类型
Modify the layout type of PIP

| API接口 API interface |  |
| --- | --- |
| SET PIP [phdl] TYPE [tp] |  |
| 描述 describe |  |
| 修改画中画布局的类型 Change the layout type of PIP layout |  |
| 参数 parameter | 描述 describe |
| phdl | [26...100]: 画中画布局ID号 [26... 100]: PIP Layout ID Number |
| tp | 1: 2X2 3: 3X3 5: 4X4 |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Set multiview PIP layout 27 type 5. | 修改画中画布局27为4X4布局 Modify the PIP layout from 27 to 4X4 layout |
| 样例 Example |  |
| TELNET登录CTL100D 修改画中画布局27为4X4布局，输入命令： SET PIP 27 TYPE 5 返回： [SUCCESS]Set multiview PIP layout 27 type 5. TELNET login CTL100D Modify the PIP layout from 27 to 4X4 layout, enter the command: SET PIP 27 TYPE 5 return: [SUCCESS]Set multiview PIP layout 27 type 5. |  |

使能画中画窗口
Enable PIP window

| API接口 API interface |  |
| --- | --- |
| SET PIP [phdl] WINDOW [w] ENABLE/DISABLE |  |
| 描述 describe |  |
| 使能画中画窗口 Enable PIP window |  |
| 参数 parameter | 描述 describe |
| phdl | [26...100]: 画中画布局ID号 [26... 100]: PIP Layout ID Number |
| w | 窗口号ID Window ID |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Set multiview PIP layout 27 type 5 windows 2 ENABLE. | 使能画中画布局27窗口2 Enable PIP Layout 27 Window 2 |
| 样例 Example |  |
| TELNET登录CTL100D 使能画中画布局27窗口2，输入命令： SET PIP 27 WINDOW 2 EANBLE 返回： [SUCCESS]Set multiview PIP layout 27 type 5 windows 2 ENABLE. TELNET login CTL100D Enable PIP layout 27 window 2, enter the command: SET PIP 27 WINDOW 2 EANBLE return: [SUCCESS]Set multiview PIP layout 27 type 5 windows 2 ENABLE. |  |

矩阵预设模块API参考 Matrix Preset Module API Reference

创建矩阵预设 Create matrix presets

| API接口 API interface |  |
| --- | --- |
| CREATE MATRIX PRESET [prs] |  |
| 描述 describe |  |
| 创建矩阵预设 Create Matrix Presets |  |
| 参数 parameter | 描述 describe |
| prs | [01...30]: 预设ID号 [01... 30]: Preset ID number |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Create matrix preset 1 done. | 创建矩阵预设，ID等于1 Create a matrix preset with ID 1 |
| 样例 Example |  |
| TELNET登录CTL100D 创建矩阵预设，ID等于1，输入命令： CREATE MATRIX PRESET 1 返回： [SUCCESS]Create matrix preset 1 done. TELNET login CTL100D Create a matrix preset with ID 1, enter the command: CREATE MATRIX PRESET 1 return: [SUCCESS]Create matrix preset 1 done. |  |

删除矩阵预设 Delete matrix preset

| API接口 API interface |  |
| --- | --- |
| DELETE MATRIX PRESET [prs] |  |
| 描述 describe |  |
| 删除矩阵预设 Delete matrix preset |  |
| 参数 parameter | 描述 describe |
| prs | [01...30]: 预设ID号 [01... 30]: Preset ID number |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Delete matrix preset 1 done. | 删除矩阵预设1 Delete matrix preset 1 |
| 样例 Example |  |
| TELNET登录CTL100D 删除矩阵预设1，输入命令： DELETE MATRIX PRESET 1 返回： [SUCCESS]Delete matrix preset 1 done. TELNET login CTL100D Delete matrix preset 1, enter the command: DELETE MATRIX PRESET 1 return: [SUCCESS]Delete matrix preset 1 done. |  |

修改矩阵预设名称 Modify the preset name of the matrix

| API接口 API interface |  |
| --- | --- |
| SET MATRIX PRESET [prs] NAME [name] |  |
| 描述 describe |  |
| 修改矩阵预设的名称 Modify the preset name of the matrix |  |
| 参数 parameter | 描述 describe |
| prs | [01...30]: 预设ID号 [01... 30]: Preset ID number |
| name | 预设名称，最大16个字符 Preset name, maximum 16 characters |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Rename matrix preset 1: mp1. | 修改预设1的名称为mp1 Change the name of preset 1 to mp1 |
| 样例 Example |  |
| TELNET登录CTL100D 修改预设1的名称为mp1，输入命令： SET MATRIX PRESET 1 NAME mp1 返回： [SUCCESS]Rename matrix preset 1: mp1. TELNET login CTL100D Change the name of preset 1 to mp1, enter the command: SET MATRIX PRESET 1 NAME mp1 return: [SUCCESS]Rename matrix preset 1: mp1. |  |

新增矩阵预设条目 Add preset matrix entry

| API接口 API interface |  |
| --- | --- |
| ADD MATRIX PRESET [prs] DEC [dec] SOURCE [enc] |  |
| 描述 describe |  |
| 新增矩阵预设条目 Add preset matrix entry |  |
| 参数 parameter | 描述 describe |
| prs | [01...30]: 预设ID号 [01... 30]: Preset ID number |
| dec | [001…762]: RX ID号 0: 所有RX [001... 762]: RX ID number 0: All RXs |
| enc | [001...254]: TX ID号 0: 取消路由 [001... 254]: TX ID number 0: Cancel routing |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Add decoder 001 to matrix preset 1. | 新增矩阵预设1条目RX1信号源来自TX1 Add matrix preset 1 entry RX1 with signal source from TX1 |
| 样例 Example |  |
| TELNET登录CTL100D 新增矩阵预设1条目RX1信号源来自TX1，输入命令： ADD MATRIX PRESET 10 DEC 1 SOURCE 1 返回： [SUCCESS]Add decoder 001 to matrix preset 1. TELNET login CTL100D Add matrix preset 1 entry RX1 with signal source from TX1, enter the command: ADD MATRIX PRESET 10 DEC 1 SOURCE 1 return: [SUCCESS]Add decoder 001 to matrix preset 1. |  |

删除矩阵预设条目 Delete matrix preset entry

| API接口 API interface |  |
| --- | --- |
| DELETE MATRIX PRESET [prs] DEC [dec] |  |
| 描述 describe |  |
| 删除矩阵预设条目 Delete preset matrix entry |  |
| 参数 parameter | 描述 describe |
| prs | [01...30]: 预设ID号 [01... 30]: Preset ID number |
| dec | [001…762]: RX ID号 0: 所有RX [001... 762]: RX ID number 0: All RXs |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Delete decoder 001 from matrix preset 1. | 删除矩阵预设1中RX1条目 Delete RX1 entry in matrix preset 1 |
| 样例 Example |  |
| TELNET登录CTL100D 删除矩阵预设1中RX1条目，输入命令： DELETE MATRIX PRESET 10 DEC 1 SOURCE 1 返回： [SUCCESS]Delete decoder 001 from matrix preset 1. TELNET login CTL100D Delete the RX1 entry in matrix preset 1, enter the command: DELETE MATRIX PRESET 10 DEC 1 SOURCE 1 return: [SUCCESS]Delete decoder 001 from matrix preset 1. |  |

应用矩阵预设 Apply matrix preset

| API接口 API interface |  |
| --- | --- |
| APPLY MATRIX PRESET [prs] |  |
| 描述 describe |  |
| 应用矩阵预设 Apply matrix preset |  |
| 参数 parameter | 描述 describe |
| prs | [01...30]: 预设ID号 [01... 30]: Preset ID number |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Apply matrix preset 1. | 应用矩阵预设1 Apply Matrix Preset 1 |
| 样例 Example |  |
| TELNET登录CTL100D 应用矩阵预设1，输入命令： APPLY MATRIX PRESET 1 返回： [SUCCESS]Apply matrix preset 1. TELNET login CTL100D Apply matrix preset 1, enter the command: APPLY MATRIX PRESET 1 return: [SUCCESS]Apply matrix preset 1. |  |

系统管理模块API参考 System Management Module API Reference

设备搜索 Device search

| API接口 API interface |  |
| --- | --- |
| SEARCH |  |
| 描述 describe |  |
| 搜索在线设备 Search for online devices |  |
| 参数 parameter | 描述 describe |
| 无 none |  |
| 返回值 Return value | 描述 describe |
| 返回当前系统在线设备信息 Return the current system's online device information |  |
| 样例 Example |  |
| TELNET登录CTL100D 搜索在线设备，输入命令： SEARCH 返回： TELNET login CTL100D Search for online devices, enter the command: SEARCH return: [SUCCESS]More device in network will take more time to finish scan, please wait...done. ================================================================ Scan Device Result Info ==New Encoder None ==Project Control Encoder Index IP MAC ID 001 169.254.010.001 6c:df:fb:00:a9:5f 001 002 169.254.010.002 6c:df:fb:00:84:3c 002 ==New Decoder None ==Project Control Decoder Index IP MAC ID 001 169.254.020.001 6c:df:fb:00:36:0d 001 002 169.254.020.002 6c:df:fb:00:84:39 002 ================================================================ |  |

查看设备搜索结果 View device search results

| API接口 API interface |  |
| --- | --- |
| GET SEARCH STATUS |  |
| 描述 describe |  |
| 查看设备搜索结果 View device search results |  |
| 参数 parameter | 描述 describe |
| 无 none |  |
| 返回值 Return value | 描述 describe |
| 返回当前系统在线设备信息 Return the online device information of the current system |  |
| 样例 Example |  |
| TELNET登录CTL100D 查看设备搜索结果，输入命令： GET SEARCH STATUS 返回： TELNET login CTL100D View device search results, enter the command: GET SEARCH STATUS return: ================================================================ Scan Device Result Info ==New Encoder None ==Project Control Encoder Index IP MAC ID 001 169.254.010.001 6c:df:fb:00:a9:5f 001 002 169.254.010.002 6c:df:fb:00:84:3c 002 ==New Decoder None ==Project Control Decoder Index IP MAC ID 001 169.254.020.001 6c:df:fb:00:36:0d 001 002 169.254.020.002 6c:df:fb:00:84:39 002 ================================================================ |  |

清空设备搜索结果Clear device search results

| API接口 API interface |  |
| --- | --- |
| SEARCH RESET |  |
| 描述 describe |  |
| 清空设备搜索结果 Clear device search results |  |
| 参数 parameter | 描述 describe |
| 无 none |  |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Reset search info. | 清空设备搜索结果 Clear device search results |
| 样例 Example |  |
| TELNET登录CTL100D 清空设备搜索结果，输入命令： SEARCH RESET 返回： [SUCCESS]Reset search info. TELNET login CTL100D Clear the device search results and enter the command: SEARCH RESET return: [SUCCESS]Reset search info. |  |

自动加入新设备到系统Automatically add new devices to the system

| API接口 API interface |  |
| --- | --- |
| ADD AUTO ALL |  |
| 描述 describe |  |
| 自动将新设备加入到系统中，调用本接口前需要先调用SEARCH接口搜索在线设备。 Automatically add new devices to the system. Before calling this interface, you need to call the SEARCH interface to search for online devices. |  |
| 参数 parameter | 描述 describe |
| 无 none |  |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Add scan index 001 device to decoder 003. [SUCCESS]Add scan index 002 device to decoder 004. [SUCCESS]Add scan index 003 device to decoder 002. [SUCCESS]Add scan index 004 device to decoder 001. | 添加新设备成功。 New devices are added successfully. |
| 样例 Example |  |
| TELNET登录CTL100D 自动将新设备加入到系统，输入命令： ADD AUTO ALL 返回： [SUCCESS]Add scan index 001 device to decoder 003. [SUCCESS]Add scan index 002 device to decoder 004. [SUCCESS]Add scan index 003 device to decoder 002. [SUCCESS]Add scan index 004 device to decoder 001. TELNET login CTL100D Automatically add new devices to the system, enter the command: ADD AUTO ALL return: [SUCCESS]Add scan index 001 device to decoder 003. [SUCCESS]Add scan index 002 device to decoder 004. [SUCCESS]Add scan index 003 device to decoder 002. [SUCCESS]Add scan index 004 device to decoder 001. |  |

加入新TX设备到系统 Add a new TX device to the system

| API接口 API interface |  |
| --- | --- |
| ADD DEV [dev] ENC [enc] |  |
| 描述 describe |  |
| 加入新TX设备到系统中，调用本接口前需要先调用SEARCH接口搜索在线设备。 Add a new TX device to the system. Before calling this interface, you need to call the SEARCH interface to search for online devices. |  |
| 参数 parameter | 描述 describe |
| dev | SEARCH结果中New Encoder的Index Index of New Encoder in SEARCH results |
| enc | [001...254]: TX ID号 0: 系统自动分配ID [001... 254]: TX ID number 0: The system automatically assigns IDs |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Add scan index 001 device to encoder 004. | 将New Encoder中Index为1的TX设备加入到系统，并分配ID 4 Add the TX device with Index 1 from the New Encoder to the system and assign ID 4 |
| 样例 Example |  |
| TELNET登录CTL100D 将New Encoder中Index为1的TX设备加入到系统，并分配ID 4，输入命令： ADD DEV 1 ENC 4 返回： [SUCCESS]Add scan index 001 device to encoder 004. TELNET login CTL100D Add the TX device with Index 1 from the New Encoder to the system and assign ID 4. Enter the command: ADD DEV 1 ENC 4 return: [SUCCESS]Add scan index 001 device to encoder 004. |  |

加入新RX设备到系统 Add a new RX device to the system

| API接口 API interface |  |
| --- | --- |
| ADD DEV [dev] DEC [dec] |  |
| 描述 describe |  |
| 加入新RX设备到系统中，调用本接口前需要先调用SEARCH接口搜索在线设备。 Add a new RX device to the system. Before calling this interface, you need to call the SEARCH interface to search for online devices. |  |
| 参数 parameter | 描述 describe |
| dev | SEARCH结果中New Decoder的Index Index of New Decoder in SEARCH results |
| dec | [001...762]: RX ID号 0: 系统自动分配ID [001... 762]: RX ID number 0: The system automatically assigns IDs |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Add scan index 001 device to decoder 001. | 将New Decoder中Index为1的RX设备加入到系统，并分配ID 1 Add the RX device with Index 1 from the New Decoder to the system and assign ID 1 |
| 样例 Example |  |
| TELNET登录CTL100D 将New Decoder中Index为1的RX设备加入到系统，并分配ID 1，输入命令： ADD DEV 1 DEC 1 返回： [SUCCESS]Add scan index 001 device to decoder 001. TELNET login CTL100D Add the RX device with Index 1 from the New Decoder to the system and assign ID 1. Enter the command: ADD DEV 1 DEC 1 return: [SUCCESS]Add scan index 001 device to decoder 001. |  |

清空系统中的已有设备Clear existing devices from the system

| API接口 API interface |  |
| --- | --- |
| ADD DEV RESET |  |
| 描述 describe |  |
| 清空系统中的已有设备，并会清空与设备有关的信息，如VW。 Clearing existing devices in the system will also clear information related to the devices, such as VW. |  |
| 参数 parameter | 描述 describe |
| 无 none |  |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Reset all Encoder/Decoder/Videowall/Search configuration. | 清空系统中的已有设备及相关配置 Clear the existing devices and related configurations in the system |
| 样例 Example |  |
| TELNET登录CTL100D 清空系统中的已有设备，输入命令： ADD DEV RESET 返回： [SUCCESS]Reset all Encoder/Decoder/Videowall/Search configuration. TELNET login CTL100D Clear the existing devices in the system and enter the command: ADD DEV RESET return: [SUCCESS]Reset all Encoder/Decoder/Videowall/Search configuration. |  |

CTL100D网络配置API参考
CTL100D Network Configuration API Reference

设置CTL100D IP模式  Set CTL100D IP mode

| API接口 API interface |  |
| --- | --- |
| SET NETWORK [lan] DHCP ON/OFF |  |
| 描述 describe |  |
| 设置CTL100D CONTROL LAN口和VIDEO LAN口的IP模式。 Set the IP mode for CTL100D CONTROL LAN port and VIDEO LAN port. |  |
| 参数 parameter | 描述 describe |
| lan | LAN1: VIDEO LAN port LAN2: CONTROL LAN port |
| ON/OFF | ON: 启用DHCP OFF: 关闭DHCP，采用静态IP ON: Enable DHCP OFF: Disable DHCP and use static IP |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Set lan2 DHCP to on. Use "SET NETWORK REBOOT" command or repower device to apply new config!!! | 设置CONTROL LAN口为DHCP模式 Set the CONTROL LAN port to DHCP mode |
| 样例 Example |  |
| TELNET登录CTL100D 设置CONTROL LAN口为DHCP模式，输入命令： SET NETWORK LAN2 DHCP ON 返回： [SUCCESS]Set lan2 DHCP to on. Use "SET NETWORK REBOOT" command or repower device to apply new config!!! TELNET login CTL100D Set the CONTROL LAN port to DHCP mode and enter the command: SET NETWORK LAN2 DHCP ON return: [SUCCESS]Set lan2 DHCP to on. Use "SET NETWORK REBOOT" command or repower device to apply new config!!! |  |

设置CTL100D IP地址 Set CTL100D IP address

| API接口 API interface |  |
| --- | --- |
| SET NETWORK [lan] STATIC IP [ip] |  |
| 描述 describe |  |
| 设置CTL100D CONTROL LAN口和VIDEO LAN口的IP地址，只有当网口设置为静态IP模式时才有效。 Set the IP addresses of CTL100D CONTROL LAN port and VIDEO LAN port, which are only valid when the network port is set to static IP mode. |  |
| 参数 parameter | 描述 describe |
| lan | LAN1: VIDEO LAN port LAN2: CONTROL LAN port |
| ip | IP地址，如169.254.20.10 IP address, such as 169.254.20.10 |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Set lan2 IP address to 192.168.070.040. Use "SET NETWORK REBOOT" command or repower device to apply new config!!! | 设置CONTROL LAN口的IP为192.168.70.40 Set the IP address of CONTROL LAN port to 192.168.70.40 |
| 样例 Example |  |
| TELNET登录CTL100D 设置CONTROL LAN口的IP为192.168.70.40，输入命令： SET NETWORK LAN2 STATIC IP 192.168.70.40 返回： [SUCCESS]Set lan2 IP address to 192.168.070.040. Use "SET NETWORK REBOOT" command or repower device to apply new config!!! TELNET login CTL100D Set the IP address of CONTROL LAN port to 192.168.70.40 and enter the command: SET NETWORK LAN2 STATIC IP 192.168.70.40 return: [SUCCESS]Set lan2 IP address to 192.168.070.040. Use "SET NETWORK REBOOT" command or repower device to apply new config!!! |  |

设置CTL100D网关地址 Set CTL100D gateway address

| API接口 API interface |  |
| --- | --- |
| SET NETWORK [lan] STATIC GATEWAY [gw] |  |
| 描述 describe |  |
| 设置CTL100D CONTROL LAN口和VIDEO LAN口的网关地址，只有当网口设置为静态IP模式时才有效。 Set the gateway addresses for CTL100D CONTROL LAN port and VIDEO LAN port, which are only valid when the network port is set to static IP mode. |  |
| 参数 parameter | 描述 describe |
| lan | LAN1: VIDEO LAN port LAN2: CONTROL LAN port |
| gw | 网关地址，如169.254.0.1 Gateway address, such as 169.254.0.1 |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Set lan2 gateway address to 192.168.070.001. Use "SET NETWORK REBOOT" command or repower device to apply new config!!! | 设置CONTROL LAN口的网关地址为192.168.70.1 Set the gateway address for the CONTROL LAN port to 192.168.70.1 |
| 样例 Example |  |
| TELNET登录CTL100D 设置CONTROL LAN口的网关地址为192.168.70.1，输入命令： SET NETWORK LAN2 STATIC GATEWAY 192.168.70.1 NET RB 返回： [SUCCESS]Set lan2 gateway address to 192.168.070.001. Use "SET NETWORK REBOOT" command or repower device to apply new config!!! TELNET login CTL100D Set the gateway address for the CONTROL LAN port to 192.168.70.1, enter the command: SET NETWORK LAN2 STATIC GATEWAY 192.168.70.1 NET RB return: [SUCCESS]Set lan2 gateway address to 192.168.070.001. Use "SET NETWORK REBOOT" command or repower device to apply new config!!! |  |

设置CTL100D子网掩码 Set CTL100D subnet mask

| API接口 API interface |  |
| --- | --- |
| SET NETWORK [lan] STATIC MASK [mask] |  |
| 描述 describe |  |
| 设置CTL100D CONTROL LAN口和VIDEO LAN口的子网掩码，只有当网口设置为静态IP模式时才有效。 Set the subnet mask for CTL100D CONTROL LAN port and VIDEO LAN port, which is only valid when the network port is set to static IP mode. |  |
| 参数 parameter | 描述 describe |
| lan | LAN1: VIDEO LAN port LAN2: CONTROL LAN port |
| mask | 子网掩码，如255.255.0.0 Subnet mask, such as 255.255.0.0 |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Set lan2 subnet mask address to 255.255.255.000. Use "SET NETWORK REBOOT" command or repower device to apply new config!!! | 设置CONTROL LAN口的子网掩码为255.255.255.0 Set the subnet mask of the CONTROL LAN port to 255.255.255.0 |
| 样例 Example |  |
| TELNET登录CTL100D 设置CONTROL LAN口的子网掩码为255.255.255.0，输入命令： SET NETWORK LAN2 STATIC MASK 255.255.255.0 返回： [SUCCESS]Set lan2 subnet mask address to 255.255.255.000. Use "SET NETWORK REBOOT" command or repower device to apply new config!!! TELNET login CTL100D Set the subnet mask of the CONTROL LAN port to 255.255.255.0, enter the command: SET NETWORK LAN2 STATIC MASK 255.255.255.0 return: [SUCCESS]Set lan2 subnet mask address to 255.255.255.000. Use "SET NETWORK REBOOT" command or repower device to apply new config!!! |  |

重启CTL100D网卡 Reboot CTL100D network

| API接口 API interface |  |
| --- | --- |
| SET NETWORK REBOOT |  |
| 描述 describe |  |
| 重启CTL100D网卡，修改了网络参数后需要发送此API使配置生效。 After rebooting the CTL100D network and modifying the network parameters, this API needs to be sent for the configuration to take effect. |  |
| 参数 parameter | 描述 describe |
| 无 none |  |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Set network reboot and apply new config. | 重启网卡成功 Reboot network successfully |
| 样例 Example |  |
| TELNET登录CTL100D 重启网卡，输入命令： SET NETWORK REBOOT 返回： [SUCCESS]Set network reboot and apply new config. TELNET login CTL100D Reboot the network and enter the command: SET NETWORK REBOOT return: [SUCCESS]Set network reboot and apply new config. |  |

设置CTL100D TELNET开关 Set CTL100D TELNET on/off

| API接口 API interface |  |
| --- | --- |
| SET NETWORK TELNET ON/OFF |  |
| 描述 describe |  |
| 设置CTL100D TELNET开关，默认开启TELNET。 Set the CTL100D TELNET on. TELNET is enabled by default. |  |
| 参数 parameter | 描述 describe |
| ON/OFF | ON：开启TELNET OFF: 关闭TELNET ON: Enable TELNET OFF: Disable TELNET |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Set telnet port on. | 开启TELNET Set TELNET on |
| 样例 Example |  |
| TELNET登录CTL100D 开启TELNET，输入命令： SET NETWORK TELNET ON 返回： [SUCCESS]Set telnet port on. TELNET login CTL100D Open TELNET and enter the command: SET NETWORK TELNET ON return: [SUCCESS]Set telnet port on. |  |

设置CTL100D TELNET端口号 Set CTL100D TELNET port number

| API接口 API interface |  |
| --- | --- |
| SET NETWORK TELNET PORT [port] |  |
| 描述 describe |  |
| 设置CTL100D TELNET端口号，默认为23 Set CTL100D TELNET port number, default is 23 |  |
| 参数 parameter | 描述 describe |
| port | TELNET端口号，最大值为65535 TELNET port number, maximum value 65535 |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Set telnet port to 0030. | 设置TELNET端口号为30 Set the TELNET port number to 30 |
| 样例 Example |  |
| TELNET登录CTL100D 设置TELNET端口号为30，输入命令： SET NETWORK TELNET PORT 30 返回： [SUCCESS]Set telnet port to 0030. TELNET login CTL100D Set the TELNET port number to 30 and enter the command: SET NETWORK TELNET PORT 30 return: [SUCCESS]Set telnet port to 0030. |  |

设置CTL100D SSH开关 Set CTL100D SSH on/off

| API接口 API interface |  |
| --- | --- |
| SET NETWORK SSH ON/OFF |  |
| 描述 describe |  |
| 设置CTL100D SSH开关，默认不开启SSH。 Set CTL100D SSH on. SSH is disabled by default. |  |
| 参数 parameter | 描述 describe |
| ON/OFF | ON：开启SSH OFF: 关闭SSH ON: Enable SSH OFF: Disable SSH |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Set ssh port on. | 开启SSH Enable SSH |
| 样例 Example |  |
| TELNET登录CTL100D 开启SSH，输入命令： SET NETWORK SSH ON 返回： [SUCCESS]Set ssh port on. TELNET login CTL100D Enable SSH and enter the command: SET NETWORK SSH ON return: [SUCCESS]Set ssh port on. |  |

设置CTL100D SSH端口号 Set CTL100D SSH port number

| API接口 API interface |  |
| --- | --- |
| SET NETWORK SSH PORT [port] |  |
| 描述 describe |  |
| 设置CTL100D SSH端口号，默认为22 Set CTL100D SSH port number, default is 22 |  |
| 参数 parameter | 描述 describe |
| port | SSH端口号，最大值为65535 SSH port number, maximum value 65535 |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Set ssh port to 0040. | 设置SSH端口号为40 Set SSH port number to 40 |
| 样例 Example |  |
| TELNET登录CTL100D 设置TELNET端口号为40，输入命令： SET NETWORK SSH PORT 40 返回： [SUCCESS]Set ssh port to 0040. TELNET login CTL100D Set the TELNET port number to 40 and enter the command: SET NETWORK SSH PORT 40 return: [SUCCESS]Set ssh port to 0040. |  |

设置CTL100D HTTPS开关  Set CTL100D HTTPS on/off

| API接口 API interface |  |
| --- | --- |
| SET NETWORK HTTPS ON/OFF |  |
| 描述 describe |  |
| 设置CTL100AL HTTPS开关，默认不开启HTTPS。 Set CTL100AL HTTPS on. HTTPS is disabled by default. |  |
| 参数 parameter | 描述 describe |
| ON/OFF | ON：开启HTTPS OFF: 关闭HTTPS ON: Enable HTTPS OFF: Disable HTTPS |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Set web gui https on. | 开启HTTPS Enable HTTPS |
| 样例 Example |  |
| TELNET登录CTL100D 开启HTTPS，输入命令： SET NETWORK HTTPS ON 返回： [SUCCESS]Set web gui https on. TELNET login CTL100D Enable HTTPS, enter the command: SET NETWORK HTTPS ON return: [SUCCESS]Set web gui https on. |  |

设置CTL100D WEB开关  Set CTL100D WEB on/off

| API接口 API interface |  |
| --- | --- |
| SET NETWORK WEB ON/OFF |  |
| 描述 describe |  |
| 设置CTL100D WEB GUI开关，默认开启WEB GUI。 Set the CTL100D WEB GUI on. The WEB GUI is enabled by default. |  |
| 参数 parameter | 描述 describe |
| ON/OFF | ON：开启WEB OFF: 关闭WEB ON: Enable WEB OFF: Disable WEB |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Set web gui on. | 开启WEB Enable WEB |
| 样例 Example |  |
| TELNET登录CTL100D 开启WEB，输入命令： SET NETWORK WEB ON 返回： [SUCCESS]Set web gui on. TELNET login CTL100D Set WEB on, and enter the command: SET NETWORK WEB ON return: [SUCCESS]Set web gui on. |  |

修改CTL100D域名 Modify CTL100D domain name

| API接口 API interface |  |
| --- | --- |
| SET NETWORK DNS hostname |  |
| 描述 describe |  |
| 修改CTL100D的域名，默认域名为controller.local。 Change the domain name of CTL100D. The default domain name is controller.local. |  |
| 参数 parameter | 描述 describe |
| hostname | 域名，只支持字母、数字，以及- _等特殊字符。 Domain name only supports letters, numbers, and special characters such as - _. |
| 返回值 Return value | 描述 describe |
| [SUCCESS]Set DNS domain name to test.local. it will take about 40 seconds, and RS-232 will disable during this time, please wait... | 设置域名为test.local Set the domain name to test.local |
| 样例 Example |  |
| TELNET登录CTL100D 设置域名为test.local，输入命令： SET NETWORK DNS test 返回： [SUCCESS]Set DNS domain name to test.local. it will take about 40 seconds, and RS-232 will disable during this time, please wait... TELNET login CTL100D Set the domain name to test.local and enter the command: SET NETWORK DNS test return: [SUCCESS]Set DNS domain name to test.local. it will take about 40 seconds, and RS-232 will disable during this time, please wait... |  |