| Revision History |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No. | Summary of Changes |  | Version | Revised By | Revision Date |
| 1 | Initial Release |  | 1.0 | Jia Ni | 2025-9-25 |
| Approved by: Date: Date: |  | Reviewed by: Date: Date: | Prepared by: Date: Date: |  |  |

Contents

Overview	7

Connection and Usage Instructions	7

1 Help and Status API	8

?/HELP	8

GET STATUS	9

GET VERSION	10

2 System and Controller API	11

SET RS232BAUDRATE [a]	11

SET RESET	11

SET RESET NETWORK	12

SET RESET ALL	12

3 Encoder Control API	13

SET ENC PRESET IPMODE [mode]	13

SET ENC PRESET START IP [ip]	13

SET ENC PRESET END IP [ip]	14

SET ENC PRESET GW [gw]	14

SET ENC PRESET SM [mask]	15

SET ENC PRESET APPLY	15

SET ENC [enc] STATIC IP [ip]	16

SET ENC [enc] STATIC GATEWAY [gw]	16

SET ENC [enc] STATIC MASK [mask]	17

SET ENC [enc] NETWORK REBOOT	18

SET ENC [enc] ID [id]	18

SET ENC [enc] DELETE	19

SET ENC [enc] REBOOT	19

SET ENC [enc] RESET	20

SET ENC [enc] AUDIO HDMI/ANALOG	20

SET ENC [enc] NAME name	21

SET ENC [enc] LED ON	21

SET ENC [enc] LED OFF	21

SET ENC [enc] RXMODE	22

GET ENC [enc] STATUS	22

SET ENC [enc] GUEST [ON/OFF] [BR br] [BIT bit]	23

SET ENC [enc] SENDGUEST ASCII [message]	23

SET ENC [enc] SENDGUEST HEX [message]	23

SET ENC [enc] EDID COPY [dec]	24

SET ENC [enc] EDID DEFAULT [zz]	24

SET ENC [enc] CODECMODE TXT/IMG	25

SET ENC [enc] CODECFMT YUV444/YUV422/YUV420	25

SET ENC [enc] CODECDELAY [dly]	26

SET ENC [enc] MAINCODE H265/H264	26

SET ENC [enc] MAINBRATYPE FIX/VAR	27

SET ENC [enc] MAINIMAGE [imgqul]	27

SET ENC [enc] MAINBRA [bitrate1]	28

SET ENC [enc] SUBCODE H265/H264	28

SET ENC [enc] SUBBRATYPE FIX/VAR	29

SET ENC [enc] SUBIMAGE [imgqul]	29

SET ENC [enc] SUBBRA [bitrate2]	30

SET ENC [enc] PREVIEWCODE H265/H264	30

SET ENC [enc] PREVIEWBRATYPE FIX/VAR	31

SET ENC [enc] PREVIEWIMAGE [imgqul]	31

SET ENC [enc] PREVIEWBRA [bitrate3]	32

SET ENC [enc] SDKSTREAM ON/OFF	32

SET ENC [enc] SDKCODE H265/H264	33

SET ENC [enc] SDKBRATYPE FIX/VAR	33

SET ENC [enc] SDKIMAGE [imgqul]	34

SET ENC [enc] SDKBRA [bitrate4]	34

SET ENC [enc] SUBSDKSTREAM ON/OFF	35

SET ENC [enc] SUBSDKCODE H265/H264	35

SET ENC [enc] SUBSDKBRATYPE FIX/VAR	36

SET ENC [enc] SUBSDKIMAGE [imgqul]	36

SET ENC [enc] SUBSDKBRA [bitrate5]	37

SET ENC [enc] KVMOS [os]	37

4 Decoder Control API	38

SET DEC PRESET IPMODE [mode]	38

SET DEC PRESET START IP [ip]	38

SET DEC PRESET END IP [ip]	39

SET DEC PRESET GW [gw]	39

SET DEC PRESET SM [mask]	40

SET DEC PRESET APPLY	40

SET DEC [dec] STATIC IP [ip]	41

SET DEC [dec] STATIC GATEWAY [gw]	41

SET DEC [dec] STATIC MASK [mask]	42

SET DEC [dec] NETWORK REBOOT	43

SET DEC [dec] ID [id]	43

SET DEC [dec] DELETE	44

SET DEC [dec] REBOOT	44

SET DEC [dec] RESET	45

SET DEC [dec] NAME name	45

SET DEC [dec] LED ON	46

SET DEC [dec] LED OFF	46

SET DEC [dec] TXMODE	46

GET DEC [dec] STATUS	47

SET DEC [dec] GUEST [ON/OFF] [BR br] [BIT bit]	47

SET DEC [dec] SENDGUEST ASCII [message]	48

SET DEC [dec] SENDGUEST HEX [message]	48

SET DEC [dec] SWITCH [enc] ALL USER username	49

SET DEC [dec] SWITCH [enc] RS232	49

SET DEC [dec] OUTPUT OSD ON	50

SET DEC [dec] OUTPUT OSD OFF	50

SET DEC [dec] RESOLUTION [res]	50

SET DEC [dec] ROTATE [rtt]	51

SET DEC [dec] DECVIDDELAY [decdly]	52

SET DEC [dec] OSDCOL BLACK/WHITE	52

SET DEC [dec] OSDPOS LT/RT/LB/RB	53

SET DEC [dec] TESTMODE [tst]	53

SET DEC [dec] SIGTYPE HDMI/DVI	54

SET DEC [dec] AUOUTVOL [vol]	54

SET DEC [dec] KVMMOUSE ON/OFF	55

SET DEC [dec] KVMAROUND ON/OFF	55

5 Video Wall Control API	56

CREATE WALL HANDLE [hdl]	56

DELETE WALL HANDLE [hdl]	56

APPLY WALL [hdl] PRESET [pre]	57

SET WALL [hdl] C [c] R [r] RES [res]	57

SET WALL [hdl] CRES TOTAL [tx] [ty] [tref] DEV [dx] [dy] [dref]	58

SET WALL [hdl] CUT HOR LEFT/RIGHT [ch] VER TOP/BOTTOM [cv]	58

SET WALL [hdl] BEZEL [bu] [bd] [bl] [br]	59

SET WALL [hdl] ACTBEZEL [bu] [bd] [bl] [br]	59

SET WALL [hdl] ROTATEMODE [rttmode]	60

SET WALL [hdl] H [posx] V [posy] ROTATE [rtt]	60

SET WALL [hdl] NAME [name]	61

SET WALL [hdl] DEC [dec] H [h] V [h]	61

SET WALL [hdl] PRESET [pre] WND [wnd] CREATE [posx] [posy] [width] [height] NAME window1	62

SET WALL [hdl] PRESET [pre] WND [wnd] RESIZE [posx] [posy] [width] [height]	63

SET WALL [hdl] PRESET [pre] WND [wnd] SOURCE [enc]	63

SET WALL [hdl] PRESET [pre] WND [wnd] TOP/BOTTOM	64

SET WALL [hdl] PRESET [pre] WND [wnd] DELETE	64

6 Marquee Control API	65

7 Matrix Preset Control API	65

8 Device Management API	65

9 Network Management API	65

Overview

This document describes the command-line API supported by the HDN-CTL500H controller.

Connection and Usage Instructions

Enter the following API commands through the RS-232 control port, a Telnet session, or an SSH session to execute the corresponding functions.

The API commands are organized into the following sections.

1 Help and Status API

?/HELP

| API Command |  |
| --- | --- |
| ?/HELP |  |
| Description |  |
| Displays the command help information. |  |
| Parameter | Parameter Description |
| None | None |
| Return Value | Return Value Description |
| [HELP TEXT] | Returns a list of all API commands with brief descriptions. |
| Example |  |
| CONTROLLER> help ================================================================ CTL500H Help Information FW Version: 2.00.03A /*Help And Status Commands*/ ?/HELP : Print help information. GET STATUS : Get all status. GET VERSION : Get all version. /*Commands For System Control*/ SET RS232BAUDRATE [a] : Set RS-232 baud rate to a bps. a=[0:115200 1:57600, 2:38400, 3:19200, 4:9600] SET RESET : Reset controller system settings. SET RESET NETWORK : Reset controller network settings. SET RESET ALL : Reset controller system and network settings. ..... SET NETWORK SSH ON/OFF : Set the controller network ssh ON or OFF SET NETWORK SSH PORT [port] : Set the controller network ssh port SET NETWORK TELNET ON/OFF : Set the controller network telnet ON or OFF SET NETWORK TELNET PORT port : Set the controller network telnet port SET NETWORK WEB ON/OFF : Set the controller network web GUI ON or OFF SET NETWORK HTTPS ON/OFF : Set the controller network https ON or OFF SET NETWORK DNS [hostname] : Set the controller network domain name to [hostname] ================================================================ |  |

GET STATUS

| API Command |  |
| --- | --- |
| GET STATUS |  |
| Description |  |
| Returns the overall system and connected-device status. |  |
| Parameter | Parameter Description |
| None | None |
| Return Value | Return Value Description |
| [STATUS DATA] | Returns comprehensive system and connected-device status information. |
| Example |  |
| CONTROLLER> get status ================================================================ CTL500H Status Information FW Version: 2.00.03A Power IR Baud On On 115200 In EDID IP NET/Sig SNAPURL 001 DF004 169.254.010.001 On /On 169.254.10.1 002 DF000 169.254.010.002 On /On 169.254.10.2 Out FromIn IP NET Res Mode 001 --- 169.254.020.001 On 12 VW01 002 --- 169.254.020.002 On 07 VW02 LAN DHCP IP Gateway SubnetMask 01_POE Off 169.254.008.100 169.254.008.001 255.255.000.000 02_CTRL On 192.168.080.073 192.168.081.001 255.255.254.000 (static:192.168.006.100 192.168.006.001 255.255.255.000) Telnet SSH HTTPS WEB LAN01 MAC LAN02 MAC 0023 Off Off On 6C:DF:FB:00:01:2D 6C:DF:FB:00:01:21 DomainName controller.local ================================================================ |  |

GET VERSION

| API Command |  |
| --- | --- |
| GET VERSION |  |
| Description |  |
| Returns controller and connected-device version information. |  |
| Parameter | Parameter Description |
| None | None |
| Return Value | Return Value Description |
| [VERSION INFO] | Returns firmware, hardware, and other system version information. |
| Example |  |
| GET VERSION ================================================================ CTL500H Version Information FW Version: 2.00.03A Out 001 WMR6 Version Ip Mac 20250916(0030) 169.254.020.001 6C:DF:FB:2B:02:12 MCU Version 1.18.44 002 WMR3 Version Ip Mac 20250909(0030) 169.254.020.002 6C:DF:FB:2B:02:46 MCU Version 0.0.0 ================================================================ In 001 WMR6 Version Ip Mac 20250916(0030) 169.254.010.001 6C:DF:FB:2B:01:19 MCU Version 1.18.44 002 WMR3 Version Ip Mac 20250909(0030) 169.254.010.002 6C:DF:FB:2B:02:44 MCU Version 0.0.0 ================================================================ |  |

2 System and Controller API

SET RS232BAUDRATE [a]

| API Command |  |
| --- | --- |
| SET RS232BAUDRATE [a] |  |
| Description |  |
| Sets the controller RS-232 baud rate. |  |
| Parameter | Parameter Description |
| [a] | Select the baud rate: a=[0:115200 1:57600, 2:38400, 3:19200, 4:9600] |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration completed successfully. |
| Example |  |
| CONTROLLER> SET RS232BAUDRATE 0 [SUCCESS]Set RS-232 Baud Rate to 115200bps. CONTROLLER> |  |

SET RESET

| API Command |  |
| --- | --- |
| SET RESET |  |
| Description |  |
| Resets the controller system settings. |  |
| Parameter | Parameter Description |
| None | None |
| Return Value | Return Value Description |
| [SUCCESS] | Reset completed successfully. |
| Example |  |
| CONTROLLER> SET RESET Sure to RESET system to default settings? Type "Yes" after next prompt to confirm... CONTROLLER> |  |

SET RESET NETWORK

| API Command |  |
| --- | --- |
| SET RESET NETWORK |  |
| Description |  |
| Resets the controller network settings. |  |
| Parameter | Parameter Description |
| None | Enter "YES" to confirm the operation. |
| Return Value | Return Value Description |
| [SUCCESS] | Reset completed successfully. |
| Example |  |
| CONTROLLER> SET RESET NETWORK Sure to RESET network config to default settings? Type "Yes" after next prompt to confirm... CONTROLLER> |  |

SET RESET ALL

| API Command |  |
| --- | --- |
| SET RESET ALL |  |
| Description |  |
| Resets all controller system and network settings. |  |
| Parameter | Parameter Description |
| None | Enter "YES" to confirm the operation. |
| Return Value | Return Value Description |
| [SUCCESS] | Reset completed successfully. |
| Example |  |
| CONTROLLER> SET RESET ALL Sure to RESET system and network to default settings? Type "Yes" after next prompt to confirm... CONTROLLER> |  |

3 Encoder Control API

SET ENC PRESET IPMODE [mode]

| API Command |  |
| --- | --- |
| SET ENC PRESET IPMODE [mode] |  |
| Description |  |
| Sets the preset encoder IP assignment mode. |  |
| Parameter | Parameter Description |
| [mode] | Options: DHCP or STATIC |
| Return Value | Return Value Description |
| [SUCCESS] | Preset configuration completed successfully. |
| [FAILED] |  |
| Example | Example |
| CONTROLLER> SET ENC PRESET IPMODE STATIC CONTROLLER> |  |

SET ENC PRESET START IP [ip]

| API Command |  |
| --- | --- |
| SET ENC PRESET START IP [ip] |  |
| Description |  |
| Sets the first IP address in the preset encoder address pool. |  |
| Parameter | Parameter Description |
| [ip] | Valid IPv4 address (e.g., 192.168.1.100) |
| Return Value | Return Value Description |
| [SUCCESS] | Preset configuration completed successfully. |
| [FAILED] | Failed to configure the preset. |
| Example |  |
| CONTROLLER> SET ENC PRESET START IP 192.168.50.15 [SUCCESS]Input preset start IP address set OK. CONTROLLER> |  |

SET ENC PRESET END IP [ip]

| API Command |  |
| --- | --- |
| SET ENC PRESET END IP [ip] |  |
| Description |  |
| Sets the last IP address in the preset encoder address pool. |  |
| Parameter | Parameter Description |
| [ip] | Valid IPv4 address (must be greater than the start IP address) |
| Return Value | Return Value Description |
| [SUCCESS] | Preset configuration completed successfully. |
| [FAILED] | Failed to configure the preset. |
| Example |  |
| CONTROLLER> SET ENC PRESET END IP 192.168.1.200 [SUCCESS]Input preset end IP address set OK. CONTROLLER> |  |

SET ENC PRESET GW [gw]

| API Command |  |
| --- | --- |
| SET ENC PRESET GW [gw] |  |
| Description |  |
| Sets the preset encoder gateway address. |  |
| Parameter | Parameter Description |
| [gw] | Valid IPv4 gateway address |
| Return Value | Return Value Description |
| [SUCCESS] | Preset configuration completed successfully. |
| [FAILED] | Failed to configure the preset. |
| Example |  |
| CONTROLLER> SET ENC PRESET GW 192.168.1.1 [SUCCESS]Encoder preset gateway set OK. CONTROLLER> |  |

SET ENC PRESET SM [mask]

| API Command |  |
| --- | --- |
| SET ENC PRESET SM [mask] |  |
| Description |  |
| Sets the preset encoder subnet mask. |  |
| Parameter | Parameter Description |
| [mask] | Valid subnet mask (e.g., 255.255.255.0) |
| Return Value | Return Value Description |
| [SUCCESS] | Preset configuration completed successfully. |
| [FAILED] | Failed to configure the preset. |
| Example |  |
| CONTROLLER> SET ENC PRESET SM 255.255.255.0 [SUCCESS]Encoder preset subnet mask set OK. CONTROLLER> |  |

SET ENC PRESET APPLY

| API Command |  |
| --- | --- |
| SET ENC PRESET APPLY |  |
| Description |  |
| Applies the preset IP configuration. |  |
| Parameter | Parameter Description |
| None | None |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration applied successfully. |
| [FAILED] | Failed to apply the configuration. |
| Example |  |
| CONTROLLER> SET ENC PRESET APPLY [SUCCESS]Input preset apply OK. CONTROLLER> |  |

SET ENC [enc] STATIC IP [ip]

| API Command |  |
| --- | --- |
| SET ENC [enc] STATIC IP [ip] |  |
| Description |  |
| Sets the encoder static IP address. |  |
| Parameter | Parameter Description |
| [enc] | Encoder ID (000 = all; 001-n = individual device) |
| [ip] | Valid IPv4 address |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration completed successfully. |
| [FAILED] | Configuration failed. |
| Example |  |
| CONTROLLER> SET ENC 001 STATIC IP 169.254.10.50 [SUCCESS]Set encoder 1 static ip 169.254.10.50 CONTROLLER> |  |

SET ENC [enc] STATIC GATEWAY [gw]

| API Command |  |
| --- | --- |
| SET ENC [enc] STATIC GATEWAY [gw] |  |
| Description |  |
| Sets the encoder static gateway. |  |
| Parameter | Parameter Description |
| [enc] | Encoder ID (000 = all; 001-n = individual device) |
| [gw] | Valid IPv4 gateway address |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration completed successfully. |
| [FAILED] | Configuration failed. |
| Example |  |
| CONTROLLER> SET ENC 001 STATIC GATEWAY 192.168.1.1 [SUCCESS]Set encoder 1 static gateway 192.168.1.1 CONTROLLER> |  |

SET ENC [enc] STATIC MASK [mask]

| API Command |  |
| --- | --- |
| SET ENC [enc] STATIC MASK [mask] |  |
| Description |  |
| Sets the encoder static subnet mask. |  |
| Parameter | Parameter Description |
| [enc] | Encoder ID (000 = all; 001-n = individual device) |
| [mask] | Valid subnet mask |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration completed successfully. |
| [FAILED] | Configuration failed. |
| Example |  |
| CONTROLLER> SET ENC 001 STATIC MASK 255.255.0.0 [SUCCESS]Set encoder 1 static netmask 255.255.0.0 CONTROLLER> |  |

SET ENC [enc] NETWORK REBOOT

| API Command |  |
| --- | --- |
| SET ENC [enc] NETWORK REBOOT |  |
| Description |  |
| Restarts the encoder network interface. |  |
| Parameter | Parameter Description |
| [enc] | Encoder ID (000 = all; 001-n = individual device) |
| Return Value | Return Value Description |
| [SUCCESS] | Reboot completed successfully. |
| [FAILED] | Failed to reboot. |
| Example |  |
| CONTROLLER> SET ENC 001 NETWORK REBOOT [SUCCESS]Set enc 1 network reboot success. CONTROLLER> |  |

SET ENC [enc] ID [id]

| API Command |  |
| --- | --- |
| SET ENC [enc] ID [id] |  |
| Description |  |
| Changes the encoder ID. |  |
| Parameter | Parameter Description |
| [enc] | Encoder ID (001-n = individual device) |
| [id] | New ID (001-199) |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration completed successfully. |
| [FAILED] | Configuration failed. |
| Example |  |
| CONTROLLER> SET ENC 1 ID 3 [SUCCESS]Set input 001 ID to 003. CONTROLLER> |  |

SET ENC [enc] DELETE

| API Command |  |
| --- | --- |
| SET ENC [enc] DELETE |  |
| Description |  |
| Removes the encoder from the current system configuration. |  |
| Parameter | Parameter Description |
| [enc] | Encoder ID (001-n = individual device) |
| Return Value | Return Value Description |
| [SUCCESS] | Deleted successfully. |
| [FAILED] | Failed to delete. |
| Example |  |
| CONTROLLER> SET ENC 2 DELETE [SUCCESS]Delete input 002 done. CONTROLLER> |  |

SET ENC [enc] REBOOT

| API Command |  |
| --- | --- |
| SET ENC [enc] REBOOT |  |
| Description |  |
| Reboots the specified encoder. |  |
| Parameter | Parameter Description |
| [enc] | Encoder ID (000 = all; 001-n = individual device) |
| Return Value | Return Value Description |
| [SUCCESS] | Reboot completed successfully. |
| [FAILED] | Failed to reboot. |
| Example |  |
| CONTROLLER> SET ENC 2 REBOOT [SUCCESS]Reboot encoder 002 SUCCESS. CONTROLLER> |  |

SET ENC [enc] RESET

| API Command |  |
| --- | --- |
| SET ENC [enc] RESET |  |
| Description |  |
| Restores the encoder to factory defaults. |  |
| Parameter | Parameter Description |
| [enc] | Encoder ID (000 = all; 001-n = individual device) |
| Return Value | Return Value Description |
| [SUCCESS] | Reset completed successfully. |
| [FAILED] | Reset failed. |
| Example |  |
| CONTROLLER> SET ENC 3 RESET [SUCCESS]Set input 003 Factory Default Reset. CONTROLLER> |  |

SET ENC [enc] AUDIO HDMI/ANALOG

| API Command |  |
| --- | --- |
| SET ENC [enc] AUDIO HDMI/ANALOG |  |
| Description |  |
| Sets the encoder audio input source. |  |
| Parameter | Parameter Description |
| [enc] | Encoder ID (000 = all; 001-n = individual device) |
| HDMI/ANALOG | Audio input source (HDMI/analog audio) |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration completed successfully. |
| [FAILED] | Configuration failed. |
| Example |  |
| SET ENC 001 AUDIO HDMI |  |

SET ENC [enc] NAME name

| API Command |  |
| --- | --- |
| SET ENC [enc] NAME name |  |
| Description |  |
| Sets the encoder name. |  |
| Parameter | Parameter Description |
| [enc] | Encoder ID (000 = all; 001-n = individual device) |
| name | Name (maximum length: 16 characters) |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration completed successfully. |
| [FAILED] | Configuration failed. |
| Example |  |
| SET ENC 001 NAME Conference_Room |  |

SET ENC [enc] LED ON

| API Command |  |
| --- | --- |
| SET ENC [enc] LED ON |  |
| Description |  |
| Enables encoder identification LED flashing. |  |
| Parameter | Parameter Description |
| [enc] | Encoder ID (000 = all; 001-n = individual device) |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration completed successfully. |
| [FAILED] | Configuration failed. |
| Example |  |
| SET ENC 001 LED ON |  |

SET ENC [enc] LED OFF

| API Command |  |
| --- | --- |
| SET ENC [enc] LED OFF |  |
| Description |  |
| Disables encoder identification LED flashing (LED remains on). |  |
| Parameter | Parameter Description |
| [enc] | Encoder ID (000 = all; 001-n = individual device) |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration completed successfully. |
| [FAILED] | Configuration failed. |
| Example |  |
| SET ENC 001 LED OFF |  |

SET ENC [enc] RXMODE

| API Command |  |
| --- | --- |
| SET ENC [enc] RXMODE |  |
| Description |  |
| Switches the encoder to decoder mode. |  |
| Parameter | Parameter Description |
| [enc] | Encoder ID |
| Return Value | Return Value Description |
| [SUCCESS] | Mode switched successfully. |
| [FAILED] | Failed to switch the operating mode. |
| Example |  |
| SET ENC 001 RXMODE |  |

GET ENC [enc] STATUS

| API Command |  |
| --- | --- |
| GET ENC [enc] STATUS |  |
| Description |  |
| Returns encoder status information. |  |
| Parameter | Parameter Description |
| [enc] | Encoder ID (001-n = individual device) |
| Return Value | Return Value Description |
| [STATUS] | Returns detailed encoder status information. |
| Example |  |
| GET ENC 001 STATUS |  |

SET ENC [enc] GUEST [ON/OFF] [BR br] [BIT bit]

| API Command |  |
| --- | --- |
| SET ENC [enc] GUEST [ON/OFF] [BR br] [BIT bit] |  |
| Description |  |
| Configures encoder serial pass-through mode. |  |
| Parameter | Parameter Description |
| [enc] | Encoder ID (001-n = individual device) |
| [ON/OFF] | Enable or disable serial pass-through mode |
| [br] | Baud rate (0=2400, 1=4800, 2=9600, 3=19200, 4=38400, 5=57600, 6=115200) |
| [bit] | Data format (e.g., 8n1: 8 data bits, no parity, 1 stop bit) |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration completed successfully. |
| [FAILED] | Configuration failed. |
| Example |  |
| SET ENC 001 GUEST ON BR 6 BIT 8n1 |  |

SET ENC [enc] SENDGUEST ASCII [message]

| API Command |  |
| --- | --- |
| SET ENC [enc] SENDGUEST ASCII [message] |  |
| Description |  |
| Sends an ASCII serial message to the encoder. |  |
| Parameter | Parameter Description |
| [enc] | Encoder ID (001-n = individual device) |
| [message] | ASCII text message (maximum length: 200 characters) |
| Return Value | Return Value Description |
| [SUCCESS] | Sent successfully. |
| [FAILED] | Failed to send. |
| Example |  |
| SET ENC 001 SENDGUEST ASCII Hello World |  |

SET ENC [enc] SENDGUEST HEX [message]

| API Command |  |
| --- | --- |
| SET ENC [enc] SENDGUEST HEX [message] |  |
| Description |  |
| Sends a hexadecimal serial message to the encoder. |  |
| Parameter | Parameter Description |
| [enc] | Encoder ID (001-n = individual device) |
| [message] | Hexadecimal data (maximum length: 200 characters) |
| Return Value | Return Value Description |
| [SUCCESS] | Sent successfully. |
| [FAILED] | Failed to send. |
| Example |  |
| SET ENC 001 SENDGUEST HEX A1B2C3 |  |

SET ENC [enc] EDID COPY [dec]

| API Command |  |
| --- | --- |
| SET ENC [enc] SENDGUEST HEX [message] |  |
| Description |  |
| Sends a hexadecimal serial message to the encoder. |  |
| Parameter | Parameter Description |
| [enc] | Encoder ID (001-n = individual device) |
| [message] | Hexadecimal data (maximum length: 200 characters) |
| Return Value | Return Value Description |
| [SUCCESS] | Sent successfully. |
| [FAILED] | Failed to send. |
| Example |  |
| SET ENC 001 SENDGUEST HEX A1B2C3 |  |

SET ENC [enc] EDID DEFAULT [zz]

| API Command |  |
| --- | --- |
| SET ENC [enc] EDID DEFAULT [zz] |  |
| Description |  |
| Sets the encoder default EDID. |  |
| Parameter | Parameter Description |
| [enc] | Encoder ID (001-n = individual device) |
| [zz] | Preset ID: zz=00: 1080P 2.0 CH zz=01: 1080I 2.0 CH zz=02: 4K2K30_444 2.0 CH zz=03: 4K2K60_420 2.0 CH zz=04: 4K2K60_444 2.0 CH(Default) zz=05: DVI 1280_1024_60 No Audio zz=06: DVI 1920_1080_60 No Audio zz=07: DVI 1920_1200_60 No Audio |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration completed successfully. |
| [FAILED] | Configuration failed. |
| Example |  |
| SET ENC 001 EDID DEFAULT 04 |  |

SET ENC [enc] CODECMODE TXT/IMG

| API Command |  |
| --- | --- |
| SET ENC [enc] CODECMODE TXT/IMG |  |
| Description |  |
| Sets the encoder content-optimized encoding mode. |  |
| Parameter | Parameter Description |
| [enc] | Encoder ID (001-n = individual device) |
| TXT/IMG | Encoding mode (text/image) |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration completed successfully. |
| [FAILED] | Configuration failed. |
| Example |  |
| SET ENC 001 CODECMODE IMG |  |

SET ENC [enc] CODECFMT YUV444/YUV422/YUV420

| API Command |  |
| --- | --- |
| SET ENC [enc] CODECFMT YUV444/YUV422/YUV420 |  |
| Description |  |
| Sets the encoder color-space format. |  |
| Parameter | Parameter Description |
| [enc] | Encoder ID (001-n = individual device) |
| YUV444/YUV422/YUV420 | Color-space format (some models support YUV420 only) |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration completed successfully. |
| [FAILED] | Configuration failed. |
| Example |  |
| SET ENC 001 CODECFMT YUV420 |  |

SET ENC [enc] CODECDELAY [dly]

| API Command |  |
| --- | --- |
| SET ENC [enc] CODECDELAY [dly] |  |
| Description |  |
| Sets the encoder encoding latency. |  |
| Parameter | Parameter Description |
| [enc] | Encoder ID (001-n = individual device) |
| [dly] | Latency: 1-500 ms |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration completed successfully. |
| [FAILED] | Configuration failed. |
| Example |  |
| SET ENC 001 CODECDELAY 100 |  |

SET ENC [enc] MAINCODE H265/H264

| API Command |  |
| --- | --- |
| SET ENC [enc] MAINCODE H265/H264 |  |
| Description |  |
| Sets the encoder main-stream codec. |  |
| Parameter | Parameter Description |
| [enc] | Encoder ID (001-n = individual device) |
| H265/H264 | Video codec selection |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration completed successfully. |
| [FAILED] | Configuration failed. |
| Example |  |
| SET ENC 001 MAINCODE H265 |  |

SET ENC [enc] MAINBRATYPE FIX/VAR

| API Command |  |
| --- | --- |
| SET ENC [enc] MAINBRATYPE FIX/VAR |  |
| Description |  |
| Sets the encoder main-stream bit-rate control mode. |  |
| Parameter | Parameter Description |
| [enc] | Encoder ID (001-n = individual device) |
| FIX/VAR | Bit-rate mode (constant/variable) |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration completed successfully. |
| [FAILED] | Configuration failed. |
| Example |  |
| SET ENC 001 MAINBRATYPE FIX |  |

SET ENC [enc] MAINIMAGE [imgqul]

| API Command |  |
| --- | --- |
| SET ENC [enc] MAINIMAGE [imgqul] |  |
| Description |  |
| Sets the encoder main-stream image quality. |  |
| Parameter | Parameter Description |
| [enc] | Encoder ID (001-n = individual device) |
| [imgqul] | Quality level: 1-100 (a higher value provides higher quality) |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration completed successfully. |
| [FAILED] | Configuration failed. |
| Example |  |
| SET ENC 001 MAINIMAGE 85 |  |

SET ENC [enc] MAINBRA [bitrate1]

| API Command |  |
| --- | --- |
| SET ENC [enc] MAINBRA [bitrate1] |  |
| Description |  |
| Sets the encoder main-stream bit rate. |  |
| Parameter | Parameter Description |
| [enc] | Encoder ID (001-n = individual device) |
| [bitrate1] | Bit rate: 1-102400 kbps |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration completed successfully. |
| [FAILED] | Configuration failed. |
| Example |  |
| SET ENC 001 MAINBRA 8000 |  |

SET ENC [enc] SUBCODE H265/H264

| API Command |  |
| --- | --- |
| SET ENC [enc] SUBCODE H265/H264 |  |
| Description |  |
| Sets the encoder sub-stream codec. |  |
| Parameter | Parameter Description |
| [enc] | Encoder ID (001-n = individual device) |
| H265/H264 | Video codec selection |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration completed successfully. |
| [FAILED] | Configuration failed. |
| Example |  |
| SET ENC 001 SUBCODE H264 |  |

SET ENC [enc] SUBBRATYPE FIX/VAR

| API Command |  |
| --- | --- |
| SET ENC [enc] SUBBRATYPE FIX/VAR |  |
| Description |  |
| Sets the encoder sub-stream bit-rate control mode. |  |
| Parameter | Parameter Description |
| [enc] | Encoder ID (001-n = individual device) |
| FIX/VAR | Bit-rate mode (constant/variable) |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration completed successfully. |
| [FAILED] | Configuration failed. |
| Example |  |
| SET ENC 001 SUBBRATYPE VAR |  |

SET ENC [enc] SUBIMAGE [imgqul]

| API Command |  |
| --- | --- |
| SET ENC [enc] SUBIMAGE [imgqul] |  |
| Description |  |
| Sets the encoder sub-stream image quality. |  |
| Parameter | Parameter Description |
| [enc] | Encoder ID (001-n = individual device) |
| [imgqul] | Quality level: 1-100 (a higher value provides higher quality) |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration completed successfully. |
| [FAILED] | Configuration failed. |
| Example |  |
| SET ENC 001 SUBIMAGE 70 |  |

SET ENC [enc] SUBBRA [bitrate2]

| API Command |  |
| --- | --- |
| SET ENC [enc] SUBBRA [bitrate2] |  |
| Description |  |
| Sets the encoder sub-stream bit rate. |  |
| Parameter | Parameter Description |
| [enc] | Encoder ID (001-n = individual device) |
| [bitrate2] | Bit rate: 1-40960 kbps |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration completed successfully. |
| [FAILED] | Configuration failed. |
| Example |  |
| SET ENC 001 SUBBRA 2000 |  |

SET ENC [enc] PREVIEWCODE H265/H264

| API Command |  |
| --- | --- |
| SET ENC [enc] PREVIEWCODE H265/H264 |  |
| Description |  |
| Sets the encoder preview-stream codec. |  |
| Parameter | Parameter Description |
| [enc] | Encoder ID (001-n = individual device) |
| H265/H264 | Video codec selection |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration completed successfully. |
| [FAILED] | Configuration failed. |
| Example |  |
| SET ENC 001 PREVIEWCODE H264 |  |

SET ENC [enc] PREVIEWBRATYPE FIX/VAR

| API Command |  |
| --- | --- |
| SET ENC [enc] PREVIEWBRATYPE FIX/VAR |  |
| Description |  |
| Sets the encoder preview-stream bit-rate control mode. |  |
| Parameter | Parameter Description |
| [enc] | Encoder ID (001-n = individual device) |
| FIX/VAR | Bit-rate mode (constant/variable) |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration completed successfully. |
| [FAILED] | Configuration failed. |
| Example |  |
| SET ENC 001 PREVIEWBRATYPE FIX |  |

SET ENC [enc] PREVIEWIMAGE [imgqul]

| API Command |  |
| --- | --- |
| SET ENC [enc] PREVIEWIMAGE [imgqul] |  |
| Description |  |
| Sets the encoder preview-stream image quality. |  |
| Parameter | Parameter Description |
| [enc] | Encoder ID (001-n = individual device) |
| [imgqul] | Quality level: 1-100 (a higher value provides higher quality) |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration completed successfully. |
| [FAILED] | Configuration failed. |
| Example |  |
| SET ENC 001 PREVIEWIMAGE 60 |  |

SET ENC [enc] PREVIEWBRA [bitrate3]

| API Command |  |
| --- | --- |
| SET ENC [enc] PREVIEWBRA [bitrate3] |  |
| Description |  |
| Sets the encoder preview-stream bit rate. |  |
| Parameter | Parameter Description |
| [enc] | Encoder ID (001-n = individual device) |
| [bitrate3] | Bit rate: 1-4096 kbps |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration completed successfully. |
| [FAILED] | Configuration failed. |
| Example |  |
| SET ENC 001 PREVIEWBRA 1000 |  |

SET ENC [enc] SDKSTREAM ON/OFF

| API Command |  |
| --- | --- |
| SET ENC [enc] SDKSTREAM ON/OFF |  |
| Description |  |
| Enables or disables the RTSP main stream. |  |
| Parameter | Parameter Description |
| [enc] | Encoder ID(001-n) |
| ON/OFF | Enable or disable the RTSP main stream |
| Return Value | Return Value Description |
| [SUCCESS] | Operation completed successfully. |
| [FAILED] | Operation failed. |
| Example |  |
| SET ENC 001 SDKSTREAM ON |  |

SET ENC [enc] SDKCODE H265/H264

| API Command |  |
| --- | --- |
| SET ENC [enc] SDKCODE H265/H264 |  |
| Description |  |
| Sets the RTSP main-stream codec. |  |
| Parameter | Parameter Description |
| [enc] | Encoder ID(001-n) |
| H265/H264 | Codec |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration completed successfully. |
| [FAILED] | Configuration failed. |
| Example |  |
| SET ENC 001 SDKCODE H265 |  |

SET ENC [enc] SDKBRATYPE FIX/VAR

| API Command |  |
| --- | --- |
| SET ENC [enc] SDKBRATYPE FIX/VAR |  |
| Description |  |
| Sets the RTSP main-stream bit-rate control mode. |  |
| Parameter | Parameter Description |
| [enc] | Encoder ID(001-n) |
| FIX/VAR | Constant/variable bit-rate mode |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration completed successfully. |
| [FAILED] | Configuration failed. |
| Example |  |
| SET ENC 001 SDKBRATYPE VAR |  |

SET ENC [enc] SDKIMAGE [imgqul]

| API Command |  |
| --- | --- |
| SET ENC [enc] SDKIMAGE [imgqul] |  |
| Description |  |
| Sets the RTSP main-stream image quality. |  |
| Parameter | Parameter Description |
| [enc] | Encoder ID(001-n) |
| [imgqul] | Quality level (1-100) |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration completed successfully. |
| [FAILED] | Configuration failed. |
| Example |  |
| SET ENC 001 SDKIMAGE 90 |  |

SET ENC [enc] SDKBRA [bitrate4]

| API Command |  |
| --- | --- |
| SET ENC [enc] SDKBRA [bitrate4] |  |
| Description |  |
| Sets the RTSP main-stream bit rate. |  |
| Parameter | Parameter Description |
| [enc] | Encoder ID(001-n) |
| [bitrate4] | Bit rate: 1-40960 kbps |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration completed successfully. |
| [FAILED] | Configuration failed. |
| Example |  |
| SET ENC 001 SDKBRA 8000 |  |

SET ENC [enc] SUBSDKSTREAM ON/OFF

| API Command |  |
| --- | --- |
| SET ENC [enc] SUBSDKSTREAM ON/OFF |  |
| Description |  |
| Enables or disables the RTSP sub-stream. |  |
| Parameter | Parameter Description |
| [enc] | Encoder ID(001-n) |
| ON/OFF | Enable or disable the RTSP sub-stream |
| Return Value | Return Value Description |
| [SUCCESS] | Operation completed successfully. |
| [FAILED] | Operation failed. |
| Example |  |
| SET ENC 001 SUBSDKSTREAM ON |  |

SET ENC [enc] SUBSDKCODE H265/H264

| API Command |  |
| --- | --- |
| SET ENC [enc] SUBSDKCODE H265/H264 |  |
| Description |  |
| Sets the RTSP sub-stream codec. |  |
| Parameter | Parameter Description |
| [enc] | Encoder ID(001-n) |
| H265/H264 | Codec |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration completed successfully. |
| [FAILED] | Configuration failed. |
| Example |  |
| SET ENC 001 SUBSDKCODE H264 |  |

SET ENC [enc] SUBSDKBRATYPE FIX/VAR

| API Command |  |
| --- | --- |
| SET ENC [enc] SUBSDKBRATYPE FIX/VAR |  |
| Description |  |
| Sets the RTSP sub-stream bit-rate control mode. |  |
| Parameter | Parameter Description |
| [enc] | Encoder ID(001-n) |
| FIX/VAR | Constant/variable bit-rate mode |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration completed successfully. |
| [FAILED] | Configuration failed. |
| Example |  |
| SET ENC 001 SUBSDKBRATYPE FIX |  |

SET ENC [enc] SUBSDKIMAGE [imgqul]

| API Command |  |
| --- | --- |
| SET ENC [enc] SUBSDKIMAGE [imgqul] |  |
| Description |  |
| Sets the RTSP sub-stream image quality. |  |
| Parameter | Parameter Description |
| [enc] | Encoder ID(001-n) |
| [imgqul] | Quality level (1-100) |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration completed successfully. |
| [FAILED] | Configuration failed. |
| Example |  |
| SET ENC 001 SUBSDKIMAGE 80 |  |

SET ENC [enc] SUBSDKBRA [bitrate5]

| API Command |  |
| --- | --- |
| SET ENC [enc] SUBSDKBRA [bitrate5] |  |
| Description |  |
| Sets the RTSP sub-stream bit rate. |  |
| Parameter | Parameter Description |
| [enc] | Encoder ID(001-n) |
| [bitrate5] | Bit rate: 1-20480 kbps |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration completed successfully. |
| [FAILED] | Configuration failed. |
| Example |  |
| SET ENC 001 SUBSDKBRA 2000 |  |

SET ENC [enc] KVMOS [os]

| API Command |  |
| --- | --- |
| SET ENC [enc] KVMOS [os] |  |
| Description |  |
| Sets the KVM operating-system type. |  |
| Parameter | Parameter Description |
| [enc] | Encoder ID(000 = all ports，001-n = individual port) |
| [os] | Operating-system type: LINUX - Linux WIN - Windows MAC - macOS TOUCH - Touch device |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration completed successfully. |
| [FAILED] | Configuration failed. |
| Example |  |
| SET ENC 001 KVMOS WIN |  |

4 Decoder Control API

SET DEC PRESET IPMODE [mode]

| API Command |  |
| --- | --- |
| SET DEC PRESET IPMODE [mode] |  |
| Description |  |
| Sets the preset decoder IP assignment mode. |  |
| Parameter | Parameter Description |
| [mode] | Options: DHCP or STATIC |
| Return Value | Return Value Description |
| [SUCCESS] | Preset configuration completed successfully. |
| [FAILED] |  |
| CONTROLLER> SET DEC PRESET IPMODE STATIC CONTROLLER> |  |

SET DEC PRESET START IP [ip]

| API Command |  |
| --- | --- |
| SET DEC PRESET START IP [ip] |  |
| Description |  |
| Sets the first IP address in the preset decoder address pool. |  |
| Parameter | Parameter Description |
| [ip] | Valid IPv4 address (e.g., 192.168.1.100) |
| Return Value | Return Value Description |
| [SUCCESS] | Preset configuration completed successfully. |
| [FAILED] | Failed to configure the preset. |
| Example |  |
| CONTROLLER> SET DEC PRESET START IP 192.168.50.15 [SUCCESS]Input preset start IP address set OK. CONTROLLER> |  |

SET DEC PRESET END IP [ip]

| API Command |  |
| --- | --- |
| SET DEC PRESET END IP [ip] |  |
| Description |  |
| Sets the last IP address in the preset decoder address pool. |  |
| Parameter | Parameter Description |
| [ip] | Valid IPv4 address (must be greater than the start IP address) |
| Return Value | Return Value Description |
| [SUCCESS] | Preset configuration completed successfully. |
| [FAILED] | Failed to configure the preset. |
| Example |  |
| CONTROLLER> SET DEC PRESET END IP 192.168.1.200 [SUCCESS]Input preset end IP address set OK. CONTROLLER> |  |

SET DEC PRESET GW [gw]

| API Command |  |
| --- | --- |
| SET DEC PRESET GW [gw] |  |
| Description |  |
| Sets the preset decoder gateway address. |  |
| Parameter | Parameter Description |
| [gw] | Valid IPv4 gateway address |
| Return Value | Return Value Description |
| [SUCCESS] | Preset configuration completed successfully. |
| [FAILED] | Failed to configure the preset. |
| Example |  |
| CONTROLLER> SET DEC PRESET GW 192.168.1.1 [SUCCESS]Decoder preset gateway set OK. CONTROLLER> |  |

SET DEC PRESET SM [mask]

| API Command |  |
| --- | --- |
| SET DEC PRESET SM [mask] |  |
| Description |  |
| Sets the preset decoder subnet mask. |  |
| Parameter | Parameter Description |
| [mask] | Valid subnet mask (e.g., 255.255.255.0) |
| Return Value | Return Value Description |
| [SUCCESS] | Preset configuration completed successfully. |
| [FAILED] | Failed to configure the preset. |
| Example |  |
| CONTROLLER> SET DEC PRESET SM 255.255.255.0 [SUCCESS]Decoder preset submask set OK. CONTROLLER> |  |

SET DEC PRESET APPLY

| API Command |  |
| --- | --- |
| SET DEC PRESET APPLY |  |
| Description |  |
| Applies the preset IP configuration. |  |
| Parameter | Parameter Description |
| None | None |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration applied successfully. |
| [FAILED] | Failed to apply the configuration. |
| Example |  |
| CONTROLLER> SET DEC PRESET APPLY [SUCCESS]Decoder preset apply OK. CONTROLLER> |  |

SET DEC [dec] STATIC IP [ip]

| API Command |  |
| --- | --- |
| SET DEC [dec] STATIC IP [ip] |  |
| Description |  |
| Sets the decoder static IP address. |  |
| Parameter | Parameter Description |
| [dec] | Decoder ID (000 = all; 001-n = individual device) |
| [ip] | Valid IPv4 address |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration completed successfully. |
| [FAILED] | Configuration failed. |
| Example |  |
| CONTROLLER> SET DEC 001 STATIC IP 169.254.10.50 [SUCCESS]Set decoder 1 static ip 169.254.10.50 CONTROLLER> |  |

SET DEC [dec] STATIC GATEWAY [gw]

| API Command |  |
| --- | --- |
| SET DEC [dec] STATIC GATEWAY [gw] |  |
| Description |  |
| Sets the decoder static gateway. |  |
| Parameter | Parameter Description |
| [dec] | Decoder ID (000 = all; 001-n = individual device) |
| [gw] | Valid IPv4 gateway address |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration completed successfully. |
| [FAILED] | Configuration failed. |
| Example |  |
| CONTROLLER> SET DEC 001 STATIC GATEWAY 192.168.1.1 [SUCCESS]Set decoder 1 static gateway 192.168.1.1 CONTROLLER> |  |

SET DEC [dec] STATIC MASK [mask]

| API Command |  |
| --- | --- |
| SET DEC [dec] STATIC MASK [mask] |  |
| Description |  |
| Sets the decoder static subnet mask. |  |
| Parameter | Parameter Description |
| [dec] | Decoder ID (000 = all; 001-n = individual device) |
| [mask] | Valid subnet mask |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration completed successfully. |
| [FAILED] | Configuration failed. |
| Example |  |
| CONTROLLER> SET DEC 001 STATIC MASK 255.255.0.0 [SUCCESS]Set decoder 1 static netmask 255.255.0.0 CONTROLLER> |  |

SET DEC [dec] NETWORK REBOOT

| API Command |  |
| --- | --- |
| SET DEC [dec] NETWORK REBOOT |  |
| Description |  |
| Restarts the decoder network interface. |  |
| Parameter | Parameter Description |
| [dec] | Decoder ID (000 = all; 001-n = individual device) |
| Return Value | Return Value Description |
| [SUCCESS] | Reboot completed successfully. |
| [FAILED] | Failed to reboot. |
| Example |  |
| CONTROLLER> SET DEC 001 NETWORK REBOOT [SUCCESS]Set dec 1 network reboot success. CONTROLLER> |  |

SET DEC [dec] ID [id]

| API Command |  |
| --- | --- |
| SET DEC [dec] ID [id] |  |
| Description |  |
| Changes the decoder ID. |  |
| Parameter | Parameter Description |
| [dec] | Decoder ID (001-n = individual device) |
| [id] | New ID (001-199) |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration completed successfully. |
| [FAILED] | Configuration failed. |
| Example |  |
| CONTROLLER> SET DEC 1 ID 3 [SUCCESS]Set decoder 001 ID to 003. CONTROLLER> |  |

SET DEC [dec] DELETE

| API Command |  |
| --- | --- |
| SET DEC [dec] DELETE |  |
| Description |  |
| Removes the decoder from the current system configuration. |  |
| Parameter | Parameter Description |
| [dec] | Decoder ID (001-n = individual device) |
| Return Value | Return Value Description |
| [SUCCESS] | Deleted successfully. |
| [FAILED] | Failed to delete. |
| Example |  |
| CONTROLLER> SET DEC 2 DELETE [SUCCESS]Delete decoder 002 done. CONTROLLER> |  |

SET DEC [dec] REBOOT

| API Command |  |
| --- | --- |
| SET DEC [dec] REBOOT |  |
| Description |  |
| Reboots the specified decoder. |  |
| Parameter | Parameter Description |
| [dec] | Decoder ID (000 = all; 001-n = individual device) |
| Return Value | Return Value Description |
| [SUCCESS] | Reboot completed successfully. |
| [FAILED] | Failed to reboot. |
| Example |  |
| CONTROLLER> SET DEC 2 REBOOT [SUCCESS]Reboot decoder 002 SUCCESS. CONTROLLER> |  |

SET DEC [dec] RESET

| API Command |  |
| --- | --- |
| SET DEC [dec] RESET |  |
| Description |  |
| Restores the decoder to factory defaults. |  |
| Parameter | Parameter Description |
| [dec] | Decoder ID (000 = all; 001-n = individual device) |
| Return Value | Return Value Description |
| [SUCCESS] | Reset completed successfully. |
| [FAILED] | Reset failed. |
| Example |  |
| CONTROLLER> SET DEC 3 RESET [SUCCESS]Set decoder 003 Factory Default Reset. CONTROLLER> |  |

SET DEC [dec] NAME name

| API Command |  |
| --- | --- |
| SET DEC [dec] NAME name |  |
| Description |  |
| Sets the decoder name. |  |
| Parameter | Parameter Description |
| [dec] | Decoder ID (000 = all; 001-n = individual device) |
| name | Name (maximum length: 16 characters) |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration completed successfully. |
| [FAILED] | Configuration failed. |
| Example |  |
| SET DEC 001 NAME Conferdece_Room |  |

SET DEC [dec] LED ON

| API Command |  |
| --- | --- |
| SET DEC [dec] LED ON |  |
| Description |  |
| Enables decoder identification LED flashing. |  |
| Parameter | Parameter Description |
| [dec] | Decoder ID (000 = all; 001-n = individual device) |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration completed successfully. |
| [FAILED] | Configuration failed. |
| Example |  |
| SET DEC 001 LED ON |  |

SET DEC [dec] LED OFF

| API Command |  |
| --- | --- |
| SET DEC [dec] LED OFF |  |
| Description |  |
| Disables decoder identification LED flashing (LED remains on). |  |
| Parameter | Parameter Description |
| [dec] | Decoder ID (000 = all; 001-n = individual device) |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration completed successfully. |
| [FAILED] | Configuration failed. |
| Example |  |
| SET DEC 001 LED OFF |  |

SET DEC [dec] TXMODE

| API Command |  |
| --- | --- |
| SET DEC [dec] TXMODE |  |
| Description |  |
| Switches the decoder to encoder mode. |  |
| Parameter | Parameter Description |
| [dec] | Decoder ID |
| Return Value | Return Value Description |
| [SUCCESS] | Mode switched successfully. |
| [FAILED] | Failed to switch the operating mode. |
| Example |  |
| SET DEC 001 TXMODE |  |

GET DEC [dec] STATUS

| API Command |  |
| --- | --- |
| GET DEC [dec] STATUS |  |
| Description |  |
| Returns decoder status information. |  |
| Parameter | Parameter Description |
| [dec] | Decoder ID (001-n = individual device) |
| Return Value | Return Value Description |
| [STATUS] | Returns detailed decoder status information. |
| Example |  |
| GET DEC 001 STATUS |  |

SET DEC [dec] GUEST [ON/OFF] [BR br] [BIT bit]

| API Command |  |
| --- | --- |
| SET DEC [dec] GUEST [ON/OFF] [BR br] [BIT bit] |  |
| Description |  |
| Configures decoder serial pass-through mode. |  |
| Parameter | Parameter Description |
| [dec] | Decoder ID (001-n = individual device) |
| [ON/OFF] | Enable or disable serial pass-through mode |
| [br] | Baud rate (0=2400, 1=4800, 2=9600, 3=19200, 4=38400, 5=57600, 6=115200) |
| [bit] | Data format (e.g., 8n1: 8 data bits, no parity, 1 stop bit) |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration completed successfully. |
| [FAILED] | Configuration failed. |
| Example |  |
| SET DEC 001 GUEST ON BR 6 BIT 8n1 |  |

SET DEC [dec] SENDGUEST ASCII [message]

| API Command |  |
| --- | --- |
| SET DEC [dec] SENDGUEST ASCII [message] |  |
| Description |  |
| Sends an ASCII serial message to the decoder. |  |
| Parameter | Parameter Description |
| [dec] | Decoder ID (001-n = individual device) |
| [message] | ASCII text message (maximum length: 200 characters) |
| Return Value | Return Value Description |
| [SUCCESS] | Sent successfully. |
| [FAILED] | Failed to send. |
| Example |  |
| SET DEC 001 SENDGUEST ASCII Hello World |  |

SET DEC [dec] SENDGUEST HEX [message]

| API Command |  |
| --- | --- |
| SET DEC [dec] SENDGUEST HEX [message] |  |
| Description |  |
| Sends a hexadecimal serial message to the decoder. |  |
| Parameter | Parameter Description |
| [dec] | Decoder ID (001-n = individual device) |
| [message] | Hexadecimal data (maximum length: 200 characters) |
| Return Value | Return Value Description |
| [SUCCESS] | Sent successfully. |
| [FAILED] | Failed to send. |
| Example |  |
| SET DEC 001 SENDGUEST HEX A1B2C3 |  |

SET DEC [dec] SWITCH [enc] ALL USER username

| API Command |  |
| --- | --- |
| SET DEC [dec] SWITCH [enc] ALL USER username |  |
| Description |  |
| Routes all signals, including video and audio, from the specified encoder to the decoder. |  |
| Parameter | Parameter Description |
| [dec] | Decoder ID(001-n) |
| [enc] | Source encoder ID (001-n) |
| username | Operator name (maximum length: 16 characters) |
| Return Value | Return Value Description |
| [SUCCESS] | Routing switched successfully. |
| [FAILED] | Failed to switch the route. |
| Example |  |
| SET DEC 001 SWITCH 002 ALL USER admin |  |

SET DEC [dec] SWITCH [enc] RS232

| API Command |  |
| --- | --- |
| SET DEC [dec] SWITCH [enc] RS232 |  |
| Description |  |
| Routes the decoder RS-232 signal from the specified encoder. |  |
| Parameter | Parameter Description |
| [dec] | Decoder ID(001-n) |
| [enc] | Source encoder ID (001-n) |
| Return Value | Return Value Description |
| [SUCCESS] | Routing switched successfully. |
| [FAILED] | Failed to switch the route. |
| Example |  |
| SET DEC 001 SWITCH 002 RS232 |  |

SET DEC [dec] OUTPUT OSD ON

| API Command |  |
| --- | --- |
| SET DEC [dec] OUTPUT OSD ON |  |
| Description |  |
| Enables the decoder on-screen OSD identifier. |  |
| Parameter | Parameter Description |
| [dec] | Decoder ID(001-n) |
| Return Value | Return Value Description |
| [SUCCESS] | Operation completed successfully. |
| [FAILED] | Operation failed. |
| Example |  |
| SET DEC 001 OUTPUT OSD ON |  |

SET DEC [dec] OUTPUT OSD OFF

| API Command |  |
| --- | --- |
| SET DEC [dec] OUTPUT OSD OFF |  |
| Description |  |
| Disables the decoder on-screen OSD identifier. |  |
| Parameter | Parameter Description |
| [dec] | Decoder ID(001-n) |
| Return Value | Return Value Description |
| [SUCCESS] | Operation completed successfully. |
| [FAILED] | Operation failed. |
| Example |  |
| SET DEC 001 OUTPUT OSD OFF |  |

SET DEC [dec] RESOLUTION [res]

| API Command |  |
| --- | --- |
| SET DEC [dec] RESOLUTION [res] |  |
| Description |  |
| Sets the decoder output resolution. |  |
| Parameter | Parameter Description |
| [dec] | Decoder ID(001-n) |
| [res] | Resolution format res=[1:1280x720@50Hz 2:1280x720@60Hz 3:1920x1080@24Hz] [4:1920x1080@25Hz 5:1920x1080@30Hz 6:1920x1080@50Hz] [7:1920x1080@60Hz 8:3840x2160@24Hz 9:3840x2160@25Hz] [10:3840x2160@30Hz 11:3840x2160@50Hz 12:3840x2160@60Hz] [13:4096x2160@24Hz 14:4096x2160@25Hz 15:4096x2160@30Hz] [16:4096x2160@50Hz 17:4096x2160@60Hz 18:1360x768@60Hz] [19:1680x1050@60Hz 20:1920x1200@60Hz 0:3840x2160@60Hz] |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration completed successfully. |
| [FAILED] | Configuration failed. |
| Example |  |
| SET DEC 001 RESOLUTION 6 |  |

SET DEC [dec] ROTATE [rtt]

| API Command |  |
| --- | --- |
| SET DEC [dec] ROTATE [rtt] |  |
| Description |  |
| Sets decoder output rotation. For rotation, the resolution must be below 3840x2160@30 Hz. |  |
| Parameter | Parameter Description |
| [dec] | Decoder ID(001-n) |
| [rtt] | Rotation angle: rtt=[0:0°, 1:90°, 2:180°, 3:270°] |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration completed successfully. |
| [FAILED] | Configuration failed. |
| Example |  |
| SET DEC 001 ROTATE 90 |  |

SET DEC [dec] DECVIDDELAY [decdly]

| API Command |  |
| --- | --- |
| SET DEC [dec] DECVIDDELAY [decdly] |  |
| Description |  |
| Sets the decoder video decoding latency. |  |
| Parameter | Parameter Description |
| [dec] | Decoder ID(001-n) |
| [decdly] | Latency in milliseconds; range: 10-5000 |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration completed successfully. |
| [FAILED] | Configuration failed. |
| Example |  |
| SET DEC 001 DECVIDDELAY 200 |  |

SET DEC [dec] OSDCOL BLACK/WHITE

| API Command |  |
| --- | --- |
| SET DEC [dec] OSDCOL BLACK/WHITE |  |
| Description |  |
| Sets the decoder OSD text color. |  |
| Parameter | Parameter Description |
| [dec] | Decoder ID(001-n) |
| BLACK/WHITE | Black/white text |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration completed successfully. |
| [FAILED] | Configuration failed. |
| Example |  |
| SET DEC 001 OSDCOL WHITE |  |

SET DEC [dec] OSDPOS LT/RT/LB/RB

| API Command |  |
| --- | --- |
| SET DEC [dec] OSDPOS LT/RT/LB/RB |  |
| Description |  |
| Sets the decoder OSD position. |  |
| Parameter | Parameter Description |
| [dec] | Decoder ID(001-n) |
| LT/RT/LB/RB | Top left/top right/bottom left/bottom right |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration completed successfully. |
| [FAILED] | Configuration failed. |
| Example |  |
| SET DEC 001 OSDPOS RT |  |

SET DEC [dec] TESTMODE [tst]

| API Command |  |
| --- | --- |
| SET DEC [dec] TESTMODE [tst] |  |
| Description |  |
| Sets the decoder test-pattern mode. |  |
| Parameter | Parameter Description |
| [dec] | Decoder ID(001-n) |
| [tst] | Test-pattern mode: OFF -> Disable test mode GRAY -> Grayscale test pattern COLOR -> Color-bar test pattern GEO -> Geometry-grid test pattern WHITE -> Solid white RED -> Solid red GREEN -> Solid green MESSAGE -> OSD message test mode |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration completed successfully. |
| [FAILED] | Configuration failed. |
| Example |  |
| SET DEC 001 TESTMODE 1 |  |

SET DEC [dec] SIGTYPE HDMI/DVI

| API Command |  |
| --- | --- |
| SET DEC [dec] SIGTYPE HDMI/DVI |  |
| Description |  |
| Sets the decoder output signal type. |  |
| Parameter | Parameter Description |
| [dec] | Decoder ID(001-n) |
| HDMI/DVI | HDMI or DVI signal format |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration completed successfully. |
| [FAILED] | Configuration failed. |
| Example |  |
| SET DEC 001 SIGTYPE HDMI |  |

SET DEC [dec] AUOUTVOL [vol]

| API Command |  |
| --- | --- |
| SET DEC [dec] AUOUTVOL [vol] |  |
| Description |  |
| Sets the decoder audio output volume. |  |
| Parameter | Parameter Description |
| [dec] | Decoder ID(001-n) |
| [vol] | Volume: 0-100 |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration completed successfully. |
| [FAILED] | Configuration failed. |
| Example |  |
| SET DEC 001 AUOUTVOL 80 |  |

SET DEC [dec] KVMMOUSE ON/OFF

| API Command |  |
| --- | --- |
| SET DEC [dec] KVMMOUSE ON/OFF |  |
| Description |  |
| Shows or hides the KVM mouse pointer. |  |
| Parameter | Parameter Description |
| [dec] | Decoder ID(001-n) |
| ON/OFF | Show/hide the mouse pointer |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration completed successfully. |
| [FAILED] | Configuration failed. |
| Example |  |
| SET DEC 001 KVMMOUSE ON |  |

SET DEC [dec] KVMAROUND ON/OFF

| API Command |  |
| --- | --- |
| SET DEC [dec] KVMAROUND ON/OFF |  |
| Description |  |
| Sets KVM radial-menu activation. |  |
| Parameter | Parameter Description |
| [dec] | Decoder ID(001-n) |
| ON/OFF | Enable/disable radial-menu activation by circling the mouse |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration completed successfully. |
| [FAILED] | Configuration failed. |
| Example |  |
| SET DEC 001 KVMAROUND ON |  |

5 Video Wall Control API

CREATE WALL HANDLE [hdl]

| API Command |  |
| --- | --- |
| CREATE WALL HANDLE [hdl] |  |
| Description |  |
| Creates a new video wall. |  |
| Parameter | Parameter Description |
| [hdl] | Video wall ID (01-255) |
| Return Value | Return Value Description |
| [SUCCESS] | Created successfully. |
| [FAILED] | The handle is already in use or invalid. |
| Example |  |
| CREATE WALL HANDLE 01 |  |

DELETE WALL HANDLE [hdl]

| API Command |  |
| --- | --- |
| DELETE WALL HANDLE [hdl] |  |
| Description |  |
| Deletes the specified video wall. |  |
| Parameter | Parameter Description |
| [hdl] | Existing video wall ID |
| Return Value | Return Value Description |
| [SUCCESS] | Deleted successfully. |
| [FAILED] | The specified handle does not exist. |
| Example |  |
| DELETE WALL HANDLE 01 |  |

APPLY WALL [hdl] PRESET [pre]

| API Command |  |
| --- | --- |
| APPLY WALL [hdl] PRESET [pre] |  |
| Description |  |
| Applies a video wall preset. |  |
| Parameter | Parameter Description |
| [hdl] | Target video wall ID |
| [pre] | Preset ID (01-09) |
| Return Value | Return Value Description |
| [SUCCESS] | Preset applied successfully. |
| [FAILED] | Invalid preset ID. |
| Example |  |
| APPLY WALL 01 PRESET 02 |  |

SET WALL [hdl] C [c] R [r] RES [res]

| API Command |  |
| --- | --- |
| SET WALL [hdl] C [c] R [r] RES [res] |  |
| Description |  |
| Sets the video wall matrix dimensions and resolution. |  |
| Parameter | Parameter Description |
| [hdl] | Target video wall ID |
| [c] | Number of columns (01-50) |
| [r] | Number of rows (01-50) |
| [res] | Standard resolution code, identical to the decoder resolution codes （res = 99 = custom resolution） |
| Return Value | Return Value Description |
| [SUCCESS] | Parameters configured successfully. |
| [FAILED] | Value exceeds the allowed range. |
| Example |  |
| SET WALL 01 C 4 R 3 RES 99 |  |

SET WALL [hdl] CRES TOTAL [tx] [ty] [tref] DEV [dx] [dy] [dref]

| API Command |  |
| --- | --- |
| SET WALL [hdl] CRES TOTAL [tx] [ty] [tref] DEV [dx] [dy] [dref] |  |
| Description |  |
| Sets custom resolution parameters. |  |
| Parameter | Parameter Description |
| [tx] | Total horizontal pixels |
| [ty] | Total vertical pixels |
| [tref] | Total refresh rate (Hz) |
| [dx] | Horizontal pixels per device |
| [dy] | Vertical pixels per device |
| [dref] | Device refresh rate (Hz) |
| Return Value | Return Value Description |
| [SUCCESS] | Custom resolution applied successfully. |
| [FAILED] | Parameter mismatch. |
| Example |  |
| SET WALL 01 CRES TOTAL 7680 4320 60 DEV 3840 2160 60 |  |

SET WALL [hdl] CUT HOR LEFT/RIGHT [ch] VER TOP/BOTTOM [cv]

| API Command |  |
| --- | --- |
| SET WALL [hdl] CUT HOR LEFT/RIGHT [ch] VER TOP/BOTTOM [cv] |  |
| Description |  |
| Sets video wall cropping parameters. |  |
| Parameter | Parameter Description |
| LEFT/RIGHT | Horizontal crop direction |
| [ch] | Horizontal crop value in pixels |
| TOP/BOTTOM | Vertical crop direction |
| [cv] | Vertical crop value in pixels |
| Return Value | Return Value Description |
| [SUCCESS] | Cropping configured successfully. |
| [FAILED] | Crop value is out of range. |
| Example |  |
| SET WALL 01 CUT HOR LEFT 50 VER TOP 30 |  |

SET WALL [hdl] BEZEL [bu] [bd] [bl] [br]

| API Command |  |
| --- | --- |
| SET WALL [hdl] BEZEL [bu] [bd] [bl] [br] |  |
| Description |  |
| Sets bezel compensation parameters. |  |
| Parameter | Parameter Description |
| [bu] | Top bezel compensation (0-200 pixels) |
| [bd] | Bottom bezel compensation (0-200 pixels) |
| [bl] | Left bezel compensation (0-300 pixels) |
| [br] | Right bezel compensation (0-300 pixels) |
| Return Value | Return Value Description |
| [SUCCESS] | Parameters saved successfully. |
| [FAILED] | Compensation value is out of range. |
| Example |  |
| SET WALL 01 BEZEL 5 5 8 8 |  |

SET WALL [hdl] ACTBEZEL [bu] [bd] [bl] [br]

| API Command |  |
| --- | --- |
| SET WALL [hdl] ACTBEZEL [bu] [bd] [bl] [br] |  |
| Description |  |
| Immediately applies the bezel compensation parameters. |  |
| Parameter | Parameter Description |
| [bu] | Top bezel compensation (0-200 pixels) |
| [bd] | Bottom bezel compensation (0-200 pixels) |
| [bl] | Left bezel compensation (0-300 pixels) |
| [br] | Right bezel compensation (0-300 pixels) |
| Return Value | Return Value Description |
| [SUCCESS] | Bezel compensation applied successfully. |
| [FAILED] | Device is not ready. |
| Example |  |
| SET WALL 01 ACTBEZEL 5 5 8 8 |  |

SET WALL [hdl] ROTATEMODE [rttmode]

| API Command |  |
| --- | --- |
| SET WALL [hdl] ROTATEMODE [rttmode] |  |
| Description |  |
| Sets the video wall rotation mode. Rotation is supported below 3840x2160@30 Hz. |  |
| Parameter | Parameter Description |
| [rttmode] | 0 = normal mode; 2 = 0°/180°; 3 = 90°/270° |
| Return Value | Return Value Description |
| [SUCCESS] | Mode configured successfully. |
| [FAILED] | Unsupported mode. |
| Example |  |
| SET WALL 01 ROTATEMODE 2 |  |

SET WALL [hdl] H [posx] V [posy] ROTATE [rtt]

| API Command |  |
| --- | --- |
| SET WALL [hdl] H [posx] V [posy] ROTATE [rtt] |  |
| Description |  |
| Sets the rotation angle for a video wall decoder. |  |
| Parameter | Parameter Description |
| [hdl] | Video wall handle (01-255) |
| [posx] | Horizontal start position (pixels) |
| [posy] | Vertical start position (pixels) |
| [rtt] | Rotation angle: [0:0°, 1:90°, 2:180°, 3:270°] |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration completed successfully. |
| [FAILED] | Parameter is out of range. |
| Example |  |
| SET WALL 01 H 0 V 0 ROTATE 1 |  |

SET WALL [hdl] NAME [name]

| API Command |  |
| --- | --- |
| SET WALL [hdl] NAME [name] |  |
| Description |  |
| Sets the video wall display name. |  |
| Parameter | Parameter Description |
| [hdl] | Video wall handle (01-255) |
| [name] | Name (maximum length: 16 characters) |
| Return Value | Return Value Description |
| [SUCCESS] | Name configured successfully. |
| [FAILED] | Invalid name. |
| Example |  |
| SET WALL 01 NAME MAIN_WALL |  |

SET WALL [hdl] DEC [dec] H [h] V [h]

| API Command |  |
| --- | --- |
| SET WALL [hdl] DEC [dec] H [h] V [v] |  |
| Description |  |
| Assigns a decoder to a video wall grid position. |  |
| Parameter | Parameter Description |
| [hdl] | Video wall handle |
| [dec] | Decoder ID(001-n) |
| [h] | Horizontal grid position (01-50) |
| [v] | Vertical grid position (01-50) |
| Return Value | Return Value Description |
| [SUCCESS] | Assigned successfully. |
| [FAILED] | Position conflict. |
| Example |  |
| SET WALL 01 DEC 001 H 2 V 3 |  |

SET WALL [hdl] PRESET [pre] WND [wnd] CREATE [posx] [posy] [width] [height] NAME window1

| API Command |  |
| --- | --- |
| SET WALL [hdl] PRESET [pre] WND [wnd] CREATE [posx] [posy] [width] [height] NAME window1 |  |
| Description |  |
| Creates a new window in a video wall preset. |  |
| Parameter | Parameter Description |
| [hdl] | Video wall handle |
| [pre] | Preset ID (01-09) |
| [wnd] | Window ID (01-36) |
| [posx] | X coordinate of the window's top-left corner |
| [posy] | Y coordinate of the window's top-left corner |
| [width] | Window width (pixels) |
| [height] | Window height (pixels) |
| window1 | Window display name |
| Return Value | Return Value Description |
| [SUCCESS] | Created successfully. |
| [FAILED] | Failed to create. |
| Example |  |
| SET WALL 01 PRESET 01 WND 01 CREATE 100 200 800 600 NAME WIN1 |  |

SET WALL [hdl] PRESET [pre] WND [wnd] RESIZE [posx] [posy] [width] [height]

| API Command |  |
| --- | --- |
| SET WALL [hdl] PRESET [pre] WND [wnd] RESIZE [posx] [posy] [width] [height] |  |
| Description |  |
| Resizes and repositions a window. |  |
| Parameter | Parameter Description |
| [hdl] | Video wall handle |
| [pre] | Preset ID (01-09) |
| [wnd] | Window ID (01-36) |
| [posx] | X coordinate of the window's top-left corner |
| [posy] | Y coordinate of the window's top-left corner |
| [width] | Window width (pixels) |
| [height] | Window height (pixels) |
| Return Value | Return Value Description |
| [SUCCESS] | Window adjusted successfully. |
| [FAILED] | Failed to adjust the window. |
| Example |  |
| SET WALL 01 PRESET 01 WND 01 RESIZE 150 250 850 650 |  |

SET WALL [hdl] PRESET [pre] WND [wnd] SOURCE [enc]

| API Command |  |
| --- | --- |
| SET WALL [hdl] PRESET [pre] WND [wnd] SOURCE [enc] |  |
| Description |  |
| Routes a source to the window. |  |
| Parameter | Parameter Description |
| [hdl] | Video wall handle |
| [pre] | Preset ID (01-09) |
| [wnd] | Window ID (01-36) |
| [enc] | Encoder ID(001-n) |
| Return Value | Return Value Description |
| [SUCCESS] | Routing switched successfully. |
| [FAILED] | Encoder is offline. |
| Example |  |
| SET WALL 01 PRESET 01 WND 01 SOURCE 003 |  |

SET WALL [hdl] PRESET [pre] WND [wnd] TOP/BOTTOM

| API Command |  |
| --- | --- |
| SET WALL [hdl] PRESET [pre] WND [wnd] TOP/BOTTOM |  |
| Description |  |
| Sets the window stacking order. |  |
| Parameter | Parameter Description |
| [hdl] | Video wall handle |
| [pre] | Preset ID (01-09) |
| [wnd] | Window ID (01-36) |
| TOP/BOTTOM | TOP: bring to front; BOTTOM: send to back |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration completed successfully. |
| [FAILED] | Window does not exist. |
| Example |  |
| SET WALL 01 PRESET 01 WND 01 TOP |  |

SET WALL [hdl] PRESET [pre] WND [wnd] DELETE

| API Command |  |
| --- | --- |
| SET WALL [hdl] PRESET [pre] WND [wnd] DELETE |  |
| Description |  |
| Deletes the specified window. |  |
| Parameter | Parameter Description |
| [hdl] | Video wall handle |
| [pre] | Preset ID (01-09) |
| [wnd] | Window ID (01-36) |
| Return Value | Return Value Description |
| [SUCCESS] | Deleted successfully. |
| [FAILED] | Failed to delete. |
| Example |  |
| SET WALL 01 PRESET 01 WND 01 DELETE |  |

6 Marquee Control API

CREATE MARQUEE HANDLE [maqidx]

| API Command |  |
| --- | --- |
| CREATE MARQUEE HANDLE [maqidx] |  |
| Description |  |
| Creates a new marquee. |  |
| Parameter | Parameter Description |
| [maqidx] | Marquee ID (01-256) |
| Return Value | Return Value Description |
| [SUCCESS] | Created successfully. |
| [FAILED] | The ID is already in use. |
| Example |  |
| CREATE MARQUEE HANDLE 01 |  |

SET MARQUEE [maqidx] NAME [name]

| API Command |  |
| --- | --- |
| SET MARQUEE [maqidx] NAME [name] |  |
| Description |  |
| Sets the marquee display name. |  |
| Parameter | Parameter Description |
| [maqidx] | Marquee ID (01-256) |
| [name] | Name (maximum length: 16 characters) |
| Return Value | Return Value Description |
| [SUCCESS] | Name configured successfully. |
| [FAILED] | Invalid name. |
| Example |  |
| SET MARQUEE 01 NAME maq001 |  |

SET MARQUEE [maqidx] SIZE [w] [h]

| API Command |  |
| --- | --- |
| SET MARQUEE [maqidx] SIZE [w] [h] |  |
| Description |  |
| Sets the marquee canvas dimensions. |  |
| Parameter | Parameter Description |
| [w] | Canvas width (pixels) |
| [h] | Canvas height (pixels) |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration completed successfully. |
| [FAILED] | Dimensions exceed the allowed range. |
| Example |  |
| SET MARQUEE 01 SIZE 800 100 |  |

SET MARQUEE [maqidx] CONTENT [text]

| API Command |  |
| --- | --- |
| SET MARQUEE [maqidx] CONTENT [text] |  |
| Description |  |
| Sets the marquee text. |  |
| Parameter | Parameter Description |
| [maqidx] | Marquee ID (01-256) |
| [text] | Display text (maximum length: 150 characters) |
| Return Value | Return Value Description |
| [SUCCESS] | Content configured successfully. |
| [FAILED] | Content is empty. |
| Example |  |
| SET MARQUEE 01 CONTENT Welcome |  |

SET MARQUEE [maqidx] FNAME [fname]

| API Command |  |
| --- | --- |
| SET MARQUEE [maqidx] FNAME [fname] |  |
| Description |  |
| Sets the marquee font. |  |
| Parameter | Parameter Description |
| [maqidx] | Marquee ID (01-256) |
| [fname] | Font name (maximum length: 16 characters) |
| Return Value | Return Value Description |
| [SUCCESS] | Font configured successfully. |
| [FAILED] | Font is unavailable. |
| Example |  |
| SET MARQUEE 01 FNAME Arial |  |

SET MARQUEE [maqidx] FSIZE [fsize]

| API Command |  |
| --- | --- |
| SET MARQUEE [maqidx] FSIZE [fsize] |  |
| Description |  |
| Sets the marquee font size. |  |
| Parameter | Parameter Description |
| [maqidx] | Marquee ID (01-256) |
| [fsize] | Font size (1-5000) |
| Return Value | Return Value Description |
| [SUCCESS] | Font size configured successfully. |
| [FAILED] | Font size is out of range. |
| Example |  |
| SET MARQUEE 01 FSIZE 24 |  |

SET MARQUEE [maqidx] FCOLOR [fcolor]

| API Command |  |
| --- | --- |
| SET MARQUEE [maqidx] FCOLOR [fcolor] |  |
| Description |  |
| Sets the marquee font color. |  |
| Parameter | Parameter Description |
| [maqidx] | Marquee ID (01-256) |
| [fcolor] | Font color value (00000-FFFFF) |
| Return Value | Return Value Description |
| [SUCCESS] | Color configured successfully. |
| [FAILED] | Invalid color value. |
| Example |  |
| SET MARQUEE 01 FCOLOR FF0000 |  |

SET MARQUEE [maqidx] BCOLOR [bcolor]

| API Command |  |
| --- | --- |
| SET MARQUEE [maqidx] BCOLOR [bcolor] |  |
| Description |  |
| Sets the marquee background color. |  |
| Parameter | Parameter Description |
| [maqidx] | Marquee ID (01-256) |
| [bcolor] | Background RGB value (000000-FFFFFF) |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration completed successfully. |
| [FAILED] | Invalid parameter. |
| Example |  |
| SET MARQUEE 01 BCOLOR 0000FF |  |

SET MARQUEE [maqidx] DIRECT [dir]

| API Command |  |
| --- | --- |
| SET MARQUEE [maqidx] DIRECT [dir] |  |
| Description |  |
| Sets the marquee scroll direction. |  |
| Parameter | Parameter Description |
| [dir] | Direction: LEFT/RIGHT/UP/DOWN |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration completed successfully. |
| [FAILED] | Invalid direction. |
| Example | Example |
| SET MARQUEE 01 DIRECT LEFT |  |

SET MARQUEE [maqidx] SPEED [speed]

| API Command |  |
| --- | --- |
| SET MARQUEE [maqidx] SPEED [speed] |  |
| Description |  |
| Sets the marquee scroll speed. |  |
| Parameter | Parameter Description |
| [maqidx] | Marquee ID (01-256) |
| [speed] | Speed (1-10) |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration completed successfully. |
| [FAILED] | Speed is out of range. |
| Example |  |
| SET MARQUEE 01 SPEED 5 |  |

SET MARQUEE [maqidx] APPLY

| API Command |  |
| --- | --- |
| SET MARQUEE [maqidx] APPLY |  |
| Description |  |
| Applies all marquee settings. |  |
| Parameter | Parameter Description |
| [maqidx] | Marquee ID (01-256) |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration applied successfully. |
| [FAILED] | One or more settings are invalid. |
| Example |  |
| SET MARQUEE 01 APPLY |  |

SET MARQUEE [maqidx] DELETE

| API Command |  |
| --- | --- |
| SET MARQUEE [maqidx] DELETE |  |
| Description |  |
| Deletes the marquee. |  |
| Parameter | Parameter Description |
| [maqidx] | Marquee ID (01-256) |
| Return Value | Return Value Description |
| [SUCCESS] | Deleted successfully. |
| [FAILED] | The marquee does not exist. |
| Example |  |
| SET MARQUEE 01 DELETE |  |

SET WALL [hdl] PRESET [pre] MARQUEE [maqidx] ON [posx] [posy]

| API Command |  |
| --- | --- |
| SET WALL [hdl] PRESET [pre] MARQUEE [maqidx] ON [posx] [posy] |  |
| Description |  |
| Enables a marquee on the video wall. |  |
| Parameter | Parameter Description |
| [hdl] | Video wall ID (01-255) |
| [pre] | Preset ID (01-09) |
| [maqidx] | Marquee ID (01-256) |
| [posx] | Horizontal coordinate (pixels) |
| [posy] | Vertical coordinate (pixels) |
| Return Value | Return Value Description |
| [SUCCESS] | Enabled successfully. |
| [FAILED] | Coordinates are out of range. |
| Example |  |
| SET WALL 01 PRESET 01 MARQUEE 01 ON 100 200 |  |

SET WALL [hdl] PRESET [pre] MARQUEE [maqidx] OFF

| API Command |  |
| --- | --- |
| SET WALL [hdl] PRESET [pre] MARQUEE [maqidx] OFF |  |
| Description |  |
| Disables a marquee on the video wall. |  |
| Parameter | Parameter Description |
| [hdl] | Video wall ID (01-255) |
| [pre] | Preset ID (01-09) |
| [maqidx] | Marquee ID (01-256) |
| Return Value | Return Value Description |
| [SUCCESS] | Disabled successfully. |
| [FAILED] | Execution failed. |
| Example |  |
| SET WALL 01 PRESET 01 MARQUEE 01 OFF |  |

7 Matrix Preset Control API

CREATE MATRIX PRESET [prs]

| API Command |  |
| --- | --- |
| CREATE MATRIX PRESET [prs] |  |
| Description |  |
| Creates a matrix routing preset. |  |
| Parameter | Parameter Description |
| [prs] | Preset ID (01-30) |
| Return Value | Return Value Description |
| [SUCCESS] | Created successfully. |
| [FAILED] | The preset ID already exists. |
| Example |  |
| CREATE MATRIX PRESET 01 |  |

DELETE MATRIX PRESET [prs]

| API Command |  |
| --- | --- |
| DELETE MATRIX PRESET [prs] |  |
| Description |  |
| Deletes a matrix routing preset. |  |
| Parameter | Parameter Description |
| [prs] | Preset ID (01-30) |
| Return Value | Return Value Description |
| [SUCCESS] | Deleted successfully. |
| [FAILED] | The preset ID does not exist. |
| Example |  |
| DELETE MATRIX PRESET 01 |  |

SET MATRIX PRESET [prs] NAME [name]

| API Command |  |
| --- | --- |
| SET MATRIX PRESET [prs] NAME [name] |  |
| Description |  |
| Sets the matrix preset name. |  |
| Parameter | Parameter Description |
| [prs] | Preset ID (01-30) |
| [name] | Name (maximum length: 16 characters) |
| Return Value | Return Value Description |
| [SUCCESS] | Name configured successfully. |
| [FAILED] | The name is too long or contains invalid characters. |
| Example |  |
| SET MATRIX PRESET 01 NAME Main_Hall_Matrix |  |

ADD MATRIX PRESET [prs] DEC [dec] SOURCE [enc]

| API Command |  |
| --- | --- |
| ADD MATRIX PRESET [prs] DEC [dec] SOURCE [enc] |  |
| Description |  |
| Adds a decoder-to-encoder routing entry to a matrix preset. |  |
| Parameter | Parameter Description |
| [prs] | Preset ID (01-30) |
| [dec] | Decoder ID(001-n) |
| [enc] | Encoder ID(001-n) |
| Return Value | Return Value Description |
| [SUCCESS] | Added successfully. |
| [FAILED] | The device does not exist or conflicts with another entry. |
| Example |  |
| ADD MATRIX PRESET 01 DEC 001 SOURCE 002 |  |

DELETE MATRIX PRESET [prs] DEC [dec]

| API Command |  |
| --- | --- |
| DELETE MATRIX PRESET [prs] DEC [dec] |  |
| Description |  |
| Removes a decoder entry from a matrix preset. |  |
| Parameter | Parameter Description |
| [prs] | Preset ID (01-30) |
| [dec] | Decoder ID(001-n) |
| Return Value | Return Value Description |
| [SUCCESS] | Removed successfully. |
| [FAILED] | The decoder is not included in the preset. |
| Example |  |
| DELETE MATRIX PRESET 01 DEC 001 |  |

APPLY MATRIX PRESET [prs]

| API Command |  |
| --- | --- |
| APPLY MATRIX PRESET [prs] |  |
| Description |  |
| Applies a matrix routing preset. |  |
| Parameter | Parameter Description |
| [prs] | Preset ID (01-30) |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration applied successfully. |
| [FAILED] | The preset does not exist or contains an invalid configuration. |
| Example |  |
| APPLY MATRIX PRESET 01 |  |

8 Device Management API

SEARCH

| API Command |  |
| --- | --- |
| SEARCH |  |
| Description |  |
| Searches the network for all encoder and decoder devices. |  |
| Parameter | Parameter Description |
| None |  |
| Return Value | Return Value Description |
| [SUCCESS] | Device discovery started successfully. |
| [FAILED] | Device discovery is already in progress. |
| Example |  |
| CONTROLLER> SEARCH [SUCCESS]More device in network will take more time to finish scan, please wait... done. ================================================================ Scan Device Result Info ==New Input No. Index IP MAC Assign 001 001 192.168.000.200 6c-df-fb-2b-01-19 No ==New Output No. Index IP MAC Assign 001 001 192.168.000.200 6c-df-fb-2b-02-12 No ================================================================ |  |

SEARCH STATUS

| API Command |  |
| --- | --- |
| SEARCH STATUS |  |
| Description |  |
| Displays all discovered encoder and decoder devices. |  |
| Parameter | Parameter Description |
| None |  |
| Return Value | Return Value Description |
| [SUCCESS] | Device discovery started successfully. |
| [FAILED] | Device discovery is already in progress. |
| Example |  |
| CONTROLLER> SEARCH STATUS ================================================================ Scan Device Result Info ==New Input No. Index IP MAC Assign 001 001 192.168.000.200 6c-df-fb-2b-01-19 No ==New Output No. Index IP MAC Assign 001 001 192.168.000.200 6c-df-fb-2b-02-12 No ================================================================ |  |

SEARCH RESET

| API Command |  |
| --- | --- |
| SEARCH RESET |  |
| Description |  |
| Clears the network discovery results. |  |
| Parameter | Parameter Description |
| None |  |
| Return Value | Return Value Description |
| [SUCCESS] | Discovery results cleared successfully. |
| Example |  |
| SEARCH RESET |  |

ADD DEV RESET

| API Command |  |
| --- | --- |
| ADD DEV RESET |  |
| Description |  |
| Clears all device-related configurations. |  |
| Parameter | Parameter Description |
| None |  |
| Return Value | Return Value Description |
| [SUCCESS] | Clears the following: - All encoder configurations - All decoder configurations - Video wall configurations - Network discovery results |
| Example |  |
| ADD DEV RESET |  |

ADD DEV AUTO

| API Command |  |
| --- | --- |
| ADD DEV AUTO |  |
| Description |  |
| Automatically adds newly discovered encoder and decoder devices. |  |
| Parameter | Parameter Description |
| None |  |
| Return Value | Return Value Description |
| [SUCCESS] | New devices added automatically. |
| Example |  |
| ADD DEV AUTO |  |

9 Network Management API

SET NETWORK LAN2 DHCP ON/OFF

| API Command |  |
| --- | --- |
| SET NETWORK LAN2 DHCP ON/OFF |  |
| Description |  |
| Enables or disables DHCP on the CONTROL LAN port. |  |
| Parameter | Parameter Description |
| ON/OFF | DHCP status: ON = enabled; OFF = disabled |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration completed successfully. |
| [FAILED] | Invalid parameter. |
| Example |  |
| SET NETWORK LAN2 DHCP ON |  |

SET NETWORK LAN2 STATIC IP [ip]

| API Command |  |
| --- | --- |
| SET NETWORK LAN2 STATIC IP [ip] |  |
| Description |  |
| Sets the static IP address of the CONTROL LAN port. |  |
| Parameter | Parameter Description |
| [ip] | IPv4 address (e.g., 192.168.1.100) |
| Return Value | Return Value Description |
| [SUCCESS] | IP address configured successfully. |
| [FAILED] | Invalid IP address format. |
| Example |  |
| SET NETWORK LAN2 STATIC IP 192.168.1.100 |  |

SET NETWORK LAN2 STATIC GATEWAY [gw]

| API Command |  |
| --- | --- |
| SET NETWORK LAN2 STATIC GATEWAY [gw] |  |
| Description |  |
| Sets the static gateway of the CONTROL LAN port. |  |
| Parameter | Parameter Description |
| [gw] | IPv4 gateway address (e.g., 192.168.1.1) |
| Return Value | Return Value Description |
| [SUCCESS] | Gateway configured successfully. |
| [FAILED] | Invalid gateway address format. |
| Example |  |
| SET NETWORK LAN2 STATIC GATEWAY 192.168.1.1 |  |

SET NETWORK LAN2 STATIC MASK [mask]

| API Command |  |
| --- | --- |
| SET NETWORK LAN2 STATIC MASK [mask] |  |
| Description |  |
| Sets the static subnet mask of the CONTROL LAN port. |  |
| Parameter | Parameter Description |
| [mask] | Subnet mask (e.g., 255.255.255.0) |
| Return Value | Return Value Description |
| [SUCCESS] | Subnet mask configured successfully. |
| [FAILED] | Invalid subnet mask format. |
| Example |  |
| SET NETWORK LAN2 STATIC MASK 255.255.255.0 |  |

SET NETWORK REBOOT

| API Command |  |
| --- | --- |
| SET NETWORK REBOOT |  |
| Description |  |
| Restarts the network service to apply the configuration. |  |
| Parameter | Parameter Description |
| None |  |
| Return Value | Return Value Description |
| [SUCCESS] | Network service restarted successfully. |
| Example |  |
| SET NETWORK REBOOT |  |

SET NETWORK SSH ON/OFF

| API Command |  |
| --- | --- |
| SET NETWORK SSH ON/OFF |  |
| Description |  |
| Enables or disables the SSH service. |  |
| Parameter | Parameter Description |
| ON/OFF | ON = enable SSH; OFF = disable SSH |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration completed successfully. |
| [FAILED] | Invalid parameter or service unavailable. |
| Example |  |
| SET NETWORK SSH ON |  |

SET NETWORK SSH PORT [port]

| API Command |  |
| --- | --- |
| SET NETWORK SSH PORT [port] |  |
| Description |  |
| Sets the SSH service port. |  |
| Parameter | Parameter Description |
| [port] | Port number (1-65535) |
| Return Value | Return Value Description |
| [SUCCESS] | Port changed successfully. |
| [FAILED] | Port conflict or invalid port. |
| Example |  |
| SET NETWORK SSH PORT 2222 |  |

SET NETWORK TELNET ON/OFF

| API Command |  |
| --- | --- |
| SET NETWORK TELNET ON/OFF |  |
| Description |  |
| Enables or disables the Telnet service. |  |
| Parameter | Parameter Description |
| ON/OFF | ON = enable Telnet; OFF = disable Telnet |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration completed successfully. |
| [FAILED] | Invalid parameter. |
| Example |  |
| SET NETWORK TELNET OFF |  |

SET NETWORK TELNET PORT port

| API Command |  |
| --- | --- |
| SET NETWORK TELNET PORT [port] |  |
| Description |  |
| Sets the Telnet service port. |  |
| Parameter | Parameter Description |
| [port] | Port number (1-65535) |
| Return Value | Return Value Description |
| [SUCCESS] | Port changed successfully. |
| [FAILED] | Port conflict or invalid port. |
| Example |  |
| SET NETWORK TELNET PORT 2323 |  |

SET NETWORK WEB ON/OFF

| API Command |  |
| --- | --- |
| SET NETWORK WEB ON/OFF |  |
| Description |  |
| Enables or disables access to the web GUI. |  |
| Parameter | Parameter Description |
| ON/OFF | ON = enable web access; OFF = disable web access |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration completed successfully. |
| [FAILED] | Service unavailable. |
| Example |  |
| SET NETWORK WEB ON |  |

SET NETWORK HTTPS ON/OFF

| API Command |  |
| --- | --- |
| SET NETWORK HTTPS ON/OFF |  |
| Description |  |
| Enables or disables HTTPS access. |  |
| Parameter | Parameter Description |
| ON/OFF | ON = enable HTTPS; OFF = disable HTTPS |
| Return Value | Return Value Description |
| [SUCCESS] | Configuration completed successfully. |
| [FAILED] | Certificate error. |
| Example |  |
| SET NETWORK HTTPS ON |  |

SET NETWORK DNS [hostname]

| API Command |  |
| --- | --- |
| SET NETWORK DNS [hostname] |  |
| Description |  |
| Sets the controller hostname. |  |
| Parameter | Parameter Description |
| [hostname] | Hostname (maximum length: 64 characters) |
| Return Value | Return Value Description |
| [SUCCESS] | Hostname configured successfully. |
| [FAILED] | Invalid hostname format. |
| Example |  |
| SET NETWORK DNS ctl500h |  |