| Resume page |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| order number | Change the content summary description |  | edition | Revised | Revised date |
| 1 | A new release |  | 1.0 | 张泽辉 | 2024.10.28 |
| 2 | Add network audio stream API Add decoder ultra low latency API Add ‘DANTE DEV SEARCH’API |  | 1.1 | 张泽辉 | 2025.03.05 |
| 3 | Correct get the dante status API |  | 1.2 | 张泽辉 | 2025.03.07 |
| ratify: date: |  | examine and verify: date: | fabrication: date: |  |  |

catalogue

1. Summary	7

1.1 Brief introduction	7

1.2 Direction for use	7

1.3 Key concept	8

2. CTL100DA System API reference	9

2.1 Help information	9

2.2 Status information output	9

2.3 Configure the CTL100DA rear panel GPIO direction	10

2.4 Configure the CTL100DA rear panel GPIO output level	11

2.5 Obtain the CTL100DA rear panel GPIO input level	12

2.6 Get the GPIO status information of the CTL100DA rear panel	12

2.7 Set the CTL100DA serial port baudrate	13

2.8 Reset the CTL100DA system configuration	13

2.9 Reset the CTL100DA network configuration	14

2.10 Reset CTL100DA for all configurations	15

2.11 Restart the CTL100DA	15

3. The RX control module API reference	17

3.1 Set the RXID number	17

3.2 Set the RX name	17

3.3 Set RX routing	18

3.4 Lock up the RX VIDEO route	18

3.5 Lock up the RX AUDIO route	19

3.6 Lock up the RX IR route	20

3.7 Lock on the RX RS-232 routing	20

3.8 Lock up the RX USB route	21

3.9 Lock up the RX CEC route	22

3.10 Control that the RX power LED flashes	22

3.11 Set up the RX OSD switch	23

3.12 Set up the RX HDMI OUTPUT switch	24

3.13 Set up the RX HDMI OUTPUT MUTE	24

3.14 Set the RX output resolution	25

3.15 Set the RX screen to rotate	26

3.16 Set up the RX, and the screen is flipped	26

3.17 Set the RX IR level	27

3.18 Set the RX IO level	28

3.19 Set the RX IO direction	28

3.20 Set the RX IO output level	29

3.21 Set up the RX Relay switch	29

3.22 Set the RX image output mode	30

3.23 Set up the RX CEC / ARC switch	30

3.24 Set up the RX audio-return	31

3.25 Set the RX eARC to downgrade to the ARC	32

3.26 Set the RX, copper/fiber mode	32

3.27 Set up the RX USB disk / camera switch	33

3.28 Set the RX multicast mode	33

3.29 Set the RX dante bridge	34

3.30 Set the RX dante vlan	34

3.31 Set the RX dante vlan tag	35

3.32 Send CEC data to RX (Guest mode)	35

3.33 Send IR data to RX (Guest mode)	36

3.34 Set the RX serial port parameters	37

3.35 Start the RX serial port Guest mode	37

3.36 Exit the RX serial port, Guest mode	38

3.37 Set up the RXIP mode	38

3.38 Set up the RX IP address	39

3.39 Set up the RX subnet mask	39

3.40 Set up the RX gateway address	40

3.41 Set up the RX network card to restart	41

3.42 Remove the RX from the system	41

3.43 Restart RX	42

3.44 Reset RX	42

3.45 Get the RX status information	43

3.46 Set the RX preset IP mode	44

3.47 Set the RX preset IP start address	44

3.48 Set the RX preset IP end address	45

3.49 Set the RX preset subnet mask	45

3.50 Set the RX preset gateway address	46

3.51 Save the RX preset configuration	46

3.52 Set the RX ultra low latency mode	47

3.53 Set up the RX network audio stream	47

4. The TX control module API reference	49

4.1 Set the TXID number	49

4.2 Set the TX name	49

4.3 Lock up the TX ARC route	50

4.4 The control TX power LED flashes	50

4.5 Set up the TX audio source	51

4.6 Set up the TX EDID	52

4.7 Set up the TX copy of the RX EDID	53

4.8 Set the TX IR level	53

4.9 Set the TX IO level	54

4.10 Set the TX IO direction	54

4.11 Set the TX IO output level	55

4.12 Set up the TX Relay switch	56

4.13 Set up the TX CEC / ARC switch	56

4.14 Set the TX electric mode	57

4.15 Set the TX multicast mode	57

4.16 Set the TX dante bridge	58

4.17 Set the TX dante vlan	58

4.18 Set the TX dante vlan tag	59

4.19 Send CEC data to TX (Guest mode)	59

4.20 Send IR data to TX (Guest mode)	60

4.21 Set the TX serial port parameters	61

4.22 Start the TX serial port, Guest mode	61

4.23 Exit the TX serial port, Guest mode	62

4.24 Set up the TXIP mode	62

4.25 Set up the TX IP address	63

4.26 Set up the TX subnet mask	63

4.27 Set up the TX gateway address	64

4.28 Set up the TX network card to restart	65

4.29 Remove the TX from the system	65

4.30 Restart TX	66

4.31 Reset TX	66

4.32 Get the TX status information	67

4.33 Set the TX preset IP mode	68

4.34 Set the TX preset IP start address	68

4.35 Set the TX preset IP end address	69

4.36 Set the TX preset subnet mask	69

4.37 Set the TX preset gateway address	70

4.38 Save the TX preset configuration	70

4.39 Set up the TX network audio stream	71

5. The DANTE control module API reference	72

5.1 Set the dante name	72

5.2 Set the dante audio sample rate	72

5.3 Set the dante audio encoding	73

5.4 Set the dante TX channel name	73

5.5 Set the dante TX flow	74

5.6 Delete the dante TX flow	74

5.7 Set the dante RX channel name	75

5.8 Set the dante subscribe	76

5.9 Set the dante latency	76

5.10 Get the dante status	77

5.11 Search the dante devices	78

6. TX SS Module API reference	80

6.1 Get the TX SS status information	80

6.2 Get the TX SS mainstream URL	81

6.3 Gets the TX SS substream URL	81

6.4 restart TX SS	82

6.5 reset TX SS	82

6.6 Set the TX SS mainstream parameters	83

6.7 Set the TX SS subflow parameter	84

6.8 Set up the TX SS operating mode	85

6.9 Set up the TX SS IP	85

6.10 Set up the TX SS VLAN TAG	86

7. Video wall module API reference	87

7.1 Create a video wall	87

7.2 Remove the video wall	87

7.3 Modifies the video wall name	88

7.4 Set the video wall size	88

7.5 Video wall is assigned to RX	89

7.6 Create a video wall preset	89

7.7 Delete the video wall preset	90

7.8 Modifies the video wall preset name	90

7.9 Start the video wall preset	91

7.10 Set up the video wall preset class	91

7.11 Set up the signal source for the video wall preset class	92

7.12 Set up the video wall preset matrix group	92

7.13 Set up the signal source for the video wall preset matrix group	93

7.14 Set up the video wall screen width direction border	93

7.15 Set up the video wall bezel	94

7.16 Get the video wall status	95

8. System management module API reference	96

8.1 Device search	96

8.2 View the device search results	97

8.3 Clear the device search results	98

8.4 Automatically add new devices to the system	98

8.5 Add the new TX devices to the system	99

8.6 Add the new RX devices, to the system	100

8.7 Clear the existing equipment in the system	100

9. CTL100DA Network configuration API reference	102

9.1 Set up the CTL100DA IP mode	102

9.2 Set up the CTL100DA IP address	102

9.3 Set up the CTL100DA gateway address	103

9.4 Set the CTL100DA subnet mask	103

9.5 Restart the CTL100DA network card	104

9.6 Set the CTL100DA TELNET port number	105

9.7 Set up the CTL100DA HTTPS switch	105

9.8 Modify the CTL100DA domain name	106

Summary

Brief introduction

This document is used to introduce the relevant API instructions based on CTL100DA. This document is applicable to test engineers and software engineers.

Direction for use

Before using API instructions for CTL100DA, you must use TELNET to remotely log in to the corresponding terminal or use the serial port terminal for API instruction interaction. Either of the following methods can go to the control terminal for API interaction.

- TELNET Log in to CTL100DA with the default port number of 23.

- Use the serial port line to connect the serial port of the CTL100DA rear panel with the PC. Open the serial port terminal tool in the PC, and select the corresponding serial interface connection, you can enter the controller terminal for API interaction. The default port rate is 57600,8 bit data bit, 1 bit stop bit, no check bit.

Key concept

RX1：

RX 1 appearing below refers to RX with ID number 1.

TX1：

The TX 1 presented below refers to the TX with the ID number 1.

SS:

The SS presented below refers to the secondary flow module of the TX.

CTL100DA System API reference

Help information

| API joggle |  |
| --- | --- |
| The HELP or? |  |
| description |  |
| Print the API instructions supported by the current system |  |
| parameter | description |
| not have |  |
| returned value | description |
| HELP information | Print the HELP information |
| example |  |
| TELNET Log in to the CTL100DA input command: HELP input command: ? |  |

Status information output

| API joggle |  |
| --- | --- |
| GET STATUS |  |
| description |  |
| Output CTL100DA status information and the TX / RX status information added. |  |
| parameter | description |
| not have |  |
| returned value | description |
| status information |  |
| example |  |
| TELNET Log in to the CTL100DA input command: GET STATUS return: ================================================================ CTL100DA Status Info FW Version: 1.00.17 Power IR Baud On On 57600 ENC Type EDID IP NET/Sig 013 Gen 2 DF000 169.254.010.013 On /On DEC Type From IP NET/HDMI Res Mode 001 Gen 2 013 169.254.020.001 On /Off 00 MX 002 Gen 2 013 169.254.020.002 On /Off 00 MX LAN DHCP IP Gateway SubnetMask 01_POE Off 169.254.008.100 169.254.008.001 255.255.000.000 02_CTRL On 192.168.006.100 192.168.006.001 255.255.255.000 (static:192.168.006.100 192.168.006.001 255.255.255.000) Telnet SSH HTTPS LAN01 MAC LAN02 MAC 0023 Off Off 6C:DF:FB:00:01:2D 6C:DF:FB:00:01:21 Domain Name controller.local ================================================================ |  |

Configure the CTL100DA rear panel GPIO direction

| API joggle |  |
| --- | --- |
| SET GPIO [gpio] DIR IN/OUT |  |
| description |  |
| Configure the CTL100DA rear panel GPIO direction |  |
| parameter | description |
| gpio | 1: GPIO1 2: GPIO2 3: GPIO3 4: GPIO4 |
| IN | The GPIO acts as the input function |
| OUT | The GPIO acts as the output function |
| returned value | description |
| [SUCCESS]Set GPIO 01 as input port. | The GPIO 1 is configured as an input mode |
| [SUCCESS]Set GPIO 01 as output port. | The GPIO 1 is configured as an output mode |
| example |  |
| TELNET Log in to the CTL100DA Configure GPIO 1 as an input mode, enter a command: SET GPIO 1 DIR IN return: [SUCCESS]Set GPIO 01 as input port. Configure GPIO 1 as the output mode, enter the command: SET GPIO 1 DIR OUT return: [SUCCESS]Set GPIO 01 as output port. |  |

Configure the CTL100DA rear panel GPIO output level

| API joggle |  |
| --- | --- |
| SET GPIO [gpio] LEVEL Low/High |  |
| description |  |
| With the GPIO output level of CTL100DA rear panel, this API only affects GPIO with output. |  |
| parameter | description |
| gpio | 1: GPIO1 2: GPIO2 3: GPIO3 4: GPIO4 |
| Low /High | Low: output low level High: Output at a high level |
| returned value | description |
| [SUCCESS]Set GPIO 01 output level 0. | GPIO 1 output at low level |
| [SUCCESS]Set GPIO 01 output level 1. | The GPIO 1 is output at a high level |
| example |  |
| TELNET Log in to the CTL100DA Configure GPIO 1 output low level, input command: SET GPIO 1 LEVEL Low return: [SUCCESS]Set GPIO 01 output level 0. Configure GPIO 1 output high level, input command: SET GPIO 1 LEVEL High return: [SUCCESS]Set GPIO 01 output level 1. |  |

Obtain the CTL100DA rear panel GPIO input level

| API joggle |  |
| --- | --- |
| GET GPIO [gpio] LEVEL |  |
| description |  |
| The GPIO input level of the rear panel of CTL100DA is obtained, and this API only acts on the GPIO with the input direction. |  |
| parameter | description |
| gpio | 1: GPIO1 2: GPIO2 3: GPIO3 4: GPIO4 |
| returned value | description |
| [SUCCESS]Get GPIO 01 real input level 1. | The GPIO 1 acquisition input level is a high level |
| example |  |
| TELNET Log in to the CTL100DA Get the GPIO 1 input level, and enter the input command: GET GPIO 1 LEVEL return: [SUCCESS]Get GPIO 01 real input level 1. |  |

Get the GPIO status information of the CTL100DA rear panel

| API joggle |  |
| --- | --- |
| GET GPIO [gpio] STATUS |  |
| description |  |
| Get the GPIO status information of the CTL100DA rear panel. |  |
| parameter | description |
| gpio | Optional parameter that obtaining all GPIO states when not specified 1: GPIO1 2: GPIO2 3: GPIO3 4: GPIO4 |
| returned value | description |
| Returns the GPIO status information |  |
| example |  |
| TELNET Log in to the CTL100DA Get the GPIO 1 status information and enter the command: GET GPIO 1 STATUS return: ================================================================ CTL100DA GPIO Info FW Version: 1.00.17 GPIO DIR Set Get 01 In - 1 ================================================================ |  |

Set the CTL100DA serial port baudrate

| API joggle |  |
| --- | --- |
| SET RS232BAUDRATE [a] |  |
| description |  |
| Set CTL100DA serial port port rate to a, and the factory default is 57600 |  |
| parameter | description |
| a | [0:115200 1:57600, 2:38400, 3:19200, 4:9600] |
| returned value | description |
| [SUCCESS]Set RS232 Baud Rate to 115200bps. | Set the port rate to 57,600 successfully |
| example |  |
| TELNET Log in to the CTL100DA Set serial port port rate to 115200, enter command: SET RS232BAUDRATE 0 return: [SUCCESS]Set RS232 Baud Rate to 115200bps. Set the serial port port rate to 57600, enter the command: SET RS232BAUDRATE 1 return: [SUCCESS]Set RS232 Baud Rate to 57600bps. |  |

Reset the CTL100DA system configuration

| API joggle |  |
| --- | --- |
| SET RESET |  |
| description |  |
| Reset the system configuration information and Clear the equipment added to the system. |  |
| parameter | description |
| not have |  |
| returned value | description |
| [SUCCESS]System will reset to default config, it will take about 40 seconds, and RS232 will disable during this time, please wait... | The reset was successful |
| example |  |
| TELNET Log in to the CTL100DA Reset the system configuration, enter the command: SET RESET return: Sure to RESET system to default settings?Type "Yes" after next prompt to confirm... import yes return: [SUCCESS]System will reset to default config, it will take about 40 seconds, and RS232 will disable during this time, please wait... |  |

Reset the CTL100DA network configuration

| API joggle |  |
| --- | --- |
| SET RESET NETWORK |  |
| description |  |
| Reset the CTL100DA network configuration information. |  |
| parameter | description |
| not have |  |
| returned value | description |
| [SUCCESS]Network will reset to default config, it will take about 40 seconds, and RS232 will disable during this time, please wait... | The reset was successful |
| example |  |
| TELNET Log in to the CTL100DA Reset the system configuration, enter the command: SET RESET NETWORK return: Sure to RESET network config to default settings?Type "Yes" after next prompt to confirm... import yes return: [SUCCESS]Network will reset to default config, it will take about 40 seconds, and RS232 will disable during this time, please wait... |  |

Reset CTL100DA for all configurations

| API joggle |  |
| --- | --- |
| SET RESET ALL |  |
| description |  |
| Reset CTL100DA all configuration information. |  |
| parameter | description |
| not have |  |
| returned value | description |
| [SUCCESS]System and network will reset to default config, it will take about 40 seconds, and RS232 will disable during this time, please wait... | The reset was successful |
| example |  |
| TELNET Log in to the CTL100DA Reset the system configuration, enter the command: SET RESET ALL return: Sure to RESET system and network to default settings?Type "Yes" after next prompt to confirm... import yes return: [SUCCESS]System and network will reset to default config, it will take about 40 seconds, and RS232 will disable during this time, please wait... |  |

Restart the CTL100DA

| API joggle |  |
| --- | --- |
| SET REBOOT |  |
| description |  |
| Restart the CTL100DA |  |
| parameter | description |
| not have |  |
| returned value | description |
| System will restart, please wait... | Equipment restart |
| example |  |
| TELNET Log in to the CTL100DA input command: SET RE BOOT return: System will restart, please wait... |  |

The RX control module API reference

Set the RXID number

| API joggle |  |
| --- | --- |
| SET DEC [dec] ID [id] |  |
| description |  |
| Set the RXID number. |  |
| parameter | description |
| dec | [001... 762]: RXID number |
| id | [001... 762]: Target ID No |
| returned value | description |
| [SUCCESS]Set decoder 001 ID to 760. | Set the RX1 ID number to 760 |
| [ERROR]Decoder 100 does not exist. | RX 100 Does not exist |
| example |  |
| TELNET Log in to the CTL100DA RX1 ID Set to 760, enter the command: SET DEC 1 ID 760 return: [SUCCESS]Set decoder 001 ID to 760. |  |

Set the RX name

| API joggle |  |
| --- | --- |
| SET DEC [dec] NAME [name] |  |
| description |  |
| Set the RX name. |  |
| parameter | description |
| dec | [001... 762]: RXID number |
| name | Name, with a maximum length of 16 bytes |
| returned value | description |
| [SUCCESS]Set decoder 001 name:TEST1. | Set the RX 1 name to be the TEST 1 |
| example |  |
| TELNET Log in to the CTL100DA Set the RX 1 alias to TEST 1, enter the command: SET DEC 1 NAME TEST1 return: [SUCCESS]Set decoder 001 name:TEST1. |  |

Set RX routing

| API joggle |  |
| --- | --- |
| SET DEC [dec] SWITCH [enc] ALL |  |
| description |  |
| Set up the RX VIDEO / AUDIO / IR / RS-232 / USB / CEC routing |  |
| parameter | description |
| dec | [001... 762]: RXID number 0: All RX |
| enc | [001...762]: TXID number 0: Cancel the route |
| returned value | description |
| [SUCCESS]Set decoder 001 from encoder 003. | Set the RX1 VIDEO / AUDIO / IR / RS-232 / USB / CEC signal to route to TX 3 |
| example |  |
| TELNET Log in to the CTL100DA input command: SET DEC 1 SWITCH 3 ALL return: [SUCCESS]Set decoder 001 from encoder 003. input command: SET DEC 1 SWITCH 0 ALL return: [SUCCESS]Set decoder 001 VARSUC unselect encoder. |  |

Lock up the RX VIDEO route

| API joggle |  |
| --- | --- |
| SET DEC [dec] SWITCH [enc] VIDEO |  |
| description |  |
| Lock in the RX VIDEO signal routing. |  |
| parameter | description |
| dec | [001... 762]: RXID number 0: All RX |
| enc | [001...762]: TXID number 0: Unlock |
| returned value | description |
| [SUCCESS]Set decoder 001 video from encoder 003. | Set the RX 1 lock video signal to route to TX 3 |
| example |  |
| TELNET Log in to the CTL100DA input command: SET DEC 1 SWITCH 3 VIDEO return: [SUCCESS]Set decoder 001 video from encoder 003. input command: SET DEC 1 SWITCH 0 VIDEO return: [SUCCESS]Set decoder 001 unlocking video signals. |  |

Lock up the RX AUDIO route

| API joggle |  |
| --- | --- |
| SET DEC [dec] SWITCH [enc] AUDIO |  |
| description |  |
| Lock in the RX AUDIO signal routing. |  |
| parameter | description |
| dec | [001... 762]: RXID number 0: All RX |
| enc | [001...762]: TXID number 0: Unlock |
| returned value | description |
| [SUCCESS]Set decoder 001 audio from encoder 003. | Set the RX 1 lock audio signal to route to the TX 3 |
| example |  |
| TELNET Log in to the CTL100DA input command: SET DEC 1 SWITCH 3 AUDIO return: [SUCCESS]Set decoder 001 audio from encoder 003. input command: SET DEC 1 SWITCH 0 AUDIO return: [SUCCESS]Set decoder 001 unlocking audio signals. |  |

Lock up the RX IR route

| API joggle |  |
| --- | --- |
| SET DEC [dec] SWITCH [enc] IR |  |
| description |  |
| Locking on the RXIR signal routing. |  |
| parameter | description |
| dec | [001... 762]: RXID number 0: All RX |
| enc | [001...762]: TXID number 0: Unlock |
| returned value | description |
| [SUCCESS]Set decoder 001 IR from encoder 003. | Set the RX 1 lock IR signal to route to TX 3 |
| example |  |
| TELNET Log in to the CTL100DA input command: SET DEC 1 SWITCH 3 IR return: [SUCCESS]Set decoder 001 IR from encoder 003. input command: SET DEC 1 SWITCH 0 IR return: [SUCCESS]Set decoder 001 unlocking IR signals. |  |

Lock on the RX RS-232 routing

| API joggle |  |
| --- | --- |
| SET DEC [dec] SWITCH [enc] RS232 |  |
| description |  |
| Locking the RXRS-232 signal routing. |  |
| parameter | description |
| dec | [001... 762]: RXID number 0: All RX |
| enc | [001...762]: TXID number 0: Unlock |
| returned value | description |
| [SUCCESS]Set decoder 001 RS232 from encoder 003. | Set the RX 1 lock RS-232 signal to route to TX 3 |
| example |  |
| TELNET Log in to the CTL100DA input command: SET DEC 1 SWITCH 3 RS232 return: [SUCCESS]Set decoder 001 RS232 from encoder 003. input command: SET DEC 1 SWITCH 0 RS232 return: [SUCCESS]Set decoder 001 unlocking RS232 signals. |  |

Lock up the RX USB route

| API joggle |  |
| --- | --- |
| SET DEC [dec] SWITCH [enc] USB |  |
| description |  |
| Lock in the RX USB signal routing. |  |
| parameter | description |
| dec | [001... 762]: RXID number 0: All RX |
| enc | [001...762]: TXID number 0: Unlock |
| returned value | description |
| [SUCCESS]Set decoder 001 USB from encoder 003. | Set the RX 1 lock USB signals to route to TX 3 |
| example |  |
| TELNET Log in to the CTL100DA input command: SET DEC 1 SWITCH 3 USB return: [SUCCESS]Set decoder 001 USB from encoder 003. input command: SET DEC 1 SWITCH 0 USB return: [SUCCESS]Set decoder 001 unlocking USB signals. |  |

Lock up the RX CEC route

| API joggle |  |
| --- | --- |
| SET DEC [dec] SWITCH [enc] CEC |  |
| description |  |
| Lock in the RX CEC signal routing. |  |
| parameter | description |
| dec | [001... 762]: RXID number 0: All RX |
| enc | [001...762]: TXID number 0: Unlock |
| returned value | description |
| [SUCCESS]Set decoder 001 CEC from encoder 003. | Set the RX 1 lock CEC signal to route to TX 3 |
| example |  |
| TELNET Log in to the CTL100DA input command: SET DEC 1 SWITCH 3 CEC return: [SUCCESS]Set decoder 001 CEC from encoder 003. input command: SET DEC 1 SWITCH 0 CEC return: [SUCCESS]Set decoder 001 unlocking CEC signals. |  |

Control that the RX power LED flashes

| API joggle |  |
| --- | --- |
| SET DEC [dec] LED ON/OFF SET DEC [dec] LED ON 90 |  |
| description |  |
| Control that the RX power LED flashes. |  |
| parameter | description |
| dec | [001... 762]: RXID number 0: All RX |
| ON | The power LED flashes |
| OFF | The power LED is often on |
| ON 90 | The power light flashes for 90 seconds |
| returned value | description |
| [SUCCESS]Flash power LED on decoder 001. | Flash the RX 1 power LED |
| example |  |
| TELNET Log in to the CTL100DA Flash the power light, enter the command: SET DEC 1 LED ON return: [SUCCESS]Flash power LED on decoder 001. The power light is always on, input command: SET DEC 1 LED O FF return: [SUCCESS]Disable flash power LED on decoder 001. Flash the power LED for 90 seconds, enter the command: SET DEC 1 LED ON 90 return: [SUCCESS]Flash power LED on decoder 001 and keep 90 seconds. |  |

Set up the RX OSD switch

| API joggle |  |
| --- | --- |
| SET DEC [dec] OUTPUT OSD ON/OFF |  |
| description |  |
| switch RXOSD |  |
| parameter | description |
| dec | [001... 762]: RXID number 0: All RX |
| ON | open OSD |
| OFF | close OSD |
| returned value | description |
| [SUCCESS]Show OSD on decoder 001. | open OSD |
| example |  |
| TELNET Log in to the CTL100DA Open the OSD and enter the command: SET DEC 1 OUTPUT OSD ON return: [SUCCESS]Show OSD on decoder 001. Close the OSD and enter the command: SET DEC 1 OUTPUT OSD O FF return: [SUCCESS]Hide OSD on decoder 001. |  |

Set up the RX HDMI OUTPUT switch

| API joggle |  |
| --- | --- |
| SET DEC [dec] OUTPUT ON/OFF |  |
| description |  |
| Switch the switch RX HDMI OUTPUT. |  |
| parameter | description |
| dec | [001... 762]: RXID number 0: All RX |
| ON | Open the HDMI OUTPUT |
| OFF | Turn off the HDMI OUTPUT |
| returned value | description |
| [SUCCESS]Set decoder 001 output on. | Open the HDMI OUTPUT |
| example |  |
| TELNET Log in to the CTL100DA Open the HDMI OUTPUT, and enter the command: SET DEC 1 OUTPUT ON return: [SUCCESS]Set decoder 001 output on. Close the HDMI OUTPUT and enter the command: SET DEC 1 OUTPUT O FF return: [SUCCESS]Set decoder 001 output off. |  |

Set up the RX HDMI OUTPUT MUTE

| API joggle |  |
| --- | --- |
| SET DEC [dec] OUTPUT MUTE ON/OFF |  |
| description |  |
| Set up the RX HDMI OUTPUT MUTE (black screen). |  |
| parameter | description |
| dec | [001... 762]: RXID number 0: All RX |
| ON | Open the HDMI OUTPUT MUTE |
| OFF | Turn off the HDMI OUTPUT MUTE |
| returned value | description |
| [SUCCESS]Set decoder 001 output mute on. | Open the HDMI OUTPUT MUTE |
| example |  |
| TELNET Log in to the CTL100DA Open the HDMI OUTPUT MUTE, and enter the command: SET DEC 1 OUTPUT MUTE ON return: [SUCCESS]Set decoder 001 output mute on. Close the HDMI OUTPUT MUTE and enter the command: SET DEC 1 OUTPUT MUTE O FF return: [SUCCESS]Set decoder 001 output mute off. |  |

Set the RX output resolution

| API joggle |  |
| --- | --- |
| SET DEC [dec] OUTPUT RESOLUTION [res] |  |
| description |  |
| Set the RX output resolution to res |  |
| parameter | description |
| dec | [001... 762]: RXID number 0: All RX |
| res | 00: Bypass 01: 1080p@50 02: 1080p@60 03: 720p@50 04: 720p@60 05: 2160p@24 06: 2160p@30 07: 2160p@50 08: 2160p@60 09: 1280x1024@60 10: 1360x768@60 11: 1440x900@60 12: 1680x1050@60 13: 1920x1200@60 |
| returned value | description |
| [SUCCESS]Set decoder 001 resolution to 1080p@60Hz. | Set the RX 1 output resolution to 1080P60 |
| example |  |
| TELNET Log in to the CTL100DA Set the RX 1 output resolution to 1080P60, enter the command: SET DEC 1 OUTPUT RESOLUTION 2 return: [SUCCESS]Set decoder 001 resolution to 1080p@60Hz. |  |

Set the RX screen to rotate

| API joggle |  |
| --- | --- |
| SET DEC [dec] OUTPUT ROTATE [rtt] |  |
| description |  |
| Set the RX screen to rotate. |  |
| parameter | description |
| dec | [001... 762]: RXID number 0: All RX |
| rtt | 0:0° 1:90° 2:180° 3:270° |
| returned value | description |
| [SUCCESS]Set decoder 001 rotate 90 degree. | Set the RX 1 screen to flip by 90 degrees |
| example |  |
| TELNET Log in to the CTL100DA Set RX 1 screen flip 90 degrees, enter command: SET DEC 1 OUTPUT ROTATE 1 return: [SUCCESS]Set decoder 001 rotate 90 degree. |  |

Set up the RX, and the screen is flipped

| API joggle |  |
| --- | --- |
| SET DEC [dec] OUTPUT FLIP HOR/VER/OFF |  |
| description |  |
| Set the RX screen to flip over. |  |
| parameter | description |
| dec | [001... 762]: RXID number 0: All RX |
| HOR | flip horizontal |
| VER | flip vertical |
| OFF | Normal display |
| returned value |  |
| [SUCCESS]Set decoder 001 flip done. | Set the RX 1 screen to flip over |
| example |  |
| TELNET Log in to the CTL100DA Set the RX 1 screen to flip horizontally and enter the command: SET DEC 1 OUTPUT FLIP HOR return: [SUCCESS]Set decoder 001 flip done. |  |

Set the RX IR level

| API joggle |  |
| --- | --- |
| SET DEC [dec] IR VOL 5V/12V |  |
| description |  |
| Set the RXIR level. |  |
| parameter | description |
| dec | [001... 762]: RXID number 0: All RX |
| 5V | Use a 5V IR wire line |
| 12V | Use a 12V IR wire line |
| returned value | description |
| [SUCCESS]Set decoder 001 IR voltage 5V. | Set RX 1 to use 5V IR wire |
| example |  |
| TELNET Log in to the CTL100DA Set the RX 1 to use a 5V IR wire, enter the command: SET DEC 1 IR VOL 5V return: [SUCCESS]Set decoder 001 IR voltage 5V. |  |

Set the RX IO level

| API joggle |  |
| --- | --- |
| SET DEC [dec] IO VOL 5V/12V |  |
| description |  |
| Set the RXIO level. |  |
| parameter | description |
| dec | [001... 762]: RXID number 0: All RX |
| 5V | The IO level was set at 5V |
| 12V | The IO level was 12V |
| returned value | description |
| [SUCCESS]Set decoder 001 io voltage 5V. | Set the RX 1 IO level to 5V |
| example |  |
| TELNET Log in to the CTL100DA Set the RX 1 IO level to 5V, and enter the command: SET DEC 1 I O VOL 5V return: [SUCCESS]Set decoder 001 io voltage 5V. |  |

Set the RX IO direction

| API joggle |  |
| --- | --- |
| SET DEC [dec] IO 1 DIR IN/OUT SET DEC [dec] IO 2 DIR IN/OUT |  |
| description |  |
| Set the RXIO direction. |  |
| parameter | description |
| dec | [001... 762]: RXID number 0: All RX |
| IN | The IO is set as the input |
| OUT | The IO is set to the output |
| returned value | description |
| [SUCCESS]Set decoder 001 IO 1 as input port. | Set the RX 1 IO 1 as the input |
| example |  |
| TELNET Log in to the CTL100DA Set RX 1 IO 1 as input, enter a command: SET DEC 1 IO 1 DIR IN return: [SUCCESS]Set decoder 001 IO 1 as input port. |  |

Set the RX IO output level

| API joggle |  |
| --- | --- |
| SET DEC [dec] IO 1 OUT 0/1 SET DEC [dec] IO 2 OUT 0/1 |  |
| description |  |
| The RXIO output level is set and is only valid if the IO direction is set to the output. |  |
| parameter | description |
| dec | [001... 762]: RXID number 0: All RX |
| 0 | IO output low level |
| 1 | IO output at a high level |
| returned value | description |
| [SUCCESS]Set decoder 001 IO 1 output level 0. | Set the RX 1 IO 1 output low level |
| example |  |
| TELNET Log in to the CTL100DA Set the RX 1 IO 1 output low level, enter the command: SET DEC 1 IO 1 OUT 0 return: [SUCCESS]Set decoder 001 IO 1 output level 0. |  |

Set up the RX Relay switch

| API joggle |  |
| --- | --- |
| SET DEC [dec] RELAY 1 OPEN/CLOSE SET DEC [dec] RELAY 2 OPEN/CLOSE |  |
| description |  |
| Set up the RX Relay switch. |  |
| parameter | description |
| dec | [001... 762]: RXID number 0: All RX |
| OPEN | Relay break |
| CLOSE | Relay close |
| returned value | description |
| [SUCCESS]Set decoder 001 Relay 1 close. | Set the RX 1 Relay 1 to close |
| example |  |
| TELNET Log in to the CTL100DA Set the RX 1 Relay 1 to close, and enter the command: SET DEC 1 RELAY 1 CLOSE return: [SUCCESS]Set decoder 001 Relay 1 close. |  |

Set the RX image output mode

| API joggle |  |
| --- | --- |
| SET DEC [dec] MODE MX/VW |  |
| description |  |
| Set the RX image output mode and only the decoder in Video Wall is valid. |  |
| parameter | description |
| dec | [001... 762]: RXID number 0: All RX |
| MX | MX Mode output (display all images) |
| VW | VW Mode output (display partial images) |
| returned value | description |
| [SUCCESS]Set decoder 001 to matrix mode. | Set up the RX 1 MX mode output |
| example |  |
| TELNET Log in to the CTL100DA Set the RX 1 MX mode output, enter the command: SET DEC 1 MODE MX return: [SUCCESS]Set decoder 001 to matrix mode. |  |

Set up the RX CEC / ARC switch

| API joggle |  |
| --- | --- |
| SET DEC [dec] SAC ARC/CEC/OFF |  |
| description |  |
| Switch RX C EC / ARC switch, open the CEC by default. |  |
| parameter | description |
| dec | [001... 762]: RXID number 0: All RX |
| ARC | open ARC |
| CEC | open CEC |
| OFF | Close the CEC and the ARC |
| returned value | description |
| [SUCCESS]Set decoder 001 select ARC, the decoder will reboot if setting changed. | Set the RX 1 to turn on the ARC |
| example |  |
| TELNET Log in to the CTL100DA Set RX 1 to turn on ARC and enter the command: SET DEC 1 SAC ARC return: [SUCCESS]Set decoder 001 select ARC, the decoder will reboot if setting changed. |  |

Set up the RX audio-return

| API joggle |  |
| --- | --- |
| SET DEC [dec] ARP ARC/SPDIF |  |
| description |  |
| Set the RX audio return, only when RX turns on ARC. |  |
| parameter | description |
| dec | [001... 762]: RXID number 0: All RX |
| ARC | Audio goes back to the ARC signal |
| SPDIF | Audio goes back to the S / PDIF signal |
| returned value | description |
| [SUCCESS]Set decoder 001 audio return path to ARC. | Set the RX 1 audio return source to be the ARC |
| example |  |
| TELNET Log in to the CTL100DA Set the RX 1 audio return source to ARC, enter the command: SET DEC 1 ARP ARC return: [SUCCESS]Set decoder 001 audio return path to ARC. |  |

Set the RX eARC to downgrade to the ARC

| API joggle |  |
| --- | --- |
| SET DEC [dec] EARC DOWNGRADE ON/OFF |  |
| description |  |
| Set RX eARC to ARC, only when RX turns on ARC. |  |
| parameter | description |
| dec | [001... 762]: RXID number 0: All RX |
| ON | Audio goes back to the ARC signal |
| OFF | Audio goes back to the eARC signal |
| returned value | description |
| [SUCCESS]Set decoder 001 eARC downgrade on. | Set the RX 1 audio return source to be the ARC |
| example |  |
| TELNET Log in to the CTL100DA Set the RX 1 audio return source to ARC, enter the command: SET DEC 1 EARC DOWNGRADE ON return: Set decoder 001 eARC downgrade on. |  |

Set the RX, copper/fiber mode

| API joggle |  |
| --- | --- |
| SET DEC [dec] NET FIBER/COPPER |  |
| description |  |
| Set RX electric mode, default to electrical port. |  |
| parameter | description |
| dec | [001... 762]: RXID number 0: All RX |
| FIBER | The network card is in light mode |
| COPPER | The network card is electric mode |
| returned value | description |
| [SUCCESS]Set decoder 001 network interface to Copper. | Set the RX 1 network card to the electric mode |
| example |  |
| TELNET Log in to the CTL100DA Set the RX 1 network card to the electric mode, enter the command: SET DEC 1 NET COPPER return: [SUCCESS]Set decoder 001 network interface to Copper. |  |

Set up the RX USB disk / camera switch

| API joggle |  |
| --- | --- |
| SET DEC [dec] USB DATA ON/OFF |  |
| description |  |
| Set the RX USB disk / camera switch, the USB route of RX will only open KVM devices by default, and USB DATA ON is set to open USB disk / camera. |  |
| parameter | description |
| dec | [001... 762]: RXID number 0: All RX |
| ON | Turn on the USB disk / camera |
| OFF | Turn off the USB disk / camera |
| returned value | description |
| [SUCCESS]Set decoder 001 usb data on. | Turn on the RX1 USB disk / camera |
| example |  |
| TELNET Log in to the CTL100DA Open RX1 USB disk / camera and enter the command: SET DEC 1 USB DATA ON return: [SUCCESS]Set decoder 001 usb data on. |  |

Set the RX multicast mode

| API joggle |  |
| --- | --- |
| SET DEC [dec] MULTICAST ON/OFF |  |
| description |  |
| Set RX multicast mode on or off, default to off. |  |
| parameter | description |
| dec | [001... 762]: RXID No 0: All RX |
| ON | Set RX multicast mode on |
| OFF | Set RX multicast mode off |
| returned value | description |
| [SUCCESS]Set decoder 001 multicast on. | Set the RX 1 multicast on |
| example |  |
| TELNET Log in to the CTL100DA Set the RX 1 multicast on, enter the command: SET DEC 1 MULTICAST ON return: [SUCCESS]Set decoder 001 multicast on. |  |

Set the RX dante bridge

| API joggle |  |
| --- | --- |
| SET DEC [dec] DANTE BRIDGE ON/OFF |  |
| description |  |
| Set RX dante bridge on or off, default to off. |  |
| parameter | description |
| dec | [001... 762]: RXID No 0: All RX |
| ON | Set RX dante bridge on |
| OFF | Set RX dante bridge off |
| returned value | description |
| [SUCCESS]Set decoder 001 dante bridge on. | Set the RX 1 dante bridge on |
| example |  |
| TELNET Log in to the CTL100DA Set the RX 1 dante bridge on, enter the command: SET DEC 1 DANTE BRIDGE ON return: [SUCCESS]Set decoder 001 dante bridge on. |  |

Set the RX dante vlan

| API joggle |  |
| --- | --- |
| SET DEC [dec] DANTE VLAN ON/OFF |  |
| description |  |
| Set RX dante vlan on or off, default to off. |  |
| parameter | description |
| dec | [001... 762]: RXID No 0: All RX |
| ON | Set RX dante vlan on |
| OFF | Set RX dante vlan off |
| returned value | description |
| [SUCCESS]Set decoder 001 dante VLAN on. | Set the RX 1 dante vlan on |
| example |  |
| TELNET Log in to the CTL100DA Set the RX 1 dante vlan on, enter the command: SET DEC 1 DANTE VLAN ON return: [SUCCESS]Set decoder 001 dante VLAN on. |  |

Set the RX dante vlan tag

| API joggle |  |
| --- | --- |
| SET DEC [dec] DANTE VLAN TAG [tag] |  |
| description |  |
| Set RX dante vlan tag, default to 2. |  |
| parameter | description |
| dec | [001... 762]: RXID No 0: All RX |
| tag | [1...4095]: VLAN tag |
| returned value | description |
| [SUCCESS]Set decoder 001 dante VLAN tag to 2. | Set the RX 1 dante vlan tag to 2 |
| example |  |
| TELNET Log in to the CTL100DA Set the RX 1 dante vlan on, enter the command: SET DEC 1 DANTE VLAN TAG 2 return: [SUCCESS]Set decoder 001 dante VLAN tag to 2. |  |

Send CEC data to RX (Guest mode)

| API joggle |  |
| --- | --- |
| SET DEC [dec] CEC SEND xx xx |  |
| description |  |
| Send CEC data to RX (Guest mode) |  |
| parameter | description |
| dec | [001... 762]: RXID number 0: All RX |
| xx xx | 16 decimal CEC instruction code |
| returned value | description |
| [SUCCESS]Send CEC data to decoder 001 done. | The CEC instruction was sent successfully |
| example |  |
| TELNET Log in to the CTL100DA input command: SET DEC 1 CEC SEND 4F 36 return: [SUCCESS]Send CEC data to decoder 001 done. |  |

Send IR data to RX (Guest mode)

| API joggle |  |
| --- | --- |
| SET DEC [dec] IR SEND xx xx xx xx |  |
| description |  |
| Send IR data to RX (Guest mode) |  |
| parameter | description |
| dec | [001... 762]: RXID number 0: All RX |
| xx xx xx xx | 16 decimal IR instruction code, support CCF format |
| returned value | description |
| [SUCCESS]Send IR data to decoder 001 done. | Send the IR command successfully |
| example |  |
| TELNET Log in to the CTL100DA input command: SET DEC 1 IR SEND 0000006700000022015600ab001600600016006000160060001600160016006000160016001600600016001600160016001600160016001600160060001600160016006000160016001600600016006000160016001600600016001600160016001600600016006000160060001600160016006000160016001600600016006000160016001600160016001600160593 return: [SUCCESS]Send IR data to decoder 001 done. |  |

Set the RX serial port parameters

| API joggle |  |
| --- | --- |
| SET DEC [dec] GUEST ON/OFF BR [br] BIT [bit] |  |
| description |  |
| Set the RX serial port parameters |  |
| parameter | description |
| dec | [001... 762]: RXID number 0: All RX |
| ON | Open the serial port to Guest mode |
| OFF | Close the serial port, Guest mode |
| br | [0:300 1:600 2:1200 3:2400 4:4800 5:9600] [6:19200 7:38400 8:57600 9:115200] |
| bit | Data Bits + Parity + Stop Bits example: 8n1 Data Bits=[5...8], Parity=[n o e], Stop Bits=[1..2] |
| returned value | description |
| [SUCCESS]Set serial guest mode config done. | The RX Serial port parameters was set successfully |
| example |  |
| TELNET Log in to the CTL100DA Set RX 1 open serial port Guest mode, port rate 115200,8-bit data bit, no check bit, 1-bit stop bit, input command: SET DEC 1 GUEST ON BR 9 BIT 8N1 return: [SUCCESS]Set serial guest mode config done. |  |

Start the RX serial port Guest mode

| API joggle |  |
| --- | --- |
| SET DEC [dec] GUEST |  |
| description |  |
| Start RX serial port Guest mode and is only valid if the serial port parameter is set to GUEST ON |  |
| parameter | description |
| dec | [001... 762]: RXID number |
| returned value | description |
| not have |  |
| example |  |
| TELNET Log in to the CTL100DA Start the RX 1 serial port Guest mode and enter the command: SET DEC 1 GUEST |  |

Exit the RX serial port, Guest mode

| API joggle |  |
| --- | --- |
| EXITGUEST |  |
| description |  |
| After starting the RX serial port Guest mode, send the EXITGUEST to exit the Guest mode |  |
| parameter | description |
| not have |  |
| returned value | description |
| not have |  |
| example |  |
| TELNET Log in to the CTL100DA Exit the RX 1 serial port Guest mode and enter the command: EXITGUEST |  |

Set up the RXIP mode

| API joggle |  |
| --- | --- |
| SET DEC [dec] IPMODE DHCP/STATIC |  |
| description |  |
| Set up the IP mode for the RX |  |
| parameter | description |
| dec | [001... 762]: RXID number 0: All RX |
| DHCP | trends IP |
| STATIC | static state IP |
| returned value | description |
| [SUCCESS]Set encoder 001 ip mode to dhcp. Use "SET DEC xx NETWORK REBOOT" command to apply new config!!! | The setting is successful and needs to restart the RX to take effect |
| example |  |
| TELNET Log in to the CTL100DA Set RX 1 to Dynamic IP mode, enter the command: SET DEC 1 IPMODE DHCP return: [SUCCESS]Set encoder 001 ip mode to dhcp. Use "SET DEC xx NETWORK REBOOT" command to apply new config!!! |  |

Set up the RX IP address

| API joggle |  |
| --- | --- |
| SET DEC [dec] STATIC IP [ip] |  |
| description |  |
| Set the IP address for RX, only valid when IPMODE is STATIC. |  |
| parameter | description |
| dec | [001... 762]: RXID number |
| ip | IP addresses, such as 169.254.10.10 |
| returned value | description |
| [SUCCESS]Set decoder 001 IP address to 169.254.020.006. Use "SET DEC xx NETWORK REBOOT" command to apply new config!!! | The setting is successful and needs to restart the RX to take effect |
| example |  |
| TELNET Log in to the CTL100DA Set the IP of RX 1 to 169.254.20.6, enter the command: SET DEC 1 STATIC IP 169.254.20.6 return: [SUCCESS]Set decoder 001 IP address to 169.254.020.006. Use "SET DEC xx NETWORK REBOOT" command to apply new config!!! |  |

Set up the RX subnet mask

| API joggle |  |
| --- | --- |
| SET DEC [dec] STATIC MASK [mask] |  |
| description |  |
| Set the subnet mask for RX, only valid when IPMODE is STATIC. |  |
| parameter | description |
| dec | [001... 762]: RXID number 0: All RX |
| mask | Subnet mask, such as 255.255.0.0 |
| returned value | description |
| [SUCCESS]Set encoder 001 subnet mask address to 255.255.000.000. Use "SET DEC xx NETWORK REBOOT" command to apply new config!!! | The setting is successful and needs to restart the RX to take effect |
| example |  |
| TELNET Log in to the CTL100DA Set the subnet mask of RX 1 to 255.255.0.0, enter the command: SET DEC 1 STATIC MASK 255.255.0.0 return: [SUCCESS]Set encoder 001 subnet mask address to 255.255.000.000. Use "SET DEC xx NETWORK REBOOT" command to apply new config!!! |  |

Set up the RX gateway address

| API joggle |  |
| --- | --- |
| SET DEC [dec] STATIC GATEWAY [gw] |  |
| description |  |
| Sets the gateway address of RX and is only valid when IPMODE is STATIC. |  |
| parameter | description |
| dec | [001... 762]: RXID number 0: All RX |
| gw | Gateway address, such as 169.254.0.1 |
| returned value | description |
| [SUCCESS]Set encoder 001 gateway address to 169.254.000.001. Use "SET DEC xx NETWORK REBOOT" command to apply new config!!! | The setting is successful and needs to restart the RX to take effect |
| example |  |
| TELNET Log in to the CTL100DA Set the gateway address of RX to 169.254.0.1, enter the command: SET DEC 1 STATIC GATEWAY 169.254.0.1 return: [SUCCESS]Set encoder 001 gateway address to 169.254.000.001. Use "SET DEC xx NETWORK REBOOT" command to apply new config!!! |  |

Set up the RX network card to restart

| API joggle |  |
| --- | --- |
| SET DEC [dec] NETWORK REBOOT |  |
| description |  |
| Set up the network restart for RX |  |
| parameter | description |
| dec | [001... 762]: RXID number 0: All RX |
| returned value | description |
| [SUCCESS]Set decoder 001 reboot and apply all the new config. | The setting is successful and needs to restart the RX to take effect |
| example |  |
| TELNET Log in to the CTL100DA Restart the RX 1 network card and enter the command: SET DEC 1 NETWORK REBOOT return: [SUCCESS]Set decoder 001 reboot and apply all the new config. |  |

Remove the RX from the system

| API joggle |  |
| --- | --- |
| SET DEC [dec] DELETE |  |
| description |  |
| delete RX |  |
| parameter | description |
| dec | [001... 762]: RXID number 0: All RX |
| returned value | description |
| [SUCCESS]Delete decoder 001 done. | Delete RX 1 successfully |
| example |  |
| TELNET Log in to the CTL100DA Delete RX 1, enter the command: SET DEC 1 DELETE return: [SUCCESS]Delete decoder 001 done. |  |

Restart RX

| API joggle |  |
| --- | --- |
| SET DEC [dec] REBOOT |  |
| description |  |
| restart RX |  |
| parameter | description |
| dec | [001... 762]: RXID number 0: All RX |
| returned value | description |
| [SUCCESS]Set decoder 001 reboot and apply all the new config. | Restart success |
| example |  |
| TELNET Log in to the CTL100DA Restart RX 1, enter the command: SET DEC 1 REBOOT return: [SUCCESS]Set decoder 001 reboot and apply all the new config. |  |

Reset RX

| API joggle |  |
| --- | --- |
| SET DEC [dec] RESET |  |
| description |  |
| reset RX |  |
| parameter | description |
| dec | [001... 762]: RXID number 0: All RX |
| returned value | description |
| [SUCCESS]Set decoder 001 reset to default setting. | The reset was successful |
| example |  |
| TELNET Log in to the CTL100DA Reset the RX 1, enter the command: SET DEC 1 RESET return: [SUCCESS]Set decoder 001 reset to default setting. |  |

Get the RX status information

| API joggle |  |
| --- | --- |
| GET DEC [dec] STATUS |  |
| description |  |
| To obtain the status information of RX, to obtain the status information of all RX without the parameter dec, namely, GET DEC STATUS is the same as GET DEC 0 STATUS. |  |
| parameter | description |
| dec | [001... 762]: RXID number 0: All RX |
| returned value | description |
| Returns the RX status information | Include the version number, network information and other states |
| example |  |
| TELNET Log in to the CTL100DA Get the RX 1 status information, and enter the command: GET DEC 1 STATUS return: ================================================================ CTL100DA Decoder Info FW Version: 1.00.17 ID Type Net HPD Ver Mode Res Rotate Name 001 Gen 2 On Off 3.01.17 MX 00 0 Decoder 001 >>Fix Vid /Aud /IR /Ser /USB /CEC MCast Video Mute 000 /000 /000 /000 /000 /000 On On Off >>Sel Vid /Aud /IR /Ser /USB /CEC 013 /013 /013 /013 /013 /013 >>SAC OSP SGEn/Br/Bit ARC 4 Off /9 /8n1 >>Pin IOVOL/IODIR/IODAT IRVOL RLY PHY (1) 12 Out 0 12 Open Copper (2) 12 Out 0 Open >>IM MAC Static 6C:DF:FB:01:1A:CE >>IP GW SM 169.254.020.001 169.254.001.001 255.255.000.000 ================================================================ |  |

Set the RX preset IP mode

| API joggle |  |
| --- | --- |
| SET DEC PRESET IPMODE [mode] |  |
| description |  |
| Set the IP mode of RX preset and assign the IP of RX preset when RX is added to the system |  |
| parameter | description |
| mode | 0:AUTOIP 1:DHCP 2:STATIC |
| returned value | description |
| [SUCCESS]Set decoder preset IP to static mode. | Set successfully, the RX preset IP mode is static |
| example |  |
| TELNET Log in to the CTL100DA Set the RX preset IP mode to static, enter the command: SET DEC PRESET IPMODE 2 return: [SUCCESS]Set decoder preset IP to static mode. |  |

Set the RX preset IP start address

| API joggle |  |
| --- | --- |
| SET DEC PRESET START IP [ip] |  |
| description |  |
| Sets the IP start address for the RX preset |  |
| parameter | description |
| ip | IP addresses, such as 169.254.10.10 |
| returned value | description |
| [SUCCESS]Set decoder preset IP min 172.016.010.001. | Set the IP start address for the RX preset to 172.16.10.1 |
| example |  |
| TELNET Log in to the CTL100DA Set the IP start address for RX preset to 172.16.10.1, enter the command: SET DEC PRESET START IP 172.16.10.1 return: [SUCCESS]Set decoder preset IP min 172.016.010.001. |  |

Set the RX preset IP end address

| API joggle |  |
| --- | --- |
| SET DEC PRESET END IP [ip] |  |
| description |  |
| Set the IP end address of RX preset. The end address should be greater than the starting address and in the same network segment. |  |
| parameter | description |
| ip | IP addresses, such as 169.254.20.10 |
| returned value | description |
| [SUCCESS]Set decoder preset IP max 172.016.010.200. | Set the IP end address for the RX preset to 172.16.10.200 |
| example |  |
| TELNET Log in to the CTL100DA Set the IP end address of the RX preset to 172.16.10.200, enter the command: SET DEC PRESET END IP 172.16.10.200 return: [SUCCESS]Set decoder preset IP max 172.016.010.200. |  |

Set the RX preset subnet mask

| API joggle |  |
| --- | --- |
| SET DEC PRESET SM [mask] |  |
| description |  |
| Set the subnet mask for the RX preset |  |
| parameter | description |
| mask | Subnet mask, such as 255.255.0.0 |
| returned value | description |
| [SUCCESS]Set decoder preset netmask 255.255.000.000. | Set the subnet mask for the RX preset to 255.255.0.0 |
| example |  |
| TELNET Log in to the CTL100DA Set the subnet mask for the RX preset to 255.255.0.0, enter the command: SET DEC PRESET SM 255.255.0.0 return: [SUCCESS]Set decoder preset netmask 255.255.000.000. |  |

Set the RX preset gateway address

| API joggle |  |
| --- | --- |
| SET DEC PRESET GW [gw] |  |
| description |  |
| Set the gateway address for the RX preset |  |
| parameter | description |
| gw | Gateway address, such as 169.254.0.1 |
| returned value | description |
| [SUCCESS]Set decoder preset gateway 172.016.010.001. | Set the gateway address for the RX preset to 172.16.10.1 |
| example |  |
| TELNET Log in to the CTL100DA Set the gateway address of the RX preset to 172.16.10.1, enter the command: SET DEC PRESET GW 172.16.10.1 return: [SUCCESS]Set decoder preset gateway 172.016.010.001. |  |

Save the RX preset configuration

| API joggle |  |
| --- | --- |
| SET DEC PRESET APPLY |  |
| description |  |
| Save the preset configuration of RX, and after the preset IP mode above is set, you need to call APPLY to save it. |  |
| parameter | description |
| returned value | description |
| [SUCCESS]Set decoder preset IP done. | Saving the RX preset configuration is complete |
| example |  |
| TELNET Log in to the CTL100DA Save the RX preset configuration, and enter the command: SET DEC PRESET APPLY return: [SUCCESS]Set decoder preset IP done. |  |

Set the RX ultra low latency mode

| API joggle |  |
| --- | --- |
| SET DEC [dec] ULL ON/OFF |  |
| description |  |
| Set RX ultra low latency mode on or off, default to off. |  |
| parameter | description |
| dec | [001... 762]: RXID No 0: All RX |
| ON | Set RX ultra low latency mode on |
| OFF | Set RX ultra low latency mode off |
| returned value | description |
| [SUCCESS]Set decoder 001 ultra low latency mode on. | Set the RX 1 ultra low latency mode on |
| example |  |
| TELNET Log in to the CTL100DA Set the RX 1 ultra low latency mode on, enter the command: SET DEC 1 ULL ON return: [SUCCESS]Set decoder 001 ultra low latency mode on. |  |

Set up the RX network audio stream

| API joggle |  |
| --- | --- |
| SET DEC [dec] AUDIO STREAM DANTE/AES67/NONE |  |
| description |  |
| Set the RX network audio stream. |  |
| parameter | description |
| dec | [001... 762]: RXID number 0: All RX |
| DANTE | Using DANTE |
| AES67 | Using AES67 |
| NONE | None |
| returned value |  |
| [SUCCESS]Set decoder 001 audio stream DANTE. | Set the RX 1 network audio stream to DANTE |
| example |  |
| TELNET Log in to the CTL100DA Set the RX 1 network audio stream to DANTE，enter the command: SET DEC 1 AUDIO STREAM DANTE return: [SUCCESS]Set decoder 001 audio stream DANTE. |  |

The TX control module API reference

Set the TXID number

| API joggle |  |
| --- | --- |
| SET ENC [enc ] ID [id] |  |
| description |  |
| Set the TXID number. |  |
| parameter | description |
| enc | [001... 762]: TXID No |
| id | [001... 762]: Target ID No |
| returned value | description |
| [SUCCESS]Set enc oder 001 ID to 760. | Set the TX 1 ID number to 760 |
| [ERROR]Enc oder 100 does not exist. | The TX 100 does not exist |
| example |  |
| TELNET Log in to the CTL100DA TX 1 ID Set to 760, enter the command: SET ENC 1 ID 760 return: [SUCCESS]Set enc oder 001 ID to 760. |  |

Set the TX name

| API joggle |  |
| --- | --- |
| SET ENC [enc ] NAME [name] |  |
| description |  |
| Set the TX name. |  |
| parameter | description |
| enc | [001... 762]: TXID No |
| name | Name, with a maximum length of 16 bytes |
| returned value | description |
| [SUCCESS]Set enc oder 001 name:TEST1. | Set the TX 1 name to be the TEST 1 |
| example |  |
| TELNET Log in to the CTL100DA Set the TX 1 alias to TEST 1, enter the command: SET ENC 1 NAME TEST1 return: [SUCCESS]Set enc oder 001 name:TEST1. |  |

Lock up the TX ARC route

| API joggle |  |
| --- | --- |
| SET ENC [enc] SWITCH [dec] ARC |  |
| description |  |
| Lock in the TX ARC signal routing. |  |
| parameter | description |
| enc | [001... 762]: TXID No 0: All TX |
| dec | [001...762]: R X ID No 0: Unlock |
| returned value | description |
| [SUCCESS]Set encoder 001 ARC select decoder 003. | Set the TX 1 lock ARC signal routing to RX 3 |
| example |  |
| TELNET Log in to the CTL100DA input command: SET ENC 1 SWITCH 3 ARC return: [SUCCESS]Set encoder 001 ARC select decoder 003. input command: SET ENC 1 SWITCH 0 ARC return: [SUCCESS]Set encoder 001 ARC unselect input. |  |

The control TX power LED flashes

| API joggle |  |
| --- | --- |
| SET ENC [enc ] LED ON/OFF SET ENC [enc ] LED ON 90 |  |
| description |  |
| The control TX power LED flashes. |  |
| parameter | description |
| enc | [001... 762]: TXID No 0: All TX |
| ON | The power LED flashes |
| OFF | The power LED is often on |
| ON 90 | The power light flashes for 90 seconds |
| returned value | description |
| [SUCCESS]Flash power LED on enc oder 001. | Bthe TX 1 power LED |
| example |  |
| TELNET Log in to the CTL100DA Flash the power light, enter the command: SET ENC 1 LED ON return: [SUCCESS]Flash power LED on enc oder 001. The power light is always on, input command: SET ENC 1 LED O FF return: [SUCCESS]Disable flash power LED on enc oder 001. Flash the power LED for 90 seconds, enter the command: SET ENC 1 LED ON 90 return: [SUCCESS]Flash power LED on enc oder 001 and keep 90 seconds. |  |

Set up the TX audio source

| API joggle |  |
| --- | --- |
| SET ENC [enc] AUDIO INPUT HDMI/ANA |  |
| description |  |
| Set up the TX audio source. |  |
| parameter | description |
| enc | [001... 762]: TXID No 0: All TX |
| HDMI | Audio is from the HDMI IN |
| ANA | Audio is from the analog input AUDIO IN L / R |
| returned value | description |
| [SUCCESS]Set encoder 001 audio select hdmi. | Set the TX 1 audio source to be the HDMI |
| example |  |
| TELNET Log in to the CTL100DA Set the TX 1 audio source to HDMI, enter the command: SET ENC 1 AUDIO INPUT HDMI return: [SUCCESS]Set encoder 001 audio select hdmi. |  |

Set up the TX EDID

| API joggle |  |
| --- | --- |
| SET ENC [enc] EDID DEFAULT [edid] |  |
| description |  |
| Set up the TX EDID |  |
| parameter | description |
| enc | [001... 762]: TXID No 0: All TX |
| edid | 00: 1080P,Stereo Audio 2.0 SDR 01: 1080P,Dolby/DTS 5.1 SDR 02: 1080P,HD Audio 7.1 SDR 03: 1080I,Stereo Audio 2.0 SDR 04: 1080I,Dolby/DTS 5.1 SDR 05: 1080I,HD Audio 7.1 SDR 06: 3D,Stereo Audio 2.0 SDR 07: 3D,Dolby/DTS 5.1 SDR 08: 3D,HD Audio 7.1 SDR 09: 4K2K30_444,Stereo Audio 2.0 SDR 10: 4K2K30_444,Dolby/DTS 5.1 SDR 11: 4K2K30_444,HD Audio 7.1 SDR 12: 4K2K60_420,Stereo Audio 2.0 SDR (10/12-bit) 13: 4K2K60_420,Dolby/DTS 5.1 SDR (10/12-bit) 14: 4K2K60_420,HD Audio 7.1 SDR (10/12-bit) 15: 4K2K60_444,Stereo Audio 2.0 SDR (10/12-bit) 16: 4K2K60_444,Dolby/DTS 5.1 SDR (10/12-bit) 17: 4K2K60_444,HD Audio 7.1 SDR (10/12-bit) 18: 4K2K60_444,Stereo Audio 2.0 HDR (10/12-bit) 19: 4K2K60_444,Dolby/DTS 5.1 HDR (10/12-bit) 20: 4K2K60_444,HD Audio 7.1 HDR (10/12-bit) 21: DVI 1280x1024@60Hz, Audio None 22: DVI 1920x1080@60Hz, Audio None 23: DVI 1920x1200@60Hz, Audio None 25: User EDID 1 26: User EDID 2 |
| returned value | description |
| [SUCCESS]Set encoder 001 edid with default edid 00. | Set TX1 EDID to 1080P, Stereo Audio 2.0 SDR |
| example |  |
| TELNET Log in to the CTL100DA Set TX1 EDID to 1080P, Stereo Audio 2.0 SDR, enter the command: SET ENC 1 EDID DEFAULT 0 return: [SUCCESS]Set encoder 001 edid with default edid 00. |  |

Set up the TX copy of the RX EDID

| API joggle |  |
| --- | --- |
| SET ENC [enc] EDID COPY [dec] |  |
| description |  |
| Set the EDID of the TX copy of the RX |  |
| parameter | description |
| enc | [001... 762]: TXID No 0: All TX |
| dec | [001...762]: R X ID No |
| returned value | description |
| [SUCCESS]Copy decoder 002 edid to encoder 001. | Set the EDID of the TX 1 copy of the RX 2 |
| example |  |
| TELNET Log in to the CTL100DA Set the EDID of the TX 1 copy RX 2, enter the command: SET ENC 1 EDID COPY 2 return: [SUCCESS]Copy decoder 002 edid to encoder 001. |  |

Set the TX IR level

| API joggle |  |
| --- | --- |
| SET ENC [enc ] IR VOL 5V/12V |  |
| description |  |
| Set the TXIR level. |  |
| parameter | description |
| enc | [001... 762]: TXID No 0: All TX |
| 5V | Use a 5V IR wire line |
| 12V | Use a 12V IR wire line |
| returned value | description |
| [SUCCESS]Set enc oder 001 IR voltage 5V. | Set the TX 1 to use a 5V IR wire |
| example |  |
| TELNET Log in to the CTL100DA Set the TX 1 to use a 5V IR wire, enter the command: SET ENC 1 IR VOL 5V return: [SUCCESS]Set enc oder 001 IR voltage 5V. |  |

Set the TX IO level

| API joggle |  |
| --- | --- |
| SET ENC [enc ] IO VOL 5V/12V |  |
| description |  |
| Set the TXIO level. |  |
| parameter | description |
| enc | [001... 762]: TXID No 0: All TX |
| 5V | The IO level was set at 5V |
| 12V | The IO level was 12V |
| returned value | description |
| [SUCCESS]Set enc oder 001 io voltage 5V. | Set the TX1 IO level to 5V |
| example |  |
| TELNET Log in to the CTL100DA Set the TX1 IO level to 5V, and enter the command: SET ENC 1 I O VOL 5V return: [SUCCESS]Set enc oder 001 io voltage 5V. |  |

Set the TX IO direction

| API joggle |  |
| --- | --- |
| SET ENC [enc ] IO 1 DIR IN/OUT SET ENC [enc ] IO 2 DIR IN/OUT |  |
| description |  |
| Set the TXIO direction. |  |
| parameter | description |
| enc | [001... 762]: TXID No 0: All TX |
| IN | The IO is set as the input |
| OUT | The IO is set to the output |
| returned value | description |
| [SUCCESS]Set enc oder 001 IO 1 as input port. | Set the TX1 IO 1 as the input |
| example |  |
| TELNET Log in to the CTL100DA Set TX1 IO 1 as input, enter a command: SET ENC 1 IO 1 DIR IN return: [SUCCESS]Set enc oder 001 IO 1 as input port. |  |

Set the TX IO output level

| API joggle |  |
| --- | --- |
| SET ENC [enc ] IO 1 OUT 0/1 SET ENC [enc ] IO 2 OUT 0/1 |  |
| description |  |
| Set the TXIO output level, only if the IO direction is set to the output. |  |
| parameter | description |
| enc | [001... 762]: TXID No 0: All TX |
| 0 | IO output low level |
| 1 | IO output at a high level |
| returned value | description |
| [SUCCESS]Set enc oder 001 IO 1 output level 0. | Set the TX1 IO 1 output low level |
| example |  |
| TELNET Log in to the CTL100DA Set the TX1 IO 1 output low level, enter the command: SET ENC 1 IO 1 OUT 0 return: [SUCCESS]Set enc oder 001 IO 1 output level 0. |  |

Set up the TX Relay switch

| API joggle |  |
| --- | --- |
| SET ENC [enc ] RELAY 1 OPEN/CLOSE SET ENC [enc ] RELAY 2 OPEN/CLOSE |  |
| description |  |
| Set up the TX Relay switch. |  |
| parameter | description |
| enc | [001... 762]: TXID No 0: All TX |
| OPEN | Relay break |
| CLOSE | Relay close |
| returned value | description |
| [SUCCESS]Set enc oder 001 Relay 1 close. | Set the TX1 Relay 1 to close |
| example |  |
| TELNET Log in to the CTL100DA Set the TX1 Relay 1 to close, and enter the command: SET ENC 1 RELAY 1 CLOSE return: [SUCCESS]Set enc oder 001 Relay 1 close. |  |

Set up the TX CEC / ARC switch

| API joggle |  |
| --- | --- |
| SET ENC [enc ] SAC ARC/CEC/OFF |  |
| description |  |
| Switch TX C EC / ARC switch, open the CEC by default. |  |
| parameter | description |
| enc | [001... 762]: TXID No 0: All TX |
| ARC | open ARC |
| CEC | open CEC |
| OFF | Close the CEC and the ARC |
| returned value | description |
| [SUCCESS]Set enc oder 001 select ARC, the enc oder will reboot if setting changed. | Set the TX 1 to enable the ARC |
| example |  |
| TELNET Log in to the CTL100DA Set the TX 1 to turn on the ARC and enter the command: SET ENC 1 SAC ARC return: [SUCCESS]Set enc oder 001 select ARC, the enc oder will reboot if setting changed. |  |

Set the TX electric mode

| API joggle |  |
| --- | --- |
| SET ENC [enc ] NET FIBER/COPPER |  |
| description |  |
| Set TX optical mode, default to electrical port. |  |
| parameter | description |
| enc | [001... 762]: TXID No 0: All TX |
| FIBER | The network card is in light mode |
| COPPER | The network card is electric mode |
| returned value | description |
| [SUCCESS]Set enc oder 001 network interface to Copper. | Set the TX 1 network card to the electric mode |
| example |  |
| TELNET Log in to the CTL100DA Set the TX 1 network card to the electric mode, enter the command: SET ENC 1 NET COPPER return: [SUCCESS]Set enc oder 001 network interface to Copper. |  |

Set the TX multicast mode

| API joggle |  |
| --- | --- |
| SET ENC [enc ] MULTICAST ON/OFF |  |
| description |  |
| Set TX multicast mode on or off, default to off. |  |
| parameter | description |
| enc | [001... 762]: TXID No 0: All TX |
| ON | Set TX multicast mode on |
| OFF | Set TX multicast mode off |
| returned value | description |
| [SUCCESS]Set encoder 001 multicast on. | Set the TX 1 multicast on |
| example |  |
| TELNET Log in to the CTL100DA Set the TX 1 multicast on, enter the command: SET ENC 1 MULTICAST ON return: [SUCCESS]Set encoder 001 multicast on. |  |

Set the TX dante bridge

| API joggle |  |
| --- | --- |
| SET ENC [enc ] DANTE BRIDGE ON/OFF |  |
| description |  |
| Set TX dante bridge on or off, default to off. |  |
| parameter | description |
| enc | [001... 762]: TXID No 0: All TX |
| ON | Set TX dante bridge on |
| OFF | Set TX dante bridge off |
| returned value | description |
| [SUCCESS]Set encoder 001 dante bridge on. | Set the TX 1 dante bridge on |
| example |  |
| TELNET Log in to the CTL100DA Set the TX 1 dante bridge on, enter the command: SET ENC 1 DANTE BRIDGE ON return: [SUCCESS]Set encoder 001 dante bridge on. |  |

Set the TX dante vlan

| API joggle |  |
| --- | --- |
| SET ENC [enc ] DANTE VLAN ON/OFF |  |
| description |  |
| Set TX dante vlan on or off, default to off. |  |
| parameter | description |
| enc | [001... 762]: TXID No 0: All TX |
| ON | Set TX dante vlan on |
| OFF | Set TX dante vlan off |
| returned value | description |
| [SUCCESS]Set encoder 001 dante VLAN on. | Set the TX 1 dante vlan on |
| example |  |
| TELNET Log in to the CTL100DA Set the TX 1 dante vlan on, enter the command: SET ENC 1 DANTE VLAN ON return: [SUCCESS]Set encoder 001 dante VLAN on. |  |

Set the TX dante vlan tag

| API joggle |  |
| --- | --- |
| SET ENC [enc ] DANTE VLAN TAG [tag] |  |
| description |  |
| Set TX dante vlan tag, default to 2. |  |
| parameter | description |
| enc | [001... 762]: TXID No 0: All TX |
| tag | [1...4095]: VLAN tag |
| returned value | description |
| [SUCCESS]Set encoder 001 dante VLAN tag to 2. | Set the TX 1 dante vlan tag to 2 |
| example |  |
| TELNET Log in to the CTL100DA Set the TX 1 dante vlan on, enter the command: SET ENC 1 DANTE VLAN TAG 2 return: [SUCCESS]Set encoder 001 dante VLAN tag to 2. |  |

Send CEC data to TX (Guest mode)

| API joggle |  |
| --- | --- |
| SET ENC [enc ] CEC SEND xx xx |  |
| description |  |
| Send CEC data to TX (Guest mode) |  |
| parameter | description |
| enc | [001... 762]: TXID No 0: All TX |
| xx xx | 16 decimal CEC instruction code |
| returned value | description |
| [SUCCESS]Send CEC data to enc oder 001 done. | The CEC instruction was sent successfully |
| example |  |
| TELNET Log in to the CTL100DA input command: SET ENC 1 CEC SEND 4F 36 return: [SUCCESS]Send CEC data to enc oder 001 done. |  |

Send IR data to TX (Guest mode)

| API joggle |  |
| --- | --- |
| SET ENC [enc ] IR SEND xx xx xx xx |  |
| description |  |
| Send IR data to TX (Guest mode) |  |
| parameter | description |
| enc | [001... 762]: TXID No 0: All TX |
| xx xx xx xx | 16 decimal IR instruction code, support CCF format |
| returned value | description |
| [SUCCESS]Send IR data to enc oder 001 done. | Send the IR command successfully |
| example |  |
| TELNET Log in to the CTL100DA input command: SET ENC 1 IR SEND 0000006700000022015600ab001600600016006000160060001600160016006000160016001600600016001600160016001600160016001600160060001600160016006000160016001600600016006000160016001600600016001600160016001600600016006000160060001600160016006000160016001600600016006000160016001600160016001600160593 return: [SUCCESS]Send IR data to enc oder 001 done. |  |

Set the TX serial port parameters

| API joggle |  |
| --- | --- |
| SET ENC [enc ] GUEST ON/OFF BR [br] BIT [bit] |  |
| description |  |
| Set the TX serial port parameters |  |
| parameter | description |
| enc | [001... 762]: TXID No 0: All TX |
| ON | Open the serial port to Guest mode |
| OFF | Close the serial port, Guest mode |
| br | [0:300 1:600 2:1200 3:2400 4:4800 5:9600] [6:19200 7:38400 8:57600 9:115200] |
| bit | Data Bits + Parity + Stop Bits example: 8n1 Data Bits=[5...8], Parity=[n o e], Stop Bits=[1..2] |
| returned value | description |
| [SUCCESS]Set serial guest mode config done. | The TX serial port parameter was set successfully |
| example |  |
| TELNET Log in to the CTL100DA Set TX 1 open serial port Guest mode, port rate 115200,8-bit data bit, no check bit, 1-bit stop bit, input command: SET ENC 1 GUEST ON BR 9 BIT 8N1 return: [SUCCESS]Set serial guest mode config done. |  |

Start the TX serial port, Guest mode

| API joggle |  |
| --- | --- |
| SET ENC [enc ] GUEST |  |
| description |  |
| Start TX serial port Guest mode and is only valid if the serial port parameter is set to GUEST ON |  |
| parameter | description |
| enc | [001... 762]: TXID No |
| returned value | description |
| not have |  |
| example |  |
| TELNET Log in to the CTL100DA Start the TX 1 serial port Guest mode and enter the command: SET ENC 1 GUEST |  |

Exit the TX serial port, Guest mode

| API joggle |  |
| --- | --- |
| EXITGUEST |  |
| description |  |
| After starting the TX serial port Guest mode, send the EXITGUEST to exit the Guest mode |  |
| parameter | description |
| not have |  |
| returned value | description |
| not have |  |
| example |  |
| TELNET Log in to the CTL100DA Exit the TX 1 serial port Guest mode and enter the command: EXITGUEST |  |

Set up the TXIP mode

| API joggle |  |
| --- | --- |
| SET ENC [enc ] IPMODE DHCP/STATIC |  |
| description |  |
| Set the IP mode for the TX |  |
| parameter | description |
| enc | [001... 762]: TXID No 0: All TX |
| DHCP | trends IP |
| STATIC | static state IP |
| returned value | description |
| [SUCCESS]Set encoder 001 ip mode to dhcp. Use "SET ENC xx NETWORK REBOOT" command to apply new config!!! | The setting is successful and needs to restart TX to take effect |
| example |  |
| TELNET Log in to the CTL100DA Set TX 1 to Dynamic IP mode, enter the command: SET ENC 1 IPMODE DHCP return: [SUCCESS]Set encoder 001 ip mode to dhcp. Use "SET ENC xx NETWORK REBOOT" command to apply new config!!! |  |

Set up the TX IP address

| API joggle |  |
| --- | --- |
| SET ENC [enc ] STATIC IP [ip] |  |
| description |  |
| Set the IP address for TX, only valid when IPMODE is STATIC. |  |
| parameter | description |
| enc | [001... 762]: TXID No |
| ip | IP addresses, such as 169.254.10.10 |
| returned value | description |
| [SUCCESS]Set enc oder 001 IP address to 169.254.020.006. Use "SET ENC xx NETWORK REBOOT" command to apply new config!!! | The setting is successful and needs to restart TX to take effect |
| example |  |
| TELNET Log in to the CTL100DA Set the IP of TX 1 to 169.254.20.6, enter the command: SET ENC 1 STATIC IP 169.254.20.6 return: [SUCCESS]Set enc oder 001 IP address to 169.254.020.006. Use "SET ENC xx NETWORK REBOOT" command to apply new config!!! |  |

Set up the TX subnet mask

| API joggle |  |
| --- | --- |
| SET ENC [enc ] STATIC MASK [mask] |  |
| description |  |
| Set the subnet mask for TX, only valid when IPMODE is STATIC. |  |
| parameter | description |
| enc | [001... 762]: TXID No 0: All TX |
| mask | Subnet mask, such as 255.255.0.0 |
| returned value | description |
| [SUCCESS]Set encoder 001 subnet mask address to 255.255.000.000. Use "SET ENC xx NETWORK REBOOT" command to apply new config!!! | The setting is successful and needs to restart TX to take effect |
| example |  |
| TELNET Log in to the CTL100DA Set the subnet mask of TX 1 to 255.255.0.0, enter the command: SET ENC 1 STATIC MASK 255.255.0.0 return: [SUCCESS]Set encoder 001 subnet mask address to 255.255.000.000. Use "SET ENC xx NETWORK REBOOT" command to apply new config!!! |  |

Set up the TX gateway address

| API joggle |  |
| --- | --- |
| SET ENC [enc ] STATIC GATEWAY [gw] |  |
| description |  |
| Set the gateway address of TX, only valid when IPMODE is STATIC. |  |
| parameter | description |
| enc | [001... 762]: TXID No 0: All TX |
| gw | Gateway address, such as 169.254.0.1 |
| returned value | description |
| [SUCCESS]Set encoder 001 gateway address to 169.254.000.001. Use "SET ENC xx NETWORK REBOOT" command to apply new config!!! | The setting is successful and needs to restart TX to take effect |
| example |  |
| TELNET Log in to the CTL100DA Set the gateway address of TX to 169.254.0.1, enter the command: SET ENC 1 STATIC GATEWAY 169.254.0.1 return: [SUCCESS]Set encoder 001 gateway address to 169.254.000.001. Use "SET ENC xx NETWORK REBOOT" command to apply new config!!! |  |

Set up the TX network card to restart

| API joggle |  |
| --- | --- |
| SET ENC [enc ] NETWORK REBOOT |  |
| description |  |
| Set the network restart for TX |  |
| parameter | description |
| enc | [001... 762]: TXID No 0: All TX |
| returned value | description |
| [SUCCESS]Set enc oder 001 reboot and apply all the new config. | The setting is successful and needs to restart TX to take effect |
| example |  |
| TELNET Log in to the CTL100DA Restart the TX 1 network card and enter the command: SET ENC 1 NETWORK REBOOT return: [SUCCESS]Set enc oder 001 reboot and apply all the new config. |  |

Remove the TX from the system

| API joggle |  |
| --- | --- |
| SET ENC [enc ] DELETE |  |
| description |  |
| delete TX |  |
| parameter | description |
| enc | [001... 762]: TXID No 0: All TX |
| returned value | description |
| [SUCCESS]Delete enc oder 001 done. | The TX 1 was deleted successfully |
| example |  |
| TELNET Log in to the CTL100DA Delete TX 1, enter the command: SET ENC 1 DELETE return: [SUCCESS]Delete enc oder 001 done. |  |

Restart TX

| API joggle |  |
| --- | --- |
| SET ENC [enc ] REBOOT |  |
| description |  |
| restart TX |  |
| parameter | description |
| enc | [001... 762]: TXID No 0: All TX |
| returned value | description |
| [SUCCESS]Set enc oder 001 reboot and apply all the new config. | Restart success |
| example |  |
| TELNET Log in to the CTL100DA Restart the TX 1, and enter the command: SET ENC 1 REBOOT return: [SUCCESS]Set enc oder 001 reboot and apply all the new config. |  |

Reset TX

| API joggle |  |
| --- | --- |
| SET ENC [enc ] RESET |  |
| description |  |
| reset TX |  |
| parameter | description |
| enc | [001... 762]: TXID No 0: All TX |
| returned value | description |
| [SUCCESS]Set enc oder 001 reset to default setting. | The reset was successful |
| example |  |
| TELNET Log in to the CTL100DA Reset the TX 1, enter the command: SET ENC 1 RESET return: [SUCCESS]Set enc oder 001 reset to default setting. |  |

Get the TX status information

| API joggle |  |
| --- | --- |
| GET ENC [enc ] STATUS |  |
| description |  |
| To obtain the status information of TX, to obtain the status information of all TX without the parameter enc, namely, GET ENC STATUS is the same as GET ENC 0 STATUS. |  |
| parameter | description |
| enc | [001... 762]: TXID No 0: All TX |
| returned value | description |
| Returns the TX status information | Include the version number, network information and other states |
| example |  |
| TELNET Log in to the CTL100DA Get the TX 1 status information and enter the command: GET ENC 1 STATUS return: ================================================================ CTL100DA Encoder Info FW Version: 1.00.17 ID Type Net Sig Ver EDID Aud MCast Name 001 Gen 2 On On 3.00.01 DF000 HDMI On Encoder 001 >>Fix Arc 000 >>Sel Arc 000 >>SAC SGEn/Br/Bit ARC Off /9 /8n1 >>Pin IOVOL/IODIR/IODAT IRVOL RLY PHY (1) 12 Out 0 12 Open Copper (2) 12 Out 0 Open >>IM MAC Static 6C:DF:FB:07:87:E9 >>IP GW SM 169.254.010.001 169.254.001.001 255.255.000.000 ================================================================ |  |

Set the TX preset IP mode

| API joggle |  |
| --- | --- |
| SET ENC PRESET IPMODE [mode] |  |
| description |  |
| Set the IP mode of TX preset, and assign the IP of TX according to the TX preset mode when TX is added to the system |  |
| parameter | description |
| mode | 0:AUTOIP 1:DHCP 2:STATIC |
| returned value | description |
| [SUCCESS]Set enc oder preset IP to static mode. | Set successfully, the TX preset IP mode is static |
| example |  |
| TELNET Log in to the CTL100DA Set the TX preset IP mode to static, enter the command: SET ENC PRESET IPMODE 2 return: [SUCCESS]Set enc oder preset IP to static mode. |  |

Set the TX preset IP start address

| API joggle |  |
| --- | --- |
| SET ENC PRESET START IP [ip] |  |
| description |  |
| Sets the IP start address for the TX preset |  |
| parameter | description |
| ip | IP addresses, such as 169.254.10.10 |
| returned value | description |
| [SUCCESS]Set enc oder preset IP min 172.016.010.001. | Set the IP start address for the TX preset to 172.16.10.1 |
| example |  |
| TELNET Log in to the CTL100DA Set the IP start address for TX preset to 172.16.10.1, enter the command: SET ENC PRESET START IP 172.16.10.1 return: [SUCCESS]Set enc oder preset IP min 172.016.010.001. |  |

Set the TX preset IP end address

| API joggle |  |
| --- | --- |
| SET ENC PRESET END IP [ip] |  |
| description |  |
| Set the IP end address of TX preset. The end address should be greater than the starting address and in the same network segment. |  |
| parameter | description |
| ip | IP addresses, such as 169.254.20.10 |
| returned value | description |
| [SUCCESS]Set enc oder preset IP max 172.016.010.200. | Set the IP end address of the TX preset to 172.16.10.200 |
| example |  |
| TELNET Log in to the CTL100DA Set the IP end address of the TX preset to 172.16.10.200, enter the command: SET ENC PRESET END IP 172.16.10.200 return: [SUCCESS]Set enc oder preset IP max 172.016.010.200. |  |

Set the TX preset subnet mask

| API joggle |  |
| --- | --- |
| SET ENC PRESET SM [mask] |  |
| description |  |
| Set the subnet mask for the TX preset |  |
| parameter | description |
| mask | Subnet mask, such as 255.255.0.0 |
| returned value | description |
| [SUCCESS]Set enc oder preset netmask 255.255.000.000. | Set the subnet mask for the TX preset to 255.255.0.0 |
| example |  |
| TELNET Log in to the CTL100DA Set the subnet mask of the TX preset to 255.255.0.0, enter the command: SET ENC PRESET SM 255.255.0.0 return: [SUCCESS]Set enc oder preset netmask 255.255.000.000. |  |

Set the TX preset gateway address

| API joggle |  |
| --- | --- |
| SET ENC PRESET GW [gw] |  |
| description |  |
| Set the gateway address for the TX preset |  |
| parameter | description |
| gw | Gateway address, such as 169.254.0.1 |
| returned value | description |
| [SUCCESS]Set enc oder preset gateway 172.016.010.001. | Set the gateway address of the TX preset to 172.16.10.1 |
| example |  |
| TELNET Log in to the CTL100DA Set the gateway address of TX preset to 172.16.10.1, enter the command: SET ENC PRESET GW 172.16.10.1 return: [SUCCESS]Set enc oder preset gateway 172.016.010.001. |  |

Save the TX preset configuration

| API joggle |  |
| --- | --- |
| SET ENC PRESET APPLY |  |
| description |  |
| Save the preset configuration of TX, and after the preset IP mode above is set, you need to call APPLY to save it. |  |
| parameter | description |
| returned value | description |
| [SUCCESS]Set enc oder preset IP done. | Saving the TX preset configuration is complete |
| example |  |
| TELNET Log in to the CTL100DA Save the TX preset configuration, and enter the command: SET ENC PRESET APPLY return: [SUCCESS]Set enc oder preset IP done. |  |

Set up the TX network audio stream

| API joggle |  |
| --- | --- |
| SET ENC [enc] AUDIO STREAM DANTE/AES67/NONE |  |
| description |  |
| Set the TX network audio stream. |  |
| parameter | description |
| enc | [001... 762]: TXID No 0: All TX |
| DANTE | Using DANTE |
| AES67 | Using AES67 |
| NONE | None |
| returned value |  |
| [SUCCESS]Set encoder 001 audio stream DANTE. | Set the TX 1 network audio stream to DANTE |
| example |  |
| TELNET Log in to the CTL100DA Set the TX 1 network audio stream to DANTE，enter the command: SET ENC 1 AUDIO STREAM DANTE return: [SUCCESS]Set encoder 001 audio stream DANTE. |  |

The DANTE control module API reference

Set the dante name

| API joggle |  |
| --- | --- |
| SET DANTE DEV [devname] NAME [name] |  |
| description |  |
| Set the dante name. |  |
| parameter | description |
| devname | Dante device name |
| name | Name, with a maximum length of 31 bytes |
| returned value | description |
| [SUCCESS]Set dante Encoder-001 name to TX1. | Set the dante device Encoder-001 name to be the TX1 |
| example |  |
| TELNET Log in to the CTL100DA Set the dante device Encoder-001 name to TX1, enter the command: SET DANTE DEV Encoder-001 NAME TX1 return: [SUCCESS]Set dante Encoder-001 name to TX1. |  |

Set the dante audio sample rate

| API joggle |  |
| --- | --- |
| SET DANTE DEV [devname] SRATE [rate] |  |
| description |  |
| Set the dante audio sample rate. |  |
| parameter | description |
| devname | Dante device name |
| rate | Sample rate |
| returned value | description |
| [SUCCESS]Set dante TX1 srate to 44100. | Set the dante device TX1 audio sample rate to be 44100 |
| example |  |
| TELNET Log in to the CTL100DA Set the dante device TX1 audio sample rate to 44100, enter the command: SET DANTE DEV TX1 SRATE 44100 return: [SUCCESS]Set dante TX1 srate to 44100. |  |

Set the dante audio encoding

| API joggle |  |
| --- | --- |
| SET DANTE DEV [devname] ENC [enc] |  |
| description |  |
| Set the dante audio encoding. |  |
| parameter | description |
| devname | Dante device name |
| enc | Encoding |
| returned value | description |
| [SUCCESS][SUCCESS]Set dante TX1 enc to 24. | Set the dante device TX1 audio encoding to be PCM24 |
| example |  |
| TELNET Log in to the CTL100DA Set the dante device TX1 audio encoding to PCM24, enter the command: SET DANTE DEV TX1 ENC 24 return: [SUCCESS][SUCCESS]Set dante TX1 enc to 24. |  |

Set the dante TX channel name

| API joggle |  |
| --- | --- |
| SET DANTE DEV [devname] AUDIO/VIDEO TXCHN [chn] NAME [name] |  |
| description |  |
| Set the dante TX channel name. |  |
| parameter | description |
| devname | Dante device name |
| AUDIO | Audio channel |
| VIDEO | Video channel |
| chn | Channel number |
| name | Name, with a maximum length of 31 bytes |
| returned value | description |
| [SUCCESS]Set dante TX1 tx audio channel 1 name to ch1. | Set the dante device TX1 audio channel 1 name to ch1 |
| example |  |
| TELNET Log in to the CTL100DA Set the dante device TX1 audio channel 1 name to ch1, enter the command: SET DANTE DEV TX1 AUDIO TXCHN 1 NAME ch1 return: [SUCCESS]Set dante TX1 tx audio channel 1 name to ch1. |  |

Set the dante TX flow

| API joggle |  |
| --- | --- |
| SET DANTE DEV [devname] AUDIO/VIDEO TXFLOW [name] ID [id] SLOT [slot] |  |
| description |  |
| Set the dante TX flow. |  |
| parameter | description |
| devname | Dante device name |
| AUDIO | Audio channel |
| VIDEO | Video channel |
| name | Name, with a maximum length of 31 bytes |
| id | Flow ID |
| slot | TX channels |
| returned value | description |
| [SUCCESS]Dante TX1 add tx flow 1 success. | Set the dante device TX1 audio TX flow |
| example |  |
| TELNET Log in to the CTL100DA Set the dante device TX1 audio TX flow, enter the command: SET DANTE DEV TX1 AUDIO TXFLOW 1 ID 1 SLOT 1:2 return: [SUCCESS]Dante TX1 add tx flow 1 success. |  |

Delete the dante TX flow

| API joggle |  |
| --- | --- |
| SET DANTE DEV [devname] AUDIO/VIDEO TXFLOW [id] DELETE |  |
| description |  |
| Delete the dante TX flow. |  |
| parameter | description |
| devname | Dante device name |
| AUDIO | Audio channel |
| VIDEO | Video channel |
| id | Flow ID |
| returned value | description |
| [SUCCESS]Dante TX1 delete tx flow 1 success. | Delete the dante device audio TX flow 1 |
| example |  |
| TELNET Log in to the CTL100DA Delete the dante device audio TX flow 1, enter the command: SET DANTE DEV TX1 AUDIO TXFLOW 1 DELETE return: [SUCCESS]Dante TX1 delete tx flow 1 success. |  |

Set the dante RX channel name

| API joggle |  |
| --- | --- |
| SET DANTE DEV [devname] AUDIO/VIDEO RXCHN [chn] NAME [name] |  |
| description |  |
| Set the dante RX channel name. |  |
| parameter | description |
| devname | Dante device name |
| AUDIO | Audio channel |
| VIDEO | Video channel |
| chn | Channel number |
| name | Name, with a maximum length of 31 bytes |
| returned value | description |
| [SUCCESS]Set dante TX1 rx audio channel 1 name to 1. | Set the dante device TX1 audio rx channel 1 name to 1 |
| example |  |
| TELNET Log in to the CTL100DA Set the dante device TX1 audio rx channel 1 name to 1, enter the command: SET DANTE DEV TX1 AUDIO RXCHN 1 NAME 1 return: [SUCCESS]Set dante TX1 rx audio channel 1 name to 1. |  |

Set the dante subscribe

| API joggle |  |
| --- | --- |
| SET DANTE DEV [devname] AUDIO/VIDEO RXCHN [chn] SOURCE [txdev] CHN [chn] |  |
| description |  |
| Set the dante subscribe. |  |
| parameter | description |
| devname | Dante device name |
| AUDIO | Audio channel |
| VIDEO | Video channel |
| chn | Channel number For AUDIO: number could be 1 or 2 for stereo audio Dante device. To set one channel at a time. For multi-channel Dante device for example with 8 channels, the number would be 1,2,3,4…7,8. Similar for 16/32/64 channels. The chn number in RXCHN [chn] and CHN [chn] could be different values to implement channel-cross configuration. For VIDEO: number always is 1. |
| txdev | TX dante device name |
| returned value | description |
| [SUCCESS]Set dante Decoder-001 rx channel 1 subscribe to 1@TX1. | Set the dante device Decoder-001 subscribe TX1 channel 1 |
| example |  |
| TELNET Log in to the CTL100DA Set the dante device Decoder-001 subscribe TX1 channel 1, enter the command: SET DANTE DEV Decoder-001 VIDEO RXCHN 1 SOURCE TX1 CHN 1 return: [SUCCESS]Set dante Decoder-001 rx channel 1 subscribe to 1@TX1. |  |

Set the dante latency

| API joggle |  |
| --- | --- |
| SET DANTE DEV [devname] LATENCY [latency] |  |
| description |  |
| Set the dante latency. |  |
| parameter | description |
| devname | Dante device name |
| latency | Latency value |
| returned value | description |
| [SUCCESS]Set dante Decoder-001 latency to 5000. | Set the dante device Decoder-001 lantency to 5 milliseconds |
| example |  |
| TELNET Log in to the CTL100DA Set the dante device Decoder-001 lantency to 5 milliseconds, enter the command: SET DANTE DEV Decoder-001 LATENCY 5000 return: [SUCCESS]Set dante Decoder-001 latency to 5000. |  |

Get the dante status

| API joggle |  |
| --- | --- |
| GET DANTE DEV [devname] STATUS |  |
| description |  |
| Get the dante device status. |  |
| parameter | description |
| devname | Dante device name, 'all' means to retrieve all device status information |
| returned value | description |
| See below example. | Get the dante device RX3 status |
| example |  |
| TELNET Log in to the CTL100DA Get the dante device RX3 status, enter the command: GET DANTE DEV Decoder-003 STATUS return: ================================================================ Controller(DA) Dante Info FW Version: 2.00.05 ID PVer DVer Name 001 2.0.0 1.4.0.3 Decoder-003 >>TXChn Type Name 00 audio 01 01 audio 02 >>RXChn Type Name 00 video 01 01 audio 01 02 audio 02 >>SampleRate Support 48000 44100,48000,88200,96000 >>Encoding Support PCM 24 24 >>Latency Support 4000 5000 >>Aes67Support Aes67Enable Aes67Prefix No - - >>Primary v1 Multicast Leader >>IM MAC Static 6C:DF:FB:09:80:11 >>IP GW SM 169.254.020.003 169.254.001.001 255.255.000.000 ================================================================ |  |

Search the dante devices

| API joggle |  |
| --- | --- |
| DANTE DEV SEARCH |  |
| description |  |
| Search dante devices. |  |
| parameter | description |
| not have |  |
| returned value | description |
| See below example. | Search dante devices. |
| example |  |
| TELNET Log in to the CTL100DA Search dante devices, enter the command: DANTE DEV SEARCH return: ================================================================ Search Dante Result Info ==Dante Device Index IP MAC Name 001 169.254.27.127 34:d0:b8:27:05:a7 DA22XLR-WP-EU-V2-2705a7 002 169.254.20.1 6c:df:fb:00:00:1c DAV-00001c 003 169.254.10.1 6c:df:fb:01:1a:85 DAV-011a85 004 169.254.20.3 6c:df:fb:09:80:11 Decoder-003 ================================================================ |  |

TX SS Module API reference

Get the TX SS status information

| API joggle |  |
| --- | --- |
| GET ENC [enc] SS STATUS |  |
| description |  |
| To obtain the status information of TXSS, to obtain the status information of all TXSS without the parameter enc, namely, GET ENC SS STATUS is the same as GET ENC 0 SS STATUS. |  |
| parameter | description |
| enc | [001... 762]: TXID No 0: All TX |
| returned value | description |
| Returns the TXSS status information | Include the version number, network information and other states |
| example |  |
| TELNET Log in to the CTL100DA Get the TX 2 SS status information and enter the command: GET ENC 2 SS STATUS return: ================================================================ CTL100DA Secondary Stream Info ID WorkMode Version 002 follow 1.12.02 >>MainStream URL rtsp://169.254.110.1/live/main/av_stream >>SubStream URL rtsp://169.254.110.1/live/sub/av_stream >>MainStream EncType Res Fps RcMode BitRate Gop Profile h265 auto 30 cbr 2 30 main >>SubStream EncType Res Fps RcMode BitRate Gop Profile h264 480_270 30 cbr 3 30 baseline >>IpMode IpAdress Mask Gateway MAC static 169.254.110.1 255.255.0.0 169.254.10.1 6c:df:fb:09:47:b6 >>VlanTag TagID On 0 ================================================================ |  |

Get the TX SS mainstream URL

| API joggle |  |
| --- | --- |
| GET ENC [enc] SS MSURL |  |
| description |  |
| Get the mainstream URL of TX SS, Gen 1 device SS mainstream format is RTSP, Gen 2 device SS mainstream format is MJPG. |  |
| parameter | description |
| enc | [001... 762]: TXID No 0: All TX |
| returned value | description |
| Encoder 002 secondary stream MSURL:rtsp://169.254.110.1/live/main/av_stream. | The mainstream URL of TX 2 SS is rtsp://169.254.110.1/live/main/av_stream |
| example |  |
| TELNET Log in to the CTL100DA Get the TX2 SS mainstream URL, enter the command: GET ENC 2 SS MSURL return: Encoder 002 secondary stream MSURL:rtsp://169.254.110.1/live/main/av_stream. Get the TX1 SS mainstream URL, enter the command: GET ENC 1 SS MSURL return: Encoder 001 secondary stream MSURL:http://169.254.10.2:8080/?action=stream. |  |

Gets the TX SS substream URL

| API joggle |  |
| --- | --- |
| GET ENC [enc] SS SSURL |  |
| description |  |
| Get the substream URL of TX SS, Gen 1 device SS substream format is RTSP, not supported by Gen 2 device. |  |
| parameter | description |
| enc | [001... 762]: TXID No 0: All TX |
| returned value | description |
| Encoder 002 secondary stream SSURL:rtsp://169.254.110.1/live/sub/av_stream. | The substream URL of TX 2 SS is rtsp://169.254.110.1/live/sub/av_stream |
| example |  |
| TELNET Log in to the CTL100DA Get the TX2 SS secondary stream URL, enter the command: GET ENC 2 SS S SURL return: Encoder 002 secondary stream SSURL:rtsp://169.254.110.1/live/sub/av_stream. |  |

restart TX SS

| API joggle |  |
| --- | --- |
| SET ENC [enc] SS REBOOT |  |
| description |  |
| Restart the TX SS, not supported by the Gen 2 device. |  |
| parameter | description |
| enc | [001... 762]: TXID No 0: All TX |
| returned value | description |
| [SUCCESS]Set encoder 002 secondary stream reboot. | Restart the TX2 SS module |
| example |  |
| TELNET Log in to the CTL100DA Restart the TX2 SS module and enter the command: SET ENC 2 SS REBOOT return: [SUCCESS]Set encoder 002 secondary stream reboot. |  |

reset TX SS

| API joggle |  |
| --- | --- |
| SET ENC [enc] SS RESET |  |
| description |  |
| Reset TX SS, Gen 2 device is not supported. |  |
| parameter | description |
| enc | [001... 762]: TXID No 0: All TX |
| returned value | description |
| [SUCCESS]Set encoder 002 secondary stream reset to default setting. | Reset the TX2 SS module |
| example |  |
| TELNET Log in to the CTL100DA Reset the TX2 SS module, enter the command: SET ENC 2 SS RESET return: [SUCCESS]Set encoder 002 secondary stream reset to default setting. |  |

Set the TX SS mainstream parameters

| API joggle |  |
| --- | --- |
| SET ENC [enc] SS MAINENCATTR E [etype] H [mhor] V [mver] B [mbr] |  |
| description |  |
| Set the mainstream parameters of TXSS, not supported by the Gen 2 device. |  |
| parameter | description |
| enc | [001... 762]: TXID No 0: All TX |
| etype | 0:h264 1:h265 |
| mhor | Set the encoding output image width, range: 960~1920, the value must be even |
| mver | Set the encoding output image height, range: 540~1080, the value must be even |
| mbr | Set the encoded bit rate (bps) 0:1Mb 1:2Mb 2:4Mb 3:6Mb 4:8Mb 5:10Mb 6:12Mb 7:16Mb 8:20Mb |
| returned value | description |
| [SUCCESS]Set encoder 002 secondary stream MainEncAttr done. | Set success |
| example |  |
| TELNET Log in to the CTL100DA Set the mainstream encoding format of TX 2 SS to h265, output image size 960x540, encoding bit rate 2Mbps, input command: SET ENC 2 SS MAINENCATTR E 1 H 960 V 540 B 1 return: [SUCCESS]Set encoder 002 secondary stream MainEncAttr done. |  |

Set the TX SS subflow parameter

| API joggle |  |
| --- | --- |
| SET ENC [enc] SS SUBENCATTR E [etype] H [shor] V [sver] B [sbr] |  |
| description |  |
| Set the secondary flow parameters of TXSS, not supported by the Gen 2 device. |  |
| parameter | description |
| enc | [001... 762]: TXID No 0: All TX |
| etype | 0:h264 1:h265 |
| s hor | Set the encoding output image width, range: 320~960, the value must be even |
| s ver | Set the encoding output image height, range: 180~540, the value must be even |
| s br | Set the encoded bit rate (bps) 0:128kb 1:256kb 2:512kb 3:1Mb 4:2Mb |
| returned value | description |
| [SUCCESS]Set encoder 002 secondary stream SubEncAttr done. | Set success |
| example |  |
| TELNET Log in to the CTL100DA Set the secondary flow encoding format of TX 2 SS to h265, the output image size is 640x480, the encoding bit rate is 512 kbps, input command: SET ENC 2 SS SUBENCATTR E 1 H 640 V 480 B 2 return: [SUCCESS]Set encoder 002 secondary stream SubEncAttr done. |  |

Set up the TX SS operating mode

| API joggle |  |
| --- | --- |
| SET ENC [enc] SS WORKMODE [mode] |  |
| description |  |
| Set the working mode of TX SS, not supported by the Gen 2 device. |  |
| parameter | description |
| enc | [001... 762]: TXID No 0: All TX |
| mode | 0:FOLLOW 1:DHCP 2:STATIC |
| returned value | description |
| [SUCCESS]Set encoder 002 secondary stream workmode to follow. | Set the TX 2 SS to work in follow mode |
| example |  |
| TELNET Log in to the CTL100DA Set the TX 2 SS to work in follow mode, enter the command: SET ENC 2 SS WORKMODE 0 return: [SUCCESS]Set encoder 002 secondary stream workmode to follow. |  |

Set up the TX SS IP

| API joggle |  |
| --- | --- |
| SET ENC [enc] SS NETWORK IP [ip] MASK [mask] GATEWAY [gw] |  |
| description |  |
| Set TX SS IP, only valid when SS works at DHCP and STATIC, not supported by Gen 2 devices. |  |
| parameter | description |
| enc | [001... 762]: TXID No 0: All TX |
| ip | The IP address of the SS |
| mask | Subnet mask of the SS |
| gw | Gateway address of the SS |
| returned value | description |
| [SUCCESS]Set encoder 002 secondary stream manual ip 192.168.30.50 netmask 255.255.255.0 gateway 192.168.30.1. | The IP of TX 2 SS is set to 192.168.30.50, the subnet mask is 255.255.255.0, and the gateway is 192.168.30.1 |
| example |  |
| TELNET Log in to the CTL100DA The IP of setting TX 2 SS is 192.168.30.50, the subnet mask is 255.255.255.0, and the gateway is 192.168.30.1. Enter the command: SET ENC 2 SS NETWORK IP 192.168.30.50 MASK 255.255.255.0 GATEWAY 192.168.30.1 return: [SUCCESS]Set encoder 002 secondary stream manual ip 192.168.30.50 netmask 255.255.255.0 gateway 192.168.30.1. |  |

Set up the TX SS VLAN TAG

| API joggle |  |
| --- | --- |
| SET ENC [enc] SS VLTAG ON [id] SET ENC [enc] SS VLTAG OFF |  |
| description |  |
| Set the VLAN TAG for TX SS, the Gen 2 device is not supported. |  |
| parameter | description |
| enc | [001... 762]: TXID No 0: All TX |
| id | 1 ~ 4094 |
| returned value | description |
| [SUCCESS]Set encoder 002 secondary stream vlan ID to 10. | Set the VLAN TAG of TX 2 SS equal to 10 |
| example |  |
| TELNET Log in to the CTL100DA Set the VLAN TAG of TX 2 SS equal to 10, enter the command: SET ENC 2 SS VLTAG ON 10 return: [SUCCESS]Set encoder 002 secondary stream vlan ID to 10. Cancel the TX2 SS VLAN TAG and enter the command: SET ENC 2 SS VLTAG OFF return: [SUCCESS]Set encoder 002 secondary stream vlan ID to 0. |  |

Video wall module API reference

Create a video wall

| API joggle |  |
| --- | --- |
| CREATE WALL HANDLE [hdl] |  |
| description |  |
| Create a video wall |  |
| parameter | description |
| hdl | [01...09]: Video wall ID number |
| returned value | description |
| [SUCCESS]Create video wall 1. | Create a video wall with an ID equal to 1 |
| example |  |
| TELNET Log in to the CTL100DA Create a video wall, the ID equals 1, enter the command: CREATE WALL HANDLE 1 return: [SUCCESS]Create video wall 1. |  |

Remove the video wall

| API joggle |  |
| --- | --- |
| DELETE WALL HANDLE [hdl] |  |
| description |  |
| Remove the video wall |  |
| parameter | description |
| hdl | [01...09]: Video wall ID number |
| returned value | description |
| [SUCCESS]Delete videowall 1. | Delete the video wall of 1 |
| example |  |
| TELNET Log in to the CTL100DA Delete Video Wall 1, enter the command: DELETE WALL HANDLE 1 return: [SUCCESS]Delete videowall 1. |  |

Modifies the video wall name

| API joggle |  |
| --- | --- |
| SET WALL [hdl] NAME [name] |  |
| description |  |
| Modifies the name of the video wall |  |
| parameter | description |
| hdl | [01...09]: Video wall ID number |
| name | Video wall name, up to 16 characters |
| returned value | description |
| [SUCCESS]Rename video wall 1: VW1. | Delete video wall 1 is named VW1 |
| example |  |
| TELNET Log in to the CTL100DA Delete the video wall 1 with the name VW1, enter the command: SET WALL 1 NAME VW1 return: [SUCCESS]Rename video wall 1: VW1. |  |

Set the video wall size

| API joggle |  |
| --- | --- |
| SET WALL [hdl] C [c] R [r] |  |
| description |  |
| Set the size of the video wall. |  |
| parameter | description |
| hdl | [01...09]: Video wall ID number |
| c | Number of video wall columns |
| r | Number of video wall lines |
| returned value | description |
| [SUCCESS]Create video wall 1: NULL. | Set up the video wall 1 successfully |
| example |  |
| TELNET Log in to the CTL100DA Set up the video wall for 2x2 and enter the command: SET WALL 1 C 2 R 2 return: [SUCCESS]Create video wall 1: NULL. |  |

Video wall is assigned to RX

| API joggle |  |
| --- | --- |
| SET WALL [hdl] DEC [dec] H [h] V [v] |  |
| description |  |
| Video wall is assigned to RX |  |
| parameter | description |
| hdl | [01...09]: Video wall ID number |
| dec | [001...762]: R X ID No |
| h | Column number of video wall |
| v | Line number of video wall |
| returned value | description |
| [SUCCESS]Assign decoder 001 to video wall 1. | Assign the RX 1 to the video wall 1 |
| example |  |
| TELNET Log in to the CTL100DA Assign RX 1 to row 1, column 1 of video wall 1, enter the command: SET WALL 1 DEC 1 H 1 V 1 return: [SUCCESS]Assign decoder 001 to video wall 1. |  |

Create a video wall preset

| API joggle |  |
| --- | --- |
| CREATE WALL [hdl] PRESET [prs] |  |
| description |  |
| Create a video wall preset, and the preset 1 will automatically create a new video wall |  |
| parameter | description |
| hdl | [01...09]: Video wall ID number |
| prs | [01...09]: Preset the ID number |
| returned value | description |
| [SUCCESS]Create preset 2: NULL. | Create preset success |
| example |  |
| TELNET Log in to the CTL100DA Video Wall 1 Create a preset of 2, enter the command: CREATE WALL 1 PRESET 2 return: [SUCCESS]Create preset 2: NULL. |  |

Delete the video wall preset

| API joggle |  |
| --- | --- |
| DELETE WALL [hdl] PRESET [prs] |  |
| description |  |
| Delete the video wall preset. |  |
| parameter | description |
| hdl | [01...09]: Video wall ID number |
| prs | [01...09]: Preset the ID number |
| returned value | description |
| [SUCCESS]Delete preset: Preset 2. | Deleting the preset was successful |
| example |  |
| TELNET Log in to the CTL100DA Video Wall 1 delete the preset 2, enter the command: DELETE WALL 1 PRESET 2 return: [SUCCESS]Delete preset: Preset 2. |  |

Modifies the video wall preset name

| API joggle |  |
| --- | --- |
| SET WALL [hdl] PRESET [prs] NAME [name] |  |
| description |  |
| Modifies the name of the video wall preset |  |
| parameter | description |
| hdl | [01...09]: Video wall ID number |
| prs | [01...09]: Preset the ID number |
| name | Preset name, up to 16 characters supported |
| returned value | description |
| [SUCCESS]Rename preset 1: TEST1. | Modifies the video wall preset name to TEST 1 |
| example |  |
| TELNET Log in to the CTL100DA Modify the name of the video wall 1 preset 1 to TEST 1, enter the command: SET WALL 1 PRESET 1 NAME TEST1 return: [SUCCESS]Rename preset 1: TEST1. |  |

Start the video wall preset

| API joggle |  |
| --- | --- |
| APPLY WALL [hdl] PRESET [prs] |  |
| description |  |
| Start the video wall preset. |  |
| parameter | description |
| hdl | [01...09]: Video wall ID number |
| prs | [01...09]: Preset the ID number |
| returned value | description |
| [SUCCESS]Apply preset: Preset 1. | Started preset success |
| example |  |
| TELNET Log in to the CTL100DA Start the video wall 1 preset 1, enter the command: APPLY WALL 1 PRESET 1 return: [SUCCESS]Apply preset: Preset 1. |  |

Set up the video wall preset class

| API joggle |  |
| --- | --- |
| SET WALL [hdl] PRESET [prs] CLASS [cls] H [h] V [v] |  |
| description |  |
| Set the video wall preset class and default all RX is in group A when the preset is created. |  |
| parameter | description |
| hdl | [01...09]: Video wall ID number |
| prs | [01...09]: Preset the ID number |
| cls | [A... G]: Group ID number |
| h | Column number of video wall |
| v | Line number of video wall |
| returned value | description |
| [SUCCESS]Done. | Group was created successfully |
| example |  |
| TELNET Log in to the CTL100DA Set the screen of video wall 1 to preset 1 group B, enter the command: SET WALL 1 PRESET 1 CLASS B H 2 V 1 return: [SUCCESS]Done. |  |

Set up the signal source for the video wall preset class

| API joggle |  |
| --- | --- |
| SET WALL [hdl] PRESET [prs] CLASS [cls] SOURCE [enc] |  |
| description |  |
| Set up the signal source for the video wall preset class. |  |
| parameter | description |
| hdl | [01...09]: Video wall ID number |
| prs | [01...09]: Preset the ID number |
| cls | [A... G]: Group ID number |
| enc | [001...762]: TXID number 0: Cancel the route |
| returned value | description |
| [SUCCESS]Done. | Set success |
| example |  |
| TELNET Log in to the CTL100DA Set the signal source of the video wall 1 preset 1 packet B as TX 1, and enter the command: SET WALL 1 PRESET 1 CLASS B SOURCE 1 return: [SUCCESS]Done. |  |

Set up the video wall preset matrix group

| API joggle |  |
| --- | --- |
| SET WALL [hdl] PRESET [prs] MATRIX H [h] V [v] |  |
| description |  |
| Set the video wall preset matrix group, and the RX in the matrix group is in matrix mode. |  |
| parameter | description |
| hdl | [01...09]: Video wall ID number |
| prs | [01...09]: Preset the ID number |
| h | Column number of video wall |
| v | Line number of video wall |
| returned value | description |
| [SUCCESS]Done. | Set the matrix composition work |
| example |  |
| TELNET Log in to the CTL100DA Set the screen of the video wall 1, line 1, column 2 as the preset 1 matrix group, enter the command: SET WALL 1 PRESET 1 MATRIX H 2 V 1 return: [SUCCESS]Done. |  |

Set up the signal source for the video wall preset matrix group

| API joggle |  |
| --- | --- |
| SET WALL [hdl] PRESET [prs] MATRIX H [h] V [v] SOURCE [enc] |  |
| description |  |
| Set up the signal source for the video wall preset matrix group. |  |
| parameter | description |
| hdl | [01...09]: Video wall ID number |
| prs | [01...09]: Preset the ID number |
| h | Column number of video wall |
| v | Line number of video wall |
| enc | [001...762]: TXID number 0: Cancel the route |
| returned value | description |
| [SUCCESS]Done. | Set success |
| example |  |
| TELNET Log in to the CTL100DA Set the signal source of the video wall 1 preset 1 row 1 as TX 1, input command: SET WALL 1 PRESET 1 MATRIX H 2 V 1 SOURCE 1 return: [SUCCESS]Done. |  |

Set up the video wall screen width direction border

| API joggle |  |
| --- | --- |
| SET WALL [hdl] H [h] V [v] WIDTH BEZEL BW [b] IW [i] |  |
| description |  |
| Set the border of the width of the video wall screen. |  |
| parameter | description |
| hdl | [01...09]: Video wall ID number |
| h | Column number of video wall |
| v | Line number of video wall |
| b | [100...1000]: Original image width |
| i | [100...1000]:, visible image width, i cannot be greater than b |
| returned value | description |
| [SUCCESS]Done. | Set success |
| example |  |
| TELNET Log in to the CTL100DA Set the screen width direction of video wall 1 row 1 column 1 crop 10%, (BW-IW) / BW = 10%, enter command: SET WALL 1 H 1 V 1 WIDTH BEZEL BW 1000 IW 900 return: [SUCCESS]Done. |  |

Set up the video wall bezel

| API joggle |  |
| --- | --- |
| SET WALL [hdl] H [h] V [v] HEIGHT BEZEL BH [b] IH [i] |  |
| description |  |
| Set the border of the height direction of the video wall screen. |  |
| parameter | description |
| hdl | [01...09]: Video wall ID number |
| h | Column number of video wall |
| v | Line number of video wall |
| b | [100...1000]: Raw image height |
| i | [100...1000]:, visible image height, i cannot be greater than b |
| returned value | description |
| [SUCCESS]Done. | Set success |
| example |  |
| TELNET Log in to the CTL100DA Set the screen height orientation of video wall 1 row 1 column 10%, (BH-IH) / BH = 10%, enter command: SET WALL 1 H 1 V 1 HEIGHT BEZEL BH 1000 IH 900 return: [SUCCESS]Done. |  |

Get the video wall status

| API joggle |  |
| --- | --- |
| GET WALL [hdl] STATUS |  |
| description |  |
| Get the video wall status |  |
| parameter | description |
| hdl | [01...09]: Video wall ID number |
| returned value | description |
| Print the video wall status information |  |
| example |  |
| TELNET Log in to the CTL100DA Get the video wall 1 status information and enter the command: GET WALL 1 STATUS return: ================================================================ CTL100DA Video Wall Info FW Version: 1.00.17 VW Col Row CfgSel Name 01 02 02 01 VW1 OutID 001 002 --- --- Cfg Name 01 TEST1 Class From Screen A 001 H01V02 H02V02 B 001 H01V01 Single From H02V01 001 ================================================================ |  |

System management module API reference

Device search

| API joggle |  |
| --- | --- |
| SEARCH |  |
| description |  |
| Search for online devices |  |
| parameter | description |
| not have |  |
| returned value | description |
| Returns the current system online device information |  |
| example |  |
| TELNET Log in to the CTL100DA Search for online devices and enter a command: SEARCH return: [SUCCESS]More device in network will take more time to finish search, please wait...done. ================================================================ Search Device Result Info ==New Encoder None ==System Control Encoder Index IP MAC ID 001 169.254.010.001 6C:DF:FB:00:87:81 002 002 169.254.010.003 6C:DF:FB:00:87:82 003 003 169.254.010.002 6C:DF:FB:01:1A:84 001 004 169.254.010.004 6C:DF:FB:01:1A:6C 004 ==New Decoder None ==System Control Decoder Index IP MAC ID 001 169.254.020.002 6C:DF:FB:01:1A:C2 002 002 169.254.020.004 6C:DF:FB:00:02:EF 001 003 169.254.020.003 6C:DF:FB:00:F3:66 004 004 169.254.020.001 6C:DF:FB:01:1A:CB 003 ================================================================ |  |

View the device search results

| API joggle |  |
| --- | --- |
| GET SEARCH STATUS |  |
| description |  |
| View the device search results |  |
| parameter | description |
| not have |  |
| returned value | description |
| Returns the current system online device information |  |
| example |  |
| TELNET Log in to the CTL100DA View the device search results and enter the command: GET SEARCH STATUS return: ================================================================ Search Device Result Info ==New Encoder None ==System Control Encoder Index IP MAC ID 001 169.254.010.001 6C:DF:FB:00:87:81 002 002 169.254.010.003 6C:DF:FB:00:87:82 003 003 169.254.010.002 6C:DF:FB:01:1A:84 001 004 169.254.010.004 6C:DF:FB:01:1A:6C 004 ==New Decoder None ==System Control Decoder Index IP MAC ID 001 169.254.020.002 6C:DF:FB:01:1A:C2 002 002 169.254.020.004 6C:DF:FB:00:02:EF 001 003 169.254.020.003 6C:DF:FB:00:F3:66 004 004 169.254.020.001 6C:DF:FB:01:1A:CB 003 ================================================================ |  |

Clear the device search results

| API joggle |  |
| --- | --- |
| SEARCH RESET |  |
| description |  |
| Clear the device search results |  |
| parameter | description |
| not have |  |
| returned value | description |
| [SUCCESS]Reset search info. | Clear the device search results |
| example |  |
| TELNET Log in to the CTL100DA Clear the device search results, enter the command: SEARCH RESET return: [SUCCESS]Reset search info. |  |

Automatically add new devices to the system

| API joggle |  |
| --- | --- |
| ADD AUTO ALL |  |
| description |  |
| Automatically add new devices to the system, and call the SEARCH interface to search the online device before calling this interface. |  |
| parameter | description |
| not have |  |
| returned value | description |
| [SUCCESS]Add scan index 001 device to decoder 003. [SUCCESS]Add scan index 002 device to decoder 004. [SUCCESS]Add scan index 003 device to decoder 002. [SUCCESS]Add scan index 004 device to decoder 001. | New device was added successfully. |
| example |  |
| TELNET Log in to the CTL100DA Automatically add new devices to the system and enter the command: ADD AUTO ALL return: [SUCCESS]Add scan index 001 device to decoder 003. [SUCCESS]Add scan index 002 device to decoder 004. [SUCCESS]Add scan index 003 device to decoder 002. [SUCCESS]Add scan index 004 device to decoder 001. |  |

Add the new TX devices to the system

| API joggle |  |
| --- | --- |
| ADD DEV [dev] ENC [enc] |  |
| description |  |
| Add a new TX device to the system, and call the SEARCH interface to search the online device before calling this interface. |  |
| parameter | description |
| dev | SEARCH The Index of the New Encoder in the Results |
| enc | [001...762]: TXID number 0: The system automatically assigns an ID |
| returned value | description |
| [SUCCESS]Add scan index 001 device to encoder 004. | Add TX devices with Index 1 in New Encoder to the system and assign ID 4 |
| example |  |
| TELNET Log in to the CTL100DA Add TX devices with Index 1 in New Encoder to the system and assign ID 4, enter the command: ADD DEV 1 ENC 4 return: [SUCCESS]Add scan index 001 device to encoder 004. |  |

Add the new RX devices, to the system

| API joggle |  |
| --- | --- |
| ADD DEV [dev] DEC [dec] |  |
| description |  |
| Add a new RX device to the system, and call the SEARCH interface to search the online device before calling this interface. |  |
| parameter | description |
| dev | SEARCH The Index of the New De coder in the Results |
| dec | [001...762]: R X ID No 0: The system automatically assigns an ID |
| returned value | description |
| [SUCCESS]Add scan index 001 device to decoder 001. | The RX device with Index 1 in New De coder was added to the system and assigned ID 1 |
| example |  |
| TELNET Log in to the CTL100DA Add RX devices with Index 1 in New De coder to the system and assign ID 1, enter the command: ADD DEV 1 DEC 1 return: [SUCCESS]Add scan index 001 device to decoder 001. |  |

Clear the existing equipment in the system

| API joggle |  |
| --- | --- |
| ADD DEV RESET |  |
| description |  |
| Clear the existing equipment in the system, and will Clear the information related to the equipment, such as VW. |  |
| parameter | description |
| not have |  |
| returned value | description |
| [SUCCESS]Reset all Encoder/Decoder/Videowall/Search configuration. | Clear the existing equipment and related configuration in the system |
| example |  |
| TELNET Log in to the CTL100DA Clear the existing equipment in the system, enter the command: ADD DEV RESET return: [SUCCESS]Reset all Encoder/Decoder/Videowall/Search configuration. |  |

CTL100DA Network configuration API reference

Set up the CTL100DA IP mode

| API joggle |  |
| --- | --- |
| SET NETWORK [lan] DHCP ON/OFF |  |
| description |  |
| Set the IP mode for the CTL100DA CONTROL LAN and VIDEO LAN ports. |  |
| parameter | description |
| lan | LAN1: VIDEO LAN port LAN2: CONTROL LAN port |
| ON/OFF | ON: To enable the DHCP OFF: Turn off the DHCP with a static IP |
| returned value | description |
| [SUCCESS]Set lan2 DHCP to on. Use "SET NETWORK REBOOT" command or repower device to apply new config!!! | Set the CONTROL LAN port to the DHCP mode |
| example |  |
| TELNET Log in to the CTL100DA Set the CONTROL LAN port to the DHCP mode, and enter the command: SET NETWORK LAN2 DHCP ON return: [SUCCESS]Set lan2 DHCP to on. Use "SET NETWORK REBOOT" command or repower device to apply new config!!! |  |

Set up the CTL100DA IP address

| API joggle |  |
| --- | --- |
| SET NETWORK [lan] STATIC IP [ip] |  |
| description |  |
| Set the IP address of CTL100DA CONTROL LAN and VIDEO LAN ports, only valid when the network port is set to static IP mode. |  |
| parameter | description |
| lan | LAN1: VIDEO LAN port LAN2: CONTROL LAN port |
| ip | IP addresses, such as 169.254.20.10 |
| returned value | description |
| [SUCCESS]Set lan2 IP address to 192.168.070.040. Use "SET NETWORK REBOOT" command or repower device to apply new config!!! | Set the IP of the CONTROL LAN port to 192.168.70.40 |
| example |  |
| TELNET Log in to the CTL100DA Set the IP of the CONTROL LAN port to 192.168.70.40, enter the command: SET NETWORK LAN2 STATIC IP 192.168.70.40 return: [SUCCESS]Set lan2 IP address to 192.168.070.040. Use "SET NETWORK REBOOT" command or repower device to apply new config!!! |  |

Set up the CTL100DA gateway address

| API joggle |  |
| --- | --- |
| SET NETWORK [lan] STATIC GATEWAY [gw] |  |
| description |  |
| Set the gateway address of CTL100DA CONTROL LAN and VIDEO LAN ports, only valid when the network port is set to static IP mode. |  |
| parameter | description |
| lan | LAN1: VIDEO LAN port LAN2: CONTROL LAN port |
| gw | Gateway address, such as 169.254.0.1 |
| returned value | description |
| [SUCCESS]Set lan2 gateway address to 192.168.070.001. Use "SET NETWORK REBOOT" command or repower device to apply new config!!! | The gateway address of the CONTROL LAN port is 192.168.70.1 |
| example |  |
| TELNET Log in to the CTL100DA Set the gateway address of the CONTROL LAN port to 192.168.70.1, enter the command: SET NETWORK LAN2 STATIC GATEWAY 192.168.70.1 NET RB return: [SUCCESS]Set lan2 gateway address to 192.168.070.001. Use "SET NETWORK REBOOT" command or repower device to apply new config!!! |  |

Set the CTL100DA subnet mask

| API joggle |  |
| --- | --- |
| SET NETWORK [lan] STATIC MASK [mask] |  |
| description |  |
| Set the subnet mask for CTL100DA CONTROL LAN and VIDEO LAN ports only when the port is set to static IP mode. |  |
| parameter | description |
| lan | LAN1: VIDEO LAN port LAN2: CONTROL LAN port |
| mask | Subnet mask, such as 255.255.0.0 |
| returned value | description |
| [SUCCESS]Set lan2 subnet mask address to 255.255.255.000. Use "SET NETWORK REBOOT" command or repower device to apply new config!!! | Set the subnet mask of the CONTROL LAN port to 255.255.255.0 |
| example |  |
| TELNET Log in to the CTL100DA Set the subnet mask of the CONTROL LAN port to 255.255.255.0, enter the command: SET NETWORK LAN2 STATIC MASK 255.255.255.0 return: [SUCCESS]Set lan2 subnet mask address to 255.255.255.000. Use "SET NETWORK REBOOT" command or repower device to apply new config!!! |  |

Restart the CTL100DA network card

| API joggle |  |
| --- | --- |
| SET NETWORK REBOOT |  |
| description |  |
| Restart the CTL100DA network card, after modifying the network parameters, you need to send this API to make the configuration effective. |  |
| parameter | description |
| not have |  |
| returned value | description |
| [SUCCESS]Set network reboot and apply new config. | The network card was restarted successfully |
| example |  |
| TELNET Log in to the CTL100DA Restart the network card and enter the command: SET NETWORK REBOOT return: [SUCCESS]Set network reboot and apply new config. |  |

Set the CTL100DA TELNET port number

| API joggle |  |
| --- | --- |
| SET NETWORK TELNET PORT [port] |  |
| description |  |
| Set the CTL100DA TELNET port number, which default to 23 |  |
| parameter | description |
| port | TELNET Port number, with a maximum value of 65535 |
| returned value | description |
| [SUCCESS]Set telnet port to 0030. | Set the TELNET port number to 30 |
| example |  |
| TELNET Log in to the CTL100DA Set the TELNET port number to 30, and enter the command: SET NETWORK TELNET PORT 30 return: [SUCCESS]Set telnet port to 0030. |  |

Set up the CTL100DA HTTPS switch

| API joggle |  |
| --- | --- |
| SET NETWORK HTTPS ON/OFF |  |
| description |  |
| Set the CTL100DA HTTPS switch, and the HTTPS is not turned on by default. |  |
| parameter | description |
| ON/OFF | ON: Open the HTTPS OFF: Turn off the HTTPS |
| returned value | description |
| [SUCCESS]Set web gui https on. | open HTTPS |
| example |  |
| TELNET Log in to the CTL100DA Open the HTTPS and enter the command: SET NETWORK HTTPS ON return: [SUCCESS]Set web gui https on. |  |

Modify the CTL100DA domain name

| API joggle |  |
| --- | --- |
| SET NETWORK DNS hostname |  |
| description |  |
| Modify the domain name of CTL100DA, and the default domain name is controller.local。 |  |
| parameter | description |
| hostname | Domain name, only support letters, numbers, and special characters such as _. |
| returned value | description |
| [SUCCESS]Set DNS domain name to test.local. System will restart, please wait... | Set the domain name to the test.local |
| example |  |
| TELNET Log in to the CTL100DA Set the domain name to the test.local, Enter the command: SET NETWORK DNS test return: [SUCCESS]Set DNS domain name to test.local. System will restart, please wait... |  |